|  | Huawei Server Huawei-iBMC-Cmdlets Plug-in V1.3.1 |  |  |  |
| --- | --- | --- | --- | --- |
|  | User Guide |  |  |  |
|  | Issue | 01 |  |  |
|  | Date | 2020-05-06 |  |  |
|  | HUAWEI TECHNOLOGIES CO., LTD. |  |  |  |

Copyright © Huawei Technologies Co., Ltd. 2020. All rights reserved. No part of this document may be reproduced or transmitted in any form or by any means without prior written consent of Huawei Technologies Co., Ltd. Trademarks and Permissions and other Huawei trademarks are trademarks of Huawei Technologies Co., Ltd. All other trademarks and trade names mentioned in this document are the property of their respective holders. Notice The purchased products, services and features are stipulated by the contract made between Huawei and the customer. All or part of the products, services and features described in this document may not be within the purchase scope or the usage scope. Unless otherwise specified in the contract, all statements, information, and recommendations in this document are provided "AS IS" without warranties, guarantees or representations of any kind, either express or implied. The information in this document is subject to change without notice. Every effort has been made in the preparation of this document to ensure accuracy of the contents, but all statements, information, and recommendations in this document do not constitute a warranty of any kind, express or implied.

| Huawei Technologies Co., Ltd. |  |
| --- | --- |
| Address: | Huawei Industrial Base Bantian, Longgang Shenzhen 518129 People's Republic of China |
| Website: | https://e.huawei.com |

About This Document

Purpose

This document describes how to install and use Huawei-iBMC-Cmdlets.

Intended Audience

This document is intended for:

Technical support engineers

Maintenance engineers

Symbol Conventions

The symbols that may be found in this document are defined as follows.

| Symbol | Description |
| --- | --- |
|  | Indicates an imminently hazardous situation which, if not avoided, will result in death or serious injury. |
|  | Indicates a potentially hazardous situation which, if not avoided, could result in death or serious injury. |
|  | Indicates a potentially hazardous situation which, if not avoided, may result in minor or moderate injury. |
|  | Indicates a potentially hazardous situation which, if not avoided, could result in equipment damage, data loss, performance deterioration, or unanticipated results. NOTICE is used to address practices not related to personal injury. |
|  | Calls attention to important information, best practices and tips. NOTE is used to address information not related to personal injury, equipment damage, and environment deterioration. |

Change History

Changes between document issues are cumulative. The latest document issue contains all the changes made in earlier issues.

| Issue | Date | Description |
| --- | --- | --- |
| 01 | 2020-05-06 | This issue is the first official release. |

Contents

About This Document	ii

1 Introduction	1

2 Installing and Uninstalling Huawei-iBMC-Cmdlets	3

2.1 Preparations	3

2.2 Installing and Uninstalling Huawei-iBMC-Cmdlets Online	5

2.3 Installing and Uninstalling Huawei-iBMC-Cmdlets on a Local PC	5

3 Using Huawei-iBMC-Cmdlets	7

3.1 Viewing the Help Information	13

3.2 Querying the Huawei-iBMC-Cmdlets Version Information	16

3.3 Connecting to the iBMC	17

3.4 Disconnecting from the iBMC	21

3.5 Testing the iBMC Connection Status	23

3.6 Querying the Session Service Information	24

3.7 Modifying the Session Service Information	25

3.8 Querying the User List	26

3.9 Creating a User	27

3.10 Modifying a User	29

3.11 Removing a User	30

3.12 Querying the iBMC Service Information	31

3.13 Setting the iBMC Service Information	32

3.14 Querying the Power Consumption Information	33

3.15 Querying the System Information	34

3.16 Querying the Ethernet Port Information	37

3.17 Setting the Power Supply Status	38

3.18 Restarting the iBMC	39

3.19 Restarting the System	40

3.20 Querying the Asset Tag	41

3.21 Setting the Asset Tag	41

3.22 Querying the Virtual Media Information	42

3.23 Connecting to the Virtual Media	44

3.24 Disconnecting from the Virtual Media	45

3.25 Querying the Boot Device	46

3.26 Setting the Boot Device	47

3.27 Querying the Boot Sequence	48

3.28 Setting the Boot Sequence	49

3.29 Exporting the BIOS Configuration	50

3.30 Importing the BIOS Configuration	52

3.31 Restoring the Factory Settings	53

3.32 Restoring the Default BIOS Settings	54

3.33 Querying the NTP Resource Information	55

3.34 Setting NTP Resource Attributes	56

3.35 Importing the NTP Group Key	58

3.36 Querying the SMTP Notification List	59

3.37 Setting the SMTP Notification Object	60

3.38 Querying the SMTP Resource Information	61

3.39 Setting SMTP Resource Attributes	62

3.40 Querying the SNMP Resource Information	64

3.41 Setting SNMP Resource Attributes	65

3.42 Querying the SNMP Trap Server	67

3.43 Setting the SNMP Trap Server	69

3.44 Querying the SNMP Trap Resource Information	70

3.45 Setting SNMP Trap Resource Attributes	71

3.46 Querying the Syslog Notification List	73

3.47 Setting the Syslog Notification Object	74

3.48 Querying the Syslog Resource Information	75

3.49 Setting the Syslog Resource Information	76

3.50 Querying the In-Band Firmware Version	78

3.51 Querying the Out-of-Band Firmware Version	78

3.52 Upgrading the In-Band Firmware	79

3.53 Upgrading the Out-of-Band Firmware	81

3.54 Querying the CPU Health Status	83

3.55 Querying the RAID Controller Card Health Status	84

3.56 Querying the Memory Health Status	85

3.57 Querying the Power Supply Health Status	86

3.58 Querying the Drive Health Status	87

3.59 Querying the NIC Health Status	88

3.60 Querying the Fan Health Status	89

3.61 Querying the CPU Information	90

3.62 Querying the RAID Controller Card Information	93

3.63 Querying the Memory Information	96

3.64 Querying the Power Supply Information	98

3.65 Querying the Drive Information	101

3.66 Querying the NIC Information	107

3.67 Querying the Fan Information	109

3.68 Querying the Configuration Result Resource of the Smart Provisioning Service	111

3.69 Modifying the Smart Provisioning Service Resource Attributes	113

3.70 Exporting the RAID Configuration of the Smart Provisioning Service	113

3.71 Creating the RAID Configuration of the Smart Provisioning Service	114

3.72 Deleting the RAID Configuration of the Smart Provisioning Service	116

3.73 Modifying the RAID Configuration of the Smart Provisioning Service	117

3.74 Querying the RAID Configuration Resource of the Smart Provisioning Service	118

3.75 Querying the OS Installation Configuration Resources of the Smart Provisioning Service	120

3.76 Creating the OS Installation Configuration for the Smart Provisioning Service	122

3.77 Restoring the Default Configuration of a Specified RAID Controller Card	128

3.78 Modifying the Information of a Specified RAID Controller Card Resource	129

3.79 Modifying the Attributes of a Specified Drive	130

3.80 Querying the Logical Drive Resource Information	131

3.81 Modifying the Logical Drive Resource Attributes	135

3.82 Initializing a Logical Drive	138

3.83 Deleting a Logical Drive	139

3.84 Creating a Logical Drive	140

3.85 Collecting iBMC Logs in One-click Mode	144

3.86 Downloading a BMC File	146

3.87 Uploading a File	147

3.88 Querying the License Service Information	148

3.89 Installing a License	150

3.90 Exporting a License	151

3.91 Deleting a License	152

3.92 Querying iBMC Network Port Information	153

3.93 Setting iBMC Network port Information	155

3.94 Querying LDAP Information	157

3.95 Modifying LDAP Information	158

3.96 Importing an LDAP certificate to the iBMC	160

3.97 Setting the LDAP Status	162

4 FAQs	164

4.1 How to Invoke Cmdlets Without Using a Plaintext Password	164

4.2 How to Use the CA Root Certificate to Authenticate a Huawei Server	165

4.3 How to Invoke a Cmdlets Interface	174

4.4 How to Record Logs as a Non-Administrator User	175

A Obtaining Help	177

A.1 Preparing to Contact Huawei Technical Support	177

A.2 Obtaining Help from Huawei Support Website	177

Introduction

Function Description

Huawei-iBMC-Cmdlets is a command-line tool developed based on the PowerShell language. It connects to the iBMC over the Redfish interface to implement management of Huawei servers. Huawei-iBMC-Cmdlets provides the following functions:

Query of basic server information

Configuration of iBMC and BIOS parameters

Upgrade of the iBMC, BIOS, and CPLD firmware

Upgrade of the Smart Provisioning, RAID controller card, and NIC firmware

Reset and power control of the server

Configuration of settings related to operating system (OS) deployment

Query of basic information and health status of components

RAID configuration

Log collection

Concurrent access to a maximum of 256 servers

Access to the iBMC using an IPv4 or IPv6 address or a domain name

Supported Servers

| Architecture | Type | Server Model |
| --- | --- | --- |
| x86 | Rack server | RH1288 V3 |
|  |  | RH2288 V3 |
|  |  | RH2288H V3 |
|  |  | 1288H V5 |
|  |  | 2288 V5 |
|  |  | 2288H V5 |
|  |  | 2488H V5 |
|  | Blade server | CH121 V3 |
|  |  | CH242 V3 |
|  | High-density server | XH622 V3 |
|  |  | XH321 V5 |
| Arm | Rack server | TaiShan 200 server (model 2280) |
|  |  | TaiShan 200 server (model 5280) |
|  |  | TaiShan 200 server (model 5290) |
|  |  | TaiShan 200 server (model 2480) |

Supported Components

| Type | Model |
| --- | --- |
| RAID controller card | LSI SAS3008, LSI SAS3108, Avago SAS3408iMR, Avago SAS3508, and Avago SAS3004 |
| NIC | X722, SP212, SP330, and SP380 |

Version Mapping

| Version | Mapping Version |
| --- | --- |
| BIOS | x86 server: 0.39 or later Arm server: 1.12 or later |
| iBMC | x86 V5 server: V322 or later x86 V3 server: V308 or later Arm server: V556 or later (for version number in Vxxx format) 3.01.01.00 or later (for version number in x.xx.xx.xx format) |
| Smart Provisioning | x86 server: V116 or later Arm server: V129 or later |

Installing and Uninstalling Huawei-iBMC-Cmdlets

2.1  Preparations

2.2  Installing and Uninstalling Huawei-iBMC-Cmdlets Online

2.3  Installing and Uninstalling Huawei-iBMC-Cmdlets on a Local PC

  - Preparations

Huawei-iBMC-Cmdlets supports .Net Framework 4.5 or later. Perform the following operations to view the .Net Framework version:

  - In the Start search box, enter regedit.exe and press Enter to open the Registry Editor.

  - The HKEY_LOCAL_MACHINE\SOFTWARE\Microsoft\NET Framework Setup\NDP\v4\Full file shows the .Net Framework version, as shown in Figure 2-1.

Registry Editor

Huawei-iBMC-Cmdlets supports PowerShell 5.0 or later. Perform the following operations to view the PowerShell version:

  - Open the PowerShell CLI.

  - Run the $PSVersionTable command to view the PowerShell version, as shown in Figure 2-2.

Viewing the PowerShell version

  - If the PowerShell version is earlier than 5.0, perform the following operations to install PowerShell 5.0 or later:

Figure 2-3 lists the PowerShell installed by default for each Windows version. PowerShell 5.1 is used as an example.

PowerShell installed by default

    - Uninstall the PowerShell of the earlier version.

    - Download PowerShell 5.1 from the following link:

https://docs.microsoft.com/en-us/powershell/scripting/install/installing-windows-powershell?view=powershell-6

    - Click Download, select W2K12-KB3191565-x64.msu, and click Next.

PowerShell 5.1 download page

    - Double-click W2K12-KB3191565-x64.msu and install PowerShell 5.1 using the default settings.

    - Restart the OS for the installation to take effect.

Installing and Uninstalling Huawei-iBMC-Cmdlets Online

Prerequisites

Ensure that the environment can access the Internet.

Procedure

Install Huawei-iBMC-Cmdlets.

  - Open the PowerShell CLI.

  - Run the following command to install Huawei-iBMC-Cmdlets:

PS C:\Users\Administrator> install-module Huawei-iBMC-Cmdlets

  - Run the following command to check whether Huawei-iBMC-Cmdlets is successfully installed:

PS C:\Users\Administrator> Get-Module | ? Name -eq Huawei-iBMC-Cmdlets  
  
ModuleType Version Name                                ExportedCommands  
---------- ------- ----                                ----------------  
Script     1.3.1      Huawei-iBMC-Cmdlets                 {Add-iBMCUser, Close-iBMCRedfishSession, Connect-iBMC, Con...

Uninstall Huawei-iBMC-Cmdlets.

  - Open the PowerShell CLI.

  - Run the following command to uninstall Huawei-iBMC-Cmdlets:

PS C:\Users\Administrator> uninstall-module Huawei-iBMC-Cmdlets

Installing and Uninstalling Huawei-iBMC-Cmdlets on a Local PC

Prerequisites

You have obtained the Huawei-iBMC-Cmdlets VX.X.zip software package, in which X.X indicates the version, for example, Huawei-iBMC-Cmdlets V1.0.zip.

Procedure

Install Huawei-iBMC-Cmdlets.

  - Decompress the Huawei-iBMC-Cmdlets V1.3.1.zip software package to obtain the Huawei-iBMC-Cmdlets folder, as shown in Figure 2-5.

Huawei-iBMC-Cmdlets folder

  - Run the $env:PSModulePath command to view the installation directory supported by PowerShell.

PS C:\Users\Administrator> $env:PSModulePath  
C:\Users\Administrator\Documents\WindowsPowerShell\Modules;C:\Program Files\WindowsPowerShell\Modules;C:\Windows\system32\WindowsPowerShell\v1.3.1\Modules

  - Copy the Huawei-iBMC-Cmdlets folder to the PowerShell installation directory.

  - Open the PowerShell CLI.

  - Run the following command to install Huawei-iBMC-Cmdlets:

PS C:\Users\Administrator> Import-Module Huawei-iBMC-Cmdlets -Force

  - Run the following command to check whether Huawei-iBMC-Cmdlets is successfully installed:

PS C:\Users\Administrator> Get-Module | ? Name -eq Huawei-iBMC-Cmdlets  
  
ModuleType Version Name                                ExportedCommands  
---------- ------- ----                                ----------------  
Script     1.3.1      Huawei-iBMC-Cmdlets                 {Add-iBMCUser, Close-iBMCRedfishSession, Connect-iBMC, Con...

Uninstall Huawei-iBMC-Cmdlets.

  - Open the PowerShell CLI.

  - Run the following command to uninstall Huawei-iBMC-Cmdlets:

PS C:\Users\Administrator> remove-module –name Huawei-iBMC-Cmdlets

  - Delete the Huawei-iBMC-Cmdlets file from the PowerShell module directory.

Using Huawei-iBMC-Cmdlets

You can run the following command to view all Huawei-iBMC-Cmdlets commands:

Get-Command -Module Huawei-iBMC-Cmdlets

PS C:\Users\Administrator> Get-Command -Module Huawei-iBMC-Cmdlets 
 
CommandType     Name                                               Version    Source 
-----------     ----                                               -------    ------ 
Function        Add-iBMCSPRAIDVolume                               1.3.1      Huawei-iBMC-Cmdlets 
Function        Add-iBMCUser                                       1.3.1      Huawei-iBMC-Cmdlets 
Function        Add-iBMCVolume                                     1.3.1      Huawei-iBMC-Cmdlets 
Function        Clear-iBMCSPRAIDSetting                            1.3.1      Huawei-iBMC-Cmdlets 
Function        Connect-iBMC                                       1.3.1      Huawei-iBMC-Cmdlets 
Function        Connect-iBMCVirtualMedia                           1.3.1      Huawei-iBMC-Cmdlets 
Function        Disconnect-iBMC                                    1.3.1      Huawei-iBMC-Cmdlets 
Function        Disconnect-iBMCVirtualMedia                        1.3.1      Huawei-iBMC-Cmdlets 
Function        Export-iBMCBIOSSetting                             1.3.1      Huawei-iBMC-Cmdlets 
Function        Export-iBMCLicense                                 1.3.1      Huawei-iBMC-Cmdlets 
Function        Export-iBMCMaintenanceInfo                         1.3.1      Huawei-iBMC-Cmdlets 
Function        Export-iBMCSPRAIDSetting                           1.3.1      Huawei-iBMC-Cmdlets 
Function        Get-iBMCAssetTag                                   1.3.1      Huawei-iBMC-Cmdlets 
Function        Get-iBMCBootSourceOverride                         1.3.1      Huawei-iBMC-Cmdlets 
Function        Get-iBMCBootupSequence                             1.3.1      Huawei-iBMC-Cmdlets 
Function        Get-iBMCDrives                                     1.3.1      Huawei-iBMC-Cmdlets 
Function        Get-iBMCDrivesHealth                               1.3.1      Huawei-iBMC-Cmdlets 
Function        Get-iBMCFans                                       1.3.1      Huawei-iBMC-Cmdlets 
Function        Get-iBMCFansHealth                                 1.3.1      Huawei-iBMC-Cmdlets 
Function        Get-iBMCInbandFirmware                             1.3.1      Huawei-iBMC-Cmdlets 
Function        Get-iBMCIP                                         1.3.1      Huawei-iBMC-Cmdlets 
Function        Get-iBMCLDAP                                       1.3.1      Huawei-iBMC-Cmdlets 
Function        Get-iBMCLicense                                    1.3.1      Huawei-iBMC-Cmdlets 
Function        Get-iBMCMemory                                     1.3.1      Huawei-iBMC-Cmdlets 
Function        Get-iBMCMemoryHealth                               1.3.1      Huawei-iBMC-Cmdlets 
Function        Get-iBMCModuleVersion                              1.3.1      Huawei-iBMC-Cmdlets 
Function        Get-iBMCNetworkAdapters                            1.3.1      Huawei-iBMC-Cmdlets 
Function        Get-iBMCNetworkAdaptersHealth                      1.3.1      Huawei-iBMC-Cmdlets 
Function        Get-iBMCNTPSetting                                 1.3.1      Huawei-iBMC-Cmdlets 
Function        Get-iBMCOSDeployConfig                             1.3.1      Huawei-iBMC-Cmdlets 
Function        Get-iBMCOutbandFirmware                            1.3.1      Huawei-iBMC-Cmdlets 
Function        Get-iBMCPowerInfo                                  1.3.1      Huawei-iBMC-Cmdlets 
Function        Get-iBMCPowerSupplies                              1.3.1      Huawei-iBMC-Cmdlets 
Function        Get-iBMCPowerSuppliesHealth                        1.3.1      Huawei-iBMC-Cmdlets 
Function        Get-iBMCProcessors                                 1.3.1      Huawei-iBMC-Cmdlets 
Function        Get-iBMCProcessorsHealth                           1.3.1      Huawei-iBMC-Cmdlets 
Function        Get-iBMCRAIDControllers                            1.3.1      Huawei-iBMC-Cmdlets 
Function        Get-iBMCRAIDControllersHealth                      1.3.1      Huawei-iBMC-Cmdlets 
Function        Get-iBMCServices                                   1.3.1      Huawei-iBMC-Cmdlets 
Function        Get-iBMCSessionTimeout                             1.3.1      Huawei-iBMC-Cmdlets 
Function        Get-iBMCSMTPRecipients                             1.3.1      Huawei-iBMC-Cmdlets 
Function        Get-iBMCSMTPSetting                                1.3.1      Huawei-iBMC-Cmdlets 
Function        Get-iBMCSNMPSetting                                1.3.1      Huawei-iBMC-Cmdlets 
Function        Get-iBMCSNMPTrapServer                             1.3.1      Huawei-iBMC-Cmdlets 
Function        Get-iBMCSNMPTrapSetting                            1.3.1      Huawei-iBMC-Cmdlets 
Function        Get-iBMCSPRAIDSetting                              1.3.1      Huawei-iBMC-Cmdlets 
Function        Get-iBMCSPTaskResult                               1.3.1      Huawei-iBMC-Cmdlets 
Function        Get-iBMCSyslogServer                               1.3.1      Huawei-iBMC-Cmdlets 
Function        Get-iBMCSyslogSetting                              1.3.1      Huawei-iBMC-Cmdlets 
Function        Get-iBMCSystemInfo                                 1.3.1      Huawei-iBMC-Cmdlets 
Function        Get-iBMCSystemNetworkSettings                      1.3.1      Huawei-iBMC-Cmdlets 
Function        Get-iBMCUser                                       1.3.1      Huawei-iBMC-Cmdlets 
Function        Get-iBMCVirtualMedia                               1.3.1      Huawei-iBMC-Cmdlets 
Function        Get-iBMCVolume                                     1.3.1      Huawei-iBMC-Cmdlets 
Function        Import-iBMCBIOSSetting                             1.3.1      Huawei-iBMC-Cmdlets 
Function        Import-iBMCLDAPCert                                1.3.1      Huawei-iBMC-Cmdlets 
Function        Import-iBMCNTPGroupKey                             1.3.1      Huawei-iBMC-Cmdlets 
Function        Initialize-iBMCVolume                              1.3.1      Huawei-iBMC-Cmdlets 
Function        Install-iBMCLicense                                1.3.1      Huawei-iBMC-Cmdlets 
Function        Invoke-iBMCFileDownload                            1.3.1      Huawei-iBMC-Cmdlets 
Function        Invoke-iBMCFileUpload                              1.3.1      Huawei-iBMC-Cmdlets 
Function        Remove-iBMCLicense                                 1.3.1      Huawei-iBMC-Cmdlets 
Function        Remove-iBMCUser                                    1.3.1      Huawei-iBMC-Cmdlets 
Function        Remove-iBMCVolume                                  1.3.1      Huawei-iBMC-Cmdlets 
Function        Reset-iBMC                                         1.3.1      Huawei-iBMC-Cmdlets 
Function        Reset-iBMCBIOSSetting                              1.3.1      Huawei-iBMC-Cmdlets 
Function        Restore-iBMCFactorySetting                         1.3.1      Huawei-iBMC-Cmdlets 
Function        Restore-iBMCRAIDController                         1.3.1      Huawei-iBMC-Cmdlets 
Function        Set-iBMCAssetTag                                   1.3.1      Huawei-iBMC-Cmdlets 
Function        Set-iBMCBootSourceOverride                         1.3.1      Huawei-iBMC-Cmdlets 
Function        Set-iBMCBootupSequence                             1.3.1      Huawei-iBMC-Cmdlets 
Function        Set-iBMCDrive                                      1.3.1      Huawei-iBMC-Cmdlets 
Function        Set-iBMCFruControl                                 1.3.1      Huawei-iBMC-Cmdlets 
Function        Set-iBMCIP                                         1.3.1      Huawei-iBMC-Cmdlets 
Function        Set-iBMCLDAP                                       1.3.1      Huawei-iBMC-Cmdlets 
Function        Set-iBMCLDAPServiceEnabled                         1.3.1      Huawei-iBMC-Cmdlets 
Function        Set-iBMCNTPSetting                                 1.3.1      Huawei-iBMC-Cmdlets 
Function        Set-iBMCOSDeployConfig                             1.3.1      Huawei-iBMC-Cmdlets 
Function        Set-iBMCRAIDController                             1.3.1      Huawei-iBMC-Cmdlets 
Function        Set-iBMCServerPower                                1.3.1      Huawei-iBMC-Cmdlets 
Function        Set-iBMCService                                    1.3.1      Huawei-iBMC-Cmdlets 
Function        Set-iBMCSessionTimeout                             1.3.1      Huawei-iBMC-Cmdlets 
Function        Set-iBMCSMTPRecipient                              1.3.1      Huawei-iBMC-Cmdlets 
Function        Set-iBMCSMTPSetting                                1.3.1      Huawei-iBMC-Cmdlets 
Function        Set-iBMCSNMPSetting                                1.3.1      Huawei-iBMC-Cmdlets 
Function        Set-iBMCSNMPTrapServer                             1.3.1      Huawei-iBMC-Cmdlets 
Function        Set-iBMCSNMPTrapSetting                            1.3.1      Huawei-iBMC-Cmdlets 
Function        Set-iBMCSPRAIDSetting                              1.3.1      Huawei-iBMC-Cmdlets 
Function        Set-iBMCSPService                                  1.3.1      Huawei-iBMC-Cmdlets 
Function        Set-iBMCSyslogServer                               1.3.1      Huawei-iBMC-Cmdlets 
Function        Set-iBMCSyslogSetting                              1.3.1      Huawei-iBMC-Cmdlets 
Function        Set-iBMCUser                                       1.3.1      Huawei-iBMC-Cmdlets 
Function        Set-iBMCVolume                                     1.3.1      Huawei-iBMC-Cmdlets 
Function        Test-iBMCConnect                                   1.3.1      Huawei-iBMC-Cmdlets 
Function        Update-iBMCInbandFirmware                          1.3.1      Huawei-iBMC-Cmdlets 
Function        Update-iBMCOutbandFirmware                         1.3.1      Huawei-iBMC-Cmdlets

3.1  Viewing the Help Information

3.2  Querying the Huawei-iBMC-Cmdlets Version Information

3.3  Connecting to the iBMC

3.4  Disconnecting from the iBMC

3.5  Testing the iBMC Connection Status

3.6  Querying the Session Service Information

3.7  Modifying the Session Service Information

3.8  Querying the User List

3.9  Creating a User

3.10  Modifying a User

3.11  Removing a User

3.12  Querying the iBMC Service Information

3.13  Setting the iBMC Service Information

3.14  Querying the Power Consumption Information

3.15  Querying the System Information

3.16  Querying the Ethernet Port Information

3.17  Setting the Power Supply Status

3.18  Restarting the iBMC

3.19  Restarting the System

3.20  Querying the Asset Tag

3.21  Setting the Asset Tag

3.22  Querying the Virtual Media Information

3.23  Connecting to the Virtual Media

3.24  Disconnecting from the Virtual Media

3.25  Querying the Boot Device

3.26  Setting the Boot Device

3.27  Querying the Boot Sequence

3.28  Setting the Boot Sequence

3.29  Exporting the BIOS Configuration

3.30  Importing the BIOS Configuration

3.31  Restoring the Factory Settings

3.32  Restoring the Default BIOS Settings

3.33  Querying the NTP Resource Information

3.34  Setting NTP Resource Attributes

3.35  Importing the NTP Group Key

3.36  Querying the SMTP Notification List

3.37  Setting the SMTP Notification Object

3.38  Querying the SMTP Resource Information

3.39  Setting SMTP Resource Attributes

3.40  Querying the SNMP Resource Information

3.41  Setting SNMP Resource Attributes

3.42  Querying the SNMP Trap Server

3.43  Setting the SNMP Trap Server

3.44  Querying the SNMP Trap Resource Information

3.45  Setting SNMP Trap Resource Attributes

3.46  Querying the Syslog Notification List

3.47  Setting the Syslog Notification Object

3.48  Querying the Syslog Resource Information

3.49  Setting the Syslog Resource Information

3.50  Querying the In-Band Firmware Version

3.51  Querying the Out-of-Band Firmware Version

3.52  Upgrading the In-Band Firmware

3.53  Upgrading the Out-of-Band Firmware

3.54  Querying the CPU Health Status

3.55  Querying the RAID Controller Card Health Status

3.56  Querying the Memory Health Status

3.57  Querying the Power Supply Health Status

3.58  Querying the Drive Health Status

3.59  Querying the NIC Health Status

3.60  Querying the Fan Health Status

3.61  Querying the CPU Information

3.62  Querying the RAID Controller Card Information

3.63  Querying the Memory Information

3.64  Querying the Power Supply Information

3.65  Querying the Drive Information

3.66  Querying the NIC Information

3.67  Querying the Fan Information

3.68  Querying the Configuration Result Resource of the Smart Provisioning Service

3.69  Modifying the Smart Provisioning Service Resource Attributes

3.70  Exporting the RAID Configuration of the Smart Provisioning Service

3.71  Creating the RAID Configuration of the Smart Provisioning Service

3.72  Deleting the RAID Configuration of the Smart Provisioning Service

3.73  Modifying the RAID Configuration of the Smart Provisioning Service

3.74  Querying the RAID Configuration Resource of the Smart Provisioning Service

3.75  Querying the OS Installation Configuration Resources of the Smart Provisioning Service

3.76  Creating the OS Installation Configuration for the Smart Provisioning Service

3.77  Restoring the Default Configuration of a Specified RAID Controller Card

3.78  Modifying the Information of a Specified RAID Controller Card Resource

3.79  Modifying the Attributes of a Specified Drive

3.80  Querying the Logical Drive Resource Information

3.81  Modifying the Logical Drive Resource Attributes

3.82  Initializing a Logical Drive

3.83  Deleting a Logical Drive

3.84  Creating a Logical Drive

3.85  Collecting iBMC Logs in One-click Mode

3.86  Downloading a BMC File

3.87  Uploading a File

3.88  Querying the License Service Information

3.89  Installing a License

3.90  Exporting a License

3.91  Deleting a License

3.92  Querying iBMC Network Port Information

3.93  Setting iBMC Network port Information

3.94  Querying LDAP Information

3.95  Modifying LDAP Information

3.96  Importing an LDAP certificate to the iBMC

3.97  Setting the LDAP Status

  - Viewing the Help Information

Function

View the command help information.

Format

Get-Help <Name> -Full

Parameters

| Parameter | Description |
| --- | --- |
| <Name> | Specifies the parameter name. This parameter is mandatory. |
| -Full | Specifies the detailed description of the command. This parameter is optional. |

Usage Guidelines

None

Example

View the help information of the Connect-iBMC command.

PS C:\> Get-Help Connect-iBMC -Full

The command output of Connect-iBMC is as follows:

NAME 
    Connect-iBMC 
SYNOPSIS 
    Connect to iBMC Servers and initialize sessions used by other cmdlets. 
SYNTAX 
    Connect-iBMC [-Address] <String[]> [-Username] <String[]> [-Password] <String[]> [-TrustCert] [<CommonParameters>] 
    Connect-iBMC [-Address] <String[]> [-Credential] <PSCredential[]> [-TrustCert] [<CommonParameters>] 
DESCRIPTION 
    Initialize sessions for one or multiple iBMC servers and. This cmdlet has following parameters: 
    - Address - Holds the iBMC server IP/hostname. 
    - Username - Holds  the iBMC server username. 
    - Password - Holds  the iBMC server password. 
    - Credential - Holds the iBMC server Credential. 
    - TrustCert - Using this bypasses the server certificate authentication. 
PARAMETERS 
    -Address <String[]> 
        IP address or Hostname of the iBMC server. 
        Required?                    true 
        Position?                    1 
        Default value 
        Accept pipeline input?       true (ByValue, ByPropertyName) 
        Accept wildcard characters?  false 
    -Username <String[]> 
        Username of iBMC account to access the iBMC server. 
        Required?                    true 
        Position?                    2 
        Default value 
        Accept pipeline input?       true (ByPropertyName) 
        Accept wildcard characters?  false 
    -Password <String[]> 
        Password of iBMC account to access the iBMC server. 
        Required?                    true 
        Position?                    3 
        Default value 
        Accept pipeline input?       true (ByPropertyName) 
        Accept wildcard characters?  false 
    -Credential <PSCredential[]> 
        PowerShell PSCredential object having username and passwword of iBMC account to access the iBMC. 
        Required?                    true 
        Position?                    2 
        Default value 
        Accept pipeline input?       true (ByPropertyName) 
        Accept wildcard characters?  false 
    -TrustCert [<SwitchParameter>] 
        If this switch parameter is present then server certificate authentication is disabled for this iBMC session. 
        If not present, server certificate is enabled by default. 
        Required?                    false 
        Position?                    named 
        Default value                False 
        Accept pipeline input?       false 
        Accept wildcard characters?  false 
    <CommonParameters> 
        This cmdlet supports the common parameters: Verbose, Debug, 
        ErrorAction, ErrorVariable, WarningAction, WarningVariable, 
        OutBuffer, PipelineVariable, and OutVariable. For more information, see 
        about_CommonParameters (https:/go.microsoft.com/fwlink/?LinkID=113216). 
INPUTS 
OUTPUTS 
    RedfishSession[] 
    Returns the created RedfishSession if cmdlet executes successfully. 
    In case of an error or warning, exception will be returned. 
    -------------------------- EXAMPLE 1 -------------------------- 
    PS C:\>$sessions = Connect-iBMC -Address 10.1.1.2 -Username root -Password password 
    PS C:\> $sessions 
    -------------------------- EXAMPLE 2 -------------------------- 
    PS C:\>$credential = Get-Credential 
    PS C:\> $sessions = Connect-iBMC -Address 10.1.1.2 -Credential $credential 
    PS C:\> $sessions 
    -------------------------- EXAMPLE 3 -------------------------- 
    PS C:\>$sessions = Connect-iBMC -Address "10.1.1.2,5,8" -Username root -Password password 
    PS C:\> $sessions 
    -------------------------- EXAMPLE 4 -------------------------- 
    PS C:\>$sessions = Connect-iBMC -Address 10.1.1.2-10 -Username root -Password password 
    PS C:\> $sessions 
    -------------------------- EXAMPLE 5 -------------------------- 
    PS C:\>$sessions = Connect-iBMC -Address 10.1.1.2,10.1.1.3 -Username root -Password password 
    PS C:\> $sessions 
    -------------------------- EXAMPLE 6 -------------------------- 
    PS C:\>$sessions = Connect-iBMC -Address 10.1.1.2,10.1.1.3 -Username user1,user2 -Password password1,password2 
    PS C:\> $sessions 
    -------------------------- EXAMPLE 7 -------------------------- 
    PS C:\>$sessions = Connect-iBMC -Address 2018::2018 -Username root -Password password 
    PS C:\> $sessions 
    This example shows how to connect to a bmc server using ipv6 
    -------------------------- EXAMPLE 8 -------------------------- 
    PS C:\>$sessions = Connect-iBMC -Address "[2018::2018]:8080" -Username root -Password password 
    PS C:\> $sessions 
    This example shows how to connect to a bmc server using ipv6 and port 
    -------------------------- EXAMPLE 9 -------------------------- 
    PS C:\>$sessions = Connect-iBMC -Address "2018::2018,201A" -Username root -Password password 
    PS C:\> $sessions 
    This example shows how to connect to multiple bmc server using "," seperated ipv6 addresses 
    -------------------------- EXAMPLE 10 -------------------------- 
    PS C:\>$sessions = Connect-iBMC -Address "2018::2018-201A" -Username root -Password password 
    PS C:\> $sessions 
    This example shows how to connect to multiple bmc server using "-" seperated ipv6 addresses 
RELATED LINKS 
    https://github.com/Huawei/Huawei-iBMC-Cmdlets

Querying the Huawei-iBMC-Cmdlets Version Information

Function

Query the Huawei-iBMC-Cmdlets version information.

Format

Get-iBMCModuleVersion

Parameters

None

Usage Guidelines

None

Example

Query the current Huawei-iBMC-Cmdlets version.

PS C:\> Get-iBMCModuleVersion

The command output is as follows:

Name        : Huawei-iBMC-Cmdlets 
Version     : 1.3.1 
Path        : C:\Program Files\WindowsPowerShell\Modules\Huawei-iBMC-Cmdlets\Huawei-iBMC-Cmdlets.psm1 
Description : Huawei iBMC cmdlets provide cmdlets to quick access iBMC Redfish devices.               
These cmdlets contains operation used most such as: bois setting, syslog, snmp, network, power and etc.

Connecting to the iBMC

Function

Connect to the iBMC.

Format

Connect-iBMC -Address <Address> -Username <Username> -Password <Password> -TrustCert

Connect-iBMC -Address <Address> -Credential <$Credential> -TrustCert

Parameters

| Parameter | Description |
| --- | --- |
| <Address> | Specifies the IPv4/IPv6 address or domain name of the server iBMC. This parameter is mandatory. Multiple IP addresses can be separated by commas (,) or hyphens (-). If an IPv6 address is used, the IPv6 address must be enclosed in parentheses, for example, [1001::1001]. |
| <Username> | Specifies the iBMC administrator user name. This parameter is optional. Multiple user names can be separated by commas (,). |
| <Password> | Specifies the iBMC administrator password. This parameter is optional. Multiple passwords can be separated by commas (,). |
| <$Credential> | Specifies the encrypted connection to the iBMC. This parameter is optional. For details about how to create an iBMC server credential, see Example. |
| -TrustCert | Specifies bypassing server certificate authentication. This parameter is optional. |
| NOTE When the iBMC is connected in plain text mode, the Username and Password parameters are mandatory. When the iBMC is connected in encryption mode, the Credential parameter is mandatory. |  |

Usage Guidelines

None

Example

# (Recommended) Connect to the iBMC in encryption mode and create a session.

- Create an iBMC server credential.

PS C:\>$credential = Get-Credential 
 cmdlet Get-Credential at command pipeline position 1  
 Supply values for the following parameters: 
 Credential

The Windows PowerShell credential request dialog box is displayed, as shown in Figure 3-1.

Windows PowerShell credential request

- In the Windows PowerShell credential request dialog box, enter the iBMC user name and password.

- Click OK.

- Connect to the iBMC in encryption mode and create a session.

PS C:\>$session = Connect-iBMC -Address 10.1.1.2-3 -Credential $Credential -TrustCert

- View iBMC information in the session.

PS C:\>$session 
Id                  : 1  
 Name                : Manager  
 ManagerType         : BMC  
 FirmwareVersion     : 3.04  
 UUID                : 3EBAF6CC-69CD-11E7-BF57-68CC6E3CDB29  
 Model               : iBMC  
 Health              : OK  
 State               : Enabled  
 DateTime            : 2018-12-16T15:17:52+00:00  
 DateTimeLocalOffset : GMT  
 Address             : 10.1.1.2  
 BaseUri             : https://10.1.1.2  
 Location            : /redfish/v1/SessionService/Sessions/7ca5ad94c4dfc85e  
 Alive               : True  
 AuthToken           : 8b5f6599cdc89a14d36f5299a14952d9  
 TrustCert           : True  
   
 Id                  : 2  
 Name                : Manager  
 ManagerType         : BMC  
 FirmwareVersion     : 3.04  
 UUID                : 3EBAF6CC-69CD-11E7-BF57-68CC6E3CDB29  
 Model               : iBMC  
 Health              : OK  
 State               : Enabled  
 DateTime            : 2018-12-16T15:17:52+00:00  
 DateTimeLocalOffset : GMT  
 Address             : 10.1.1.3  
 BaseUri             : https://10.1.1.3  
 Location            : /redfish/v1/SessionService/Sessions/7ca5ad94c4dfc85e  
 Alive               : True  
 AuthToken           : 8b5f6599cdc89a14d36f5299a14952d9  
 TrustCert           : True

# Connect to a single iBMC and create a session.

- Create a session and connect to the iBMC.

PS C:\>$session = Connect-iBMC -Address 10.1.1.2 -Username username -Password password -TrustCert

- View iBMC information in the session.

PS C:\>$session 
 Id                  : 1  
 Name                : Manager  
 ManagerType         : BMC  
 FirmwareVersion     : 3.04  
 UUID                : 3EBAF6CC-69CD-11E7-BF57-68CC6E3CDB29  
 Model               : iBMC  
 Health              : OK  
 State               : Enabled  
 DateTime            : 2018-12-16T15:17:52+00:00  
 DateTimeLocalOffset : GMT  
 Address             : 10.1.1.2  
 BaseUri             : https://10.1.1.2  
 Location            : /redfish/v1/SessionService/Sessions/7ca5ad94c4dfc85e  
 Alive               : True  
 AuthToken           : 8b5f6599cdc89a14d36f5299a14952d9  
 TrustCert           : True

# Connect to multiple iBMCs and create a session.

- Create a session and connect to multiple iBMCs.

PS C:\> $sessions = Connect-iBMC -Address 10.1.1.2,10.1.1.3 -Username root -Password password

- View iBMC information in the session.

PS C:\> $sessions 
   
 Id                  : 1  
 Name                : Manager  
 ManagerType         : BMC  
 FirmwareVersion     : 3.04  
 UUID                : 3EBAF6CC-69CD-11E7-BF57-68CC6E3CDB29  
 Model               : iBMC  
 Health              : OK  
 State               : Enabled  
 DateTime            : 2018-12-16T15:17:52+00:00  
 DateTimeLocalOffset : GMT  
 Address             : 10.1.1.2  
 BaseUri             : https://10.1.1.2  
 Location            : /redfish/v1/SessionService/Sessions/7ca5ad94c4dfc85e  
 Alive               : True  
 AuthToken           : 8b5f6599cdc89a14d36f5299a14952d9  
 TrustCert           : True  
   
 Id                  : 2  
 Name                : Manager  
 ManagerType         : BMC  
 FirmwareVersion     : 3.04  
 UUID                : 3EBAF6CC-69CD-11E7-BF57-68CC6E3CDB29  
 Model               : iBMC  
 Health              : OK  
 State               : Enabled  
 DateTime            : 2018-12-16T15:17:52+00:00  
 DateTimeLocalOffset : GMT  
 Address             : 10.1.1.3  
 BaseUri             : https://10.1.1.3  
 Location            : /redfish/v1/SessionService/Sessions/7ca5ad94c4dfc85e  
 Alive               : True  
 AuthToken           : 8b5f6599cdc89a14d36f5299a14952d9  
 TrustCert           : True

Output Description

Output description

| Output Item | Description |
| --- | --- |
| Id | Specifies the session ID, which uniquely identifies the session resource. |
| Name | Specifies the session name |
| ManagerType | Specifies the session resource type. |
| FirmwareVersion | Specifies the session resource firmware version. |
| UUID | Specifies the universally unique identifier of the session resource. |
| Model | Specifies the session resource model. |
| Health | Specifies the session resource health status. |
| State | Specifies the session resource enabling status. |
| DateTime | Specifies the session resource system time. |
| DateTimeLocalOffset | Specifies the session resource time zone. |
| Address | Specifies the server IP address. |
| BaseUri | Specifies the network address. |
| Location | Specifies the access address. |
| Alive | Specifies the connection status. |
| AuthToken | Specifies the authentication key. |
| TrustCert | Specifies whether to bypass server certificate authentication. |

Disconnecting from the iBMC

Function

Disconnect from the iBMC.

Format

Disconnect-iBMC -Session <$session>

Parameters

| Parameter | Description |
| --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Disconnect from the iBMC.

PS C:\>Disconnect-iBMC -Session $session 
Id                  : 1 
Name                : Manager 
ManagerType         : BMC 
FirmwareVersion     : 3.16 
UUID                : 02AB0D57-4857-9D57-E811-92DD90161F12 
Model               : iBMC 
Health              : OK 
State               : Enabled 
DateTime            : 2018-12-29T02:09:19+00:00 
DateTimeLocalOffset : GMT 
Address             : 10.1.1.2 
BaseUri             : https://10.1.1.2 
Location            : /redfish/v1/SessionService/Sessions/4c8c5f281cb30981 
Alive               : False 
AuthToken           : ea9a4c9dfd25706f52343a486afe203f 
TrustCert           : True

Output Description

Output description

| Output Item | Description |
| --- | --- |
| Id | Specifies the session ID, which uniquely identifies the session resource. |
| Name | Specifies the session name |
| ManagerType | Specifies the session resource type. |
| FirmwareVersion | Specifies the session resource firmware version. |
| UUID | Specifies the universally unique identifier of the session resource. |
| Model | Specifies the session resource model. |
| Health | Specifies the session resource health status. |
| State | Specifies the session resource enabling status. |
| DateTime | Specifies the session resource system time. |
| DateTimeLocalOffset | Specifies the session resource time zone. |
| Address | Specifies the server IP address. |
| BaseUri | Specifies the network address. |
| Location | Specifies the access address. |
| Alive | Specifies the connection status. |
| AuthToken | Specifies the authentication key. |
| TrustCert | Specifies whether to bypass server certificate authentication. |

Testing the iBMC Connection Status

Function

Test the iBMC connection status.

Format

Test-iBMCConnect -Session <$session>

Parameters

| Parameter | Description |
| --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Test the iBMC connection status.

PS C:\> Test-iBMCConnect -Session $session  
  
Id                  : 1  
Name                : Manager  
ManagerType         : BMC  
FirmwareVersion     : 3.04  
UUID                : 3EBAF6CC-69CD-11E7-BF57-68CC6E3CDB29  
Model               : iBMC  
Health              : OK  
State               : Enabled  
DateTime            : 2018-12-20T17:34:28+00:00  
DateTimeLocalOffset : GMT  
Address             : 10.1.1.2  
BaseUri             : https://10.1.1.2  
Location            : /redfish/v1/SessionService/Sessions/19b0f46c5e524f1f  
Alive               : True  
AuthToken           : f239b99e1a15b0f6f32213c2fe0e2eab  
TrustCert           : True

Output Description

Output description

| Output Item | Description |
| --- | --- |
| Id | Specifies the session ID, which uniquely identifies the session resource. |
| Name | Specifies the session name |
| ManagerType | Specifies the session resource type. |
| FirmwareVersion | Specifies the session resource firmware version. |
| UUID | Specifies the universally unique identifier of the session resource. |
| Model | Specifies the session resource model. |
| Health | Specifies the session resource health status. |
| State | Specifies the session resource enabling status. |
| DateTime | Specifies the session resource system time. |
| DateTimeLocalOffset | Specifies the session resource time zone. |
| Address | Specifies the server IP address. |
| BaseUri | Specifies the network address. |
| Location | Specifies the access address. |
| Alive | Specifies the connection status. |
| AuthToken | Specifies the authentication key. |
| TrustCert | Specifies whether to bypass server certificate authentication. |

Querying the Session Service Information

Function

Query the timeout interval of the current session.

Format

Get-iBMCSessionTimeout -Session <$session>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Query the session timeout interval.

PS C:\> $SessionTimeout = Get-iBMCSessionTimeout -Session $session 
PS C:\> $SessionTimeout 
 
Host           : 10.1.1.2 
SessionTimeout : 600

Output Description

Output description

| Output Item | Description |
| --- | --- |
| Host | Host |
| SessionTimeout | Specifies the session timeout interval, in seconds. |

Modifying the Session Service Information

Function

Modify the timeout interval of the current session service on the server.

Format

Set-iBMCSessionTimeout -Session <$session> -Timeout <Timeout>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |
| <Timeout> | Specifies the session timeout interval. This parameter is mandatory. | The value is an integer ranging from 30 to 86400, and the unit is second. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Change the session timeout interval to 600 seconds.

PS C:\> Set-iBMCSessionTimeout -Session $session -Timeout 600

Querying the User List

Function

Query the iBMC user list.

Format

Get-iBMCUser -Session <$session>

Parameters

| Parameter | Description |
| --- | --- |
| <$session> | Specifies the command session. This parameter is mandatory. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Query the iBMC user list.

PS C:\> Get-iBMCUser -Session $session  
  
Host     : 10.1.1.2 
Id       : 2  
Name     : User Account  
UserName : root  
RoleId : Administrator  
Locked : False  
Enabled  : True  
Oem      : @{Huawei=}  
  
Host     : 10.1.1.2 
Id       : 3  
Name     : User Account  
UserName : 12312ada  
RoleId : Noaccess  
Locked : False  
Enabled  : False  
Oem      : @{Huawei=}

Output Description

Output description

| Output Item | Description |
| --- | --- |
| Host | Specifies the host name. |
| Id | Specifies the user ID. |
| Name | Specifies the account name. |
| UserName | Specifies the user name. |
| RoleId | Specifies the ID of the role resource configured for the account. |
| Locked | Specifies that the account service is automatically locked because the lock threshold is exceeded. When this item is set to True, the account is locked. The user administrator can set this item to False to unlock the account. |
| Enabled | Specifies whether to disable the account that has the user information deletion function. When this item is set to True, the user can log in. When this item is set to False, the account is disabled and the user cannot log in. |
| Oem | Specifies the customized attribute. |

Creating a User

Function

Create a user.

Format

Add-iBMCUser -Session <$session> -Username <Username> -Password <Password> -Role <Role>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |
| <Username> | Specifies the name of the newly created user. This parameter is mandatory. Multiple user names can be separated by commas (,). | - |
| <Password> | Specifies the password of the newly created user. This parameter is mandatory. Multiple passwords can be separated by commas (,). | - |
| <Role> | Specifies the role of the newly created user. This parameter is mandatory. Multiple roles can be separated by commas (,). | Administrator Operator Commonuser Noaccess CustomRole1 CustomRole2 CustomRole3 CustomRole4 |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Set the password of the new user to password.

PS C:\> $pwd = ConvertTo-SecureString -String password -AsPlainText -Force

- Create a user.

PS C:\> ,$sessions | Add-iBMCUser -Username new-user,new-user2 -Password $pwd,$pwd -Role Operator,Administrator  
  
Host     : 10.1.1.2 
Id       : 4  
Name     : User Account  
UserName : new-user  
RoleId : Operator  
Locked : False  
Enabled  : True  
Oem      : @{Huawei=}  
  
Host     : 10.1.1.2 
Id       : 5  
Name     : User Account  
UserName : new-user2  
RoleId : Administrator  
Locked : False  
Enabled  : True  
Oem      : @{Huawei=}

Modifying a User

Function

Modify a user.

Format

Set-iBMCUser -Session <$session> -Username<Username>-NewUsername <NewUsername> -NewPassword <NewPassword> -NewRole <Role> -Enabled <Enabled> -Unlocked<Unlocked >

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |
| <Username> | Specifies the name of the user to be modified. This parameter is mandatory. | - |
| <NewUsername> | Specifies the new name of the user to be modified. This parameter is optional. | - |
| <NewPassword> | Specifies the new password of the user to be modified. This parameter is optional. | - |
| <Role> | Specifies the new role of the user to be modified. This parameter is optional. | Administrator Operator Commonuser Noaccess CustomRole1 CustomRole2 CustomRole3 CustomRole4 |
| <Enabled> | Specifies the enabling status of the user. This parameter is optional. | True: enabled False: disabled |
| <Unlocked > | Specifies the locking status of the user. This parameter is optional. | True: unlocked False: locked |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Set a new password.

PS C:\> $newPwd = ConvertTo-SecureString -String new-password -AsPlainText -Force

- Change the user name and password.

PS C:\> ,$sessions | Set-iBMCUser -Username username -NewUsername new-user2 -NewPassword $newPwd -NewRole Administrator  
  
Host     : 10.1.1.2 
Id       : 12  
Name     : User Account  
UserName : powershell  
RoleId   : Operator  
Locked   : True  
Enabled  : True  
Oem      : @{Huawei=}

Removing a User

Function

Remove a user.

Format

Remove-iBMCUser -Session <$session> -Username <Username>

Parameters

| Parameter | Description |
| --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. |
| <Username> | Specifies the name of the user to be removed. This parameter is mandatory. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Remove the user whose name is user.

PS C:\> ,$session | Remove-iBMCUser -Username user

Querying the iBMC Service Information

Function

Query the iBMC service information.

Format

Get-iBMCServices -Session <$session>

Parameters

| Parameter | Description |
| --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Query the iBMC service information.

PS C:\> $Services = Get-iBMCServices $session  
PS C:\> $Services  
Host         : 10.1.1.2 
HTTP         : @{ProtocolEnabled=True; Port=80} 
HTTPS        : @{ProtocolEnabled=True; Port=443} 
SNMP         : @{ProtocolEnabled=True; Port=161} 
VirtualMedia : @{ProtocolEnabled=True; Port=8208} 
IPMI         : @{ProtocolEnabled=True; Port=623} 
SSH          : @{ProtocolEnabled=True; Port=22} 
KVMIP        : @{ProtocolEnabled=True; Port=2198} 
SSDP         : @{ProtocolEnabled=False; Port=1900; NotifyMulticastIntervalSeconds=600; NotifyTTL=2; NotifyIPv6Scope=Site} 
VNC          : @{ProtocolEnabled=False; Port=5900} 
Video        : @{ProtocolEnabled=True; Port=2199} 
NAT          :

Output Description

Output description

| Output Item | Description |
| --- | --- |
| Host | Specifies the host name. |
| HTTP/HTTPS/SNMP/VirtualMedia/IPMI/SSH/KVMIP/SSDP/VNC/Video/NAT | Specifies the service information supported by the iBMC. Services that are not supported by the switch module are displayed as null. |
| ProtocolEnabled | Specifies the service enabling status. |
| Port | Specifies the service port number. |
| NotifyMulticastIntervalSeconds | Specifies the multicast interval of the SSDP service message. |
| NotifyTTL | Specifies the validity period of the SSDP service message. |
| NotifyIPv6Scope | Specifies the IPv6 multicast range of the SSDP service message. Link: local link range Site: local site range Organization: local organization range |

Setting the iBMC Service Information

Function

Set the iBMC service information.

Format

Set-iBMCService -Session <$Session> -ServiceName <ServiceName> -Enabled <Enabled> -Port <Port>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| $session | Specifies the session. This parameter is mandatory. | - |
| <ServiceName> | Specifies the service information supported by the iBMC. This parameter is mandatory. | HTTP HTTPS SNMP VirtualMedia IPMI SSH KVMIP VNC Video NAT |
| <Enabled> | Specifies the service enabling status. This parameter is mandatory. | True: enabled False: disabled |
| <Port> | Specifies the service port number. This parameter is mandatory. | 1 to 65535 |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Set the iBMC service information.

PS C:\>Set-iBMCService -Session $session -ServiceName VNC -Enabled $true -Port 5900

Querying the Power Consumption Information

Function

Query the power consumption information.

Format

Get-iBMCPowerInfo -Session <$session>

Parameters

| Parameter | Description |
| --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Query the power consumption information.

PS C:\> Get-iBMCPowerInfo -Session $session  
  
Host                 : 10.1.1.2 
Id                   : 0  
Name                 : System Power Control 1  
PowerConsumedWatts : 222 Watts  
MaxConsumedWatts     : 432 Watts  
MinConsumedWatts     : 18 Watts  
AverageConsumedWatts : 183 Watts

Output Description

Output description

| Output Item | Description |
| --- | --- |
| Host | Specifies the host name. |
| Id | Specifies the power supply ID. |
| Name | Specifies the power supply name. |
| PowerConsumedWatts | Specifies the current power. |
| MaxConsumedWatts | Specifies the maximum power consumption. |
| MinConsumedWatts | Specifies the minimum power consumption. |
| AverageConsumedWatts | Specifies the average power consumption. |

Querying the System Information

Function

Query the system information.

Format

Get-iBMCSystemInfo -Session <$session>

Parameters

| Parameter | Description |
| --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Query the system information.

PS C:\> $System = Get-iBMCSystemInfo $session  
PS C:\> $System  
  
Host             : 10.1.1.2 
Id               : 1  
Name             : Computer System  
AssetTag         : my test  
Manufacturer     : Huawei  
Model            : 2288H V5  
SerialNumber     : 2102311TYBN0J3000293  
UUID             : 877AA970-58F9-8432-E811-80345C184638  
HostName         :  
PartNumber       : 02311TYB  
HostingRole      : {ApplicationServer}  
Status           : @{State=Disabled; Health=OK}  
PowerState       : Off  
Boot             : @{BootSourceOverrideTarget=Pxe; BootSourceOverrideEnabled=Continuous; BootSourceOverrideMode=Legacy; BootSourceOverride  
                    Target@Redfish.AllowableValues=System.Object[]}  
TrustedModules :  
BiosVersion      : 0.81  
ProcessorSummary : @{Count=2; Model=Central Processor; Status=}  
MemorySummary : @{TotalSystemMemoryGiB=128; Status=}  
PCIeDevices      : {}  
PCIeFunctions : {}  
Oem              : @{Huawei=}

Output Description

Output description

| Output Item | Description |
| --- | --- |
| Host | Specifies the host name. |
| Id | Specifies the system ID. |
| Name | Specifies the system name. |
| AssetTag | Specifies the system asset tag. |
| Manufacturer | Specifies the system manufacturer. |
| Model | Specifies the system model. |
| SerialNumber | Specifies the system serial number. |
| UUID | Specifies the globally unique ID of the system. |
| HostName | Specifies the host name of the system. |
| PartNumber | Specifies the part number of the memory. |
| HostingRole | Specifies the host role of the system. |
| Status | Specifies the system status. The options are as follows: State: system resource enabling status Health: system resource health status |
| PowerState | Specifies the system power status. |
| Boot | Specifies the boot-related information. |
| BootSourceOverrideTarget | Specifies the current boot device of the system. |
| BootSourceOverrideEnabled | Specifies the enabling status of the system boot parameter. The options are as follows: Disabled: The system boot parameter does not take effect. Once: The system boot parameter takes effect only once, that is, it takes effect upon the next restart. Continuous: The system boot parameter takes effect permanently. |
| BootSourceOverrideMode | Specifies the system boot mode. The options are as follows: Legacy UEFI |
| TrustedModules | Specifies the trusted module of system resources. |
| BiosVersion | Specifies the BIOS version. |
| ProcessorSummary | Specifies the CPU information. |
| Count | Specifies the number of CPUs. |
| Model | Specifies the CPU model. |
| Status | Specifies the CPU health status. |
| MemorySummary | Specifies the memory information. |
| TotalSystemMemoryGiB | Specifies the total system memory capacity. The unit is GB. |
| Status | Specifies the memory health status. |
| PCIeDevices | Specifies the access path of the PCIe device resource node. |
| PCIeFunctions | Specifies the access path of the PCIe function resource node. |
| Oem | Specifies the customized attribute. |

Querying the Ethernet Port Information

Function

Query the Ethernet port information.

Format

Get-iBMCSystemNetworkSettings -Session <$session>

Parameters

| Parameter | Description |
| --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. |

Usage Guidelines

You need to install the iBMA of the corresponding version on the OS to obtain complete Ethernet port information.

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Query the Ethernet port information.

PS C:\> $Interfaces = Get-iBMCSystemNetworkSettings $session  
PS C:\> $Interfaces  
  
Host                : 10.1.1.2 
Id                  : mainboardLOMPort1 
Name                : System Ethernet Interface 
PermanentMACAddress : 04:88:5F:D4:C6:52 
MACAddress          : 04:88:5F:D4:C6:52 
LinkStatus          : LinkUp 
IPv4Addresses       : {@{Address=10.1.1.20; SubnetMask=255.255.0.0; Gateway=10.1.0.1; AddressOrigin=}} 
IPv6Addresses       : {@{Address=2017::d5a; PrefixLength=64; AddressOrigin=SLAAC; AddressState=}, 
                      @{Address=2017::d5a;PrefixLength=64; AddressOrigin=SLAAC; AddressState=}, 
                      @{Address=fe80::4a57:2ff:feab:d5a; PrefixLength=64; AddressOrigin=Static; AddressState=}} 
IPv6DefaultGateway  : fe80::525d:acff:feed:5c27 
InterfaceType       : Physical 
BandwidthUsage      : 0 
BDF                 : 0000:1a:00.0

Output Description

Output description

| Output Item | Description |
| --- | --- |
| Host | Specifies the host name. |
| Id | Specifies the network port ID. |
| Name | Specifies the network port name. |
| PermanentMACAddress | Specifies the permanent MAC address of the network port. |
| MACAddress | Specifies the MAC address. |
| LinkStatus | Specifies the network port link status. |
| IPv4Addresses | Specifies the IPv4 address. |
| IPv6Addresses | Specifies the IPv6 address. |
| IPv6DefaultGateway | Specifies the default IPv6 gateway. |
| InterfaceType | Specifies the network port type. Physical Virtual |
| BandwidthUsage | Specifies the bandwidth usage. |
| BDF | Specifies the BDF number. |

Setting the Power Supply Status

Function

Control the power-on and power-off status of the FRU.

This operation may affect the proper running of the OS.

Format

Set-iBMCFruControl -Session <$session> -FRU <-FRU> -ControlType <ControlType>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |
| <-FRU> | Specifies the FRU value. This parameter is mandatory. | OS Base Fabric FC |
| <ControlType> | Specifies the power control type. This parameter is mandatory. | On: power-on GracefulShutdown: graceful power-off ForceRestart: forcible restart Nmi: non-maskable interrupt ForcePowerCycle: forcible power-off and power-on |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Set the server to be powered off gracefully.

PS C:\> Set-iBMCFruControl -Session $session -FRU OS -ControlType GracefulShutdown

Restarting the iBMC

Function

Restart the iBMC.

Format

Reset-iBMC -Session <$session>

Parameters

| Parameter | Description |
| --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Restart the iBMC.

PS C:\> Reset-iBMC $session

Restarting the System

Function

Control the power-on and power-off status of the server.

This operation may affect the proper running of the OS.

Format

Set-iBMCServerPower -Session <$session> -ResetType <ResetType>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |
| <ResetType> | Specifies the power control type. This parameter is mandatory. | On: power-on ForceOff: forcible power-off GracefulShutdown: graceful power-off ForceRestart: forcible restart Nmi: non-maskable interrupt ForcePowerCycle: forcible power-off and power-on |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Restart the system.

PS C:\> Set-iBMCServerPower -Session $session -ResetType ForceRestart

Querying the Asset Tag

Function

Query the asset tag.

Format

Get-iBMCAssetTag -Session <$session>

Parameters

| Parameter | Description |
| --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Query the asset tag.

PS C:\> Get-iBMCAssetTag -Session $session 
 
Host     : 10.1.1.2 
AssetTag : powershell-asset-tag

Setting the Asset Tag

Function

Set the asset tag.

Format

Set-iBMCAssetTag -Session <$session> -AssetTag <AssetTag>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |
| <AssetTag> | Specifies the asset tag. This parameter is mandatory. | Value range: The value is a string of 1 to 48 characters, consisting of digits, letters, and special characters. If you want to set this parameter to null, set this parameter to a null character string. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Set the asset tag to powershell-asset-tag.

PS C:\> Set-iBMCAssetTag $session -AssetTag 'powershell-asset-tag'

Querying the Virtual Media Information

Function

Query the virtual media information.

Format

Get-iBMCVirtualMedia -Session <$session>

Parameters

| Parameter | Description |
| --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Query the virtual media information.

PS C:\> $VirtualMedia = Get-iBMCVirtualMedia $session  
PS C:\> $VirtualMedia  
  
Host           : 10.1.1.2 
Id             : CD  
Name           : VirtualMedia  
MediaTypes     : {}  
Image          :  
ImageName      :  
ConnectedVia : NotConnected  
Inserted       : False

Output Description

Output description

| Output Item | Description |
| --- | --- |
| Host | Specifies the host name. |
| Id | Specifies the virtual media resource ID. |
| Name | Specifies the virtual media resource name. |
| MediaTypes | Specifies the media image type. CD Floppy USBStick DVD |
| Image | Specifies the image URI. |
| ImageName | Specifies the image name. |
| ConnectedVia | Specifies the connection mode. NotConnected URI Applet Oem |
| Inserted | Specifies whether to insert the virtual media. |

Connecting to the Virtual Media

Function

Connect to the virtual media.

Format

Connect-iBMCVirtualMedia -Session <$session> -ImageFilePath <ImageFilePath>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |
| <ImageFilePath> | Specifies the virtual media image URI. This parameter is mandatory. | Currently, only URIs that use the NFS, CIFS, and HTTPS protocols are supported. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Connect to the virtual media of the NFS protocol.

PS C:\> Connect-iBMCVirtualMedia $session 'cifs://user:password@10.1.1.2/C$/iso/SmartProvisioning-V112.iso' 
 
Host         : 10.1.1.2 
Id           : 1 
Name         : vmm connect task 
ActivityName : [10.1.1.3] vmm connect task 
TaskState    : Completed 
StartTime    : 2018-12-29T02:36:21+00:00 
EndTime      : 2018-12-29T02:36:34+00:00 
TaskStatus   : OK 
TaskPercent  :

Output Description

Output description

| Output Item | Description |
| --- | --- |
| Host | Specifies the host name. |
| Id | Specifies the task ID. |
| Name | Specifies the task name. |
| ActivityName | Specifies the virtual media connection status. |
| TaskState | Specifies the task execution progress. |
| StartTime | Specifies the task start time. |
| EndTime | Specifies the task end time. |
| TaskStatus | Specifies the task execution status. |
| TaskPercent | Specifies the task completion rate. |

Disconnecting from the Virtual Media

Function

Disconnect from the virtual media.

Format

Disconnect-iBMCVirtualMedia -Session <$session>

Parameters

| Parameter | Description |
| --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Disconnect from the virtual media.

PS C:\> $Tasks = Disconnect-iBMCVirtualMedia $session  
PS C:\> $Tasks  
  
Host         : 10.1.1.2 
Id           : 4  
Name         : vmm disconnect status task  
ActivityName : [10.1.1.2] vmm disconnect status task  
TaskState    : Completed  
StartTime    : 2018-11-14T18:05:20+08:00  
EndTime      : 2018-11-14T18:05:20+08:00  
TaskStatus   : OK  
TaskPercent  :

Output Description

Output description

| Output Item | Description |
| --- | --- |
| Host | Specifies the host name. |
| Id | Specifies the task ID. |
| Name | Specifies the task name. |
| ActivityName | Specifies the virtual media connection status. |
| TaskState | Specifies the task execution progress. |
| StartTime | Specifies the task start time. |
| EndTime | Specifies the task end time. |
| TaskStatus | Specifies the task execution status. |
| TaskPercent | Specifies the task completion rate. |

Querying the Boot Device

Function

Query the boot device.

Format

Get-iBMCBootSourceOverride -Session <$session>

Parameters

| Parameter | Description |
| --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Query the boot device.

PS C:\> $BootSourceOverride = Get-iBMCBootSourceOverride $session  
PS C:\> $BootSourceOverride  
  
Host     BootSourceOverrideTarget BootSourceOverrideEnabled 
----     ------------------------ ------------------------- 
10.1.1.2 None                     Disabled

Output Description

Output description

| Output Item | Description |
| --- | --- |
| Host | Specifies the host name. |
| BootSourceOverrideTarget | Specifies the current boot device of the system. The options are as follows: None Pxe Floppy Cd Hdd BiosSetup |
| BootSourceOverrideEnabled | Specifies the enabling status of the system boot parameter. The options are as follows: Disabled: The system boot parameter does not take effect. Once: The system boot parameter takes effect only once, that is, it takes effect upon the next restart. Continuous: The system boot parameter takes effect permanently. |

Setting the Boot Device

Function

Set the boot device.

Format

Set-iBMCBootSourceOverride -Session <$session> -BootSourceOverrideTarget <BootSourceOverrideTarget> -BootSourceOverrideEnabled <BootSourceOverrideEnabled>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |
| <BootSourceOverrideTarget> | Specifies the current boot device of the system. This parameter is mandatory. | None Pxe Floppy Cd Hdd BiosSetup |
| <BootSourceOverrideEnabled> | Specifies the enabling status of the system boot parameter. This parameter is mandatory. | Disabled: The system boot parameter does not take effect. Once: The system boot parameter takes effect only once, that is, it takes effect upon the next restart. Continuous: The system boot parameter takes effect permanently. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Set the boot device.

PS C:\> Set-iBMCBootupSequence -Session $session -BootSourceOverrideTarget @('Pxe','Hdd') -BootSourceOverrideEnabled @('Once', 'Continuous')

Querying the Boot Sequence

Function

Query the boot sequence.

Format

Get-iBMCBootupSequence -Session <$session>

Parameters

| Parameter | Description |
| --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Query the boot sequence.

PS C:\> $Sequence = Get-iBMCBootupSequence $session  
PS C:\> $Sequence  
  
Host           BootupSequence 
----           -------------- 
10.1.1.2       {Pxe, HDD, Cd, Others}

Output Description

Output description

| Output Item | Description |
| --- | --- |
| Host | Specifies the host name. |
| BootupSequence | Specifies the system boot sequence. The value can be any sequence of Hdd, Cd, Pxe, and Others. |

Setting the Boot Sequence

Function

Set the boot sequence.

Format

Set-iBMCBootupSequence -Session <$session> -BootUpSequence <$BootUpSequence>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |
| <$BootUpSequence> | Specifies the system boot sequence. This parameter is mandatory. | The value can be any sequence of Hdd, Cd, Pxe, and Others. |

Usage Guidelines

You can set the parameters of multiple services concurrently.

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Set the boot sequence.

PS C:\> $BootUpSequence = @(@('Pxe', 'Hdd', 'Cd', 'Others'), @('Cd', 'Pxe', 'Hdd', 'Others'))  
PS C:\> Set-iBMCBootupSequence $session $BootUpSequence

Exporting the BIOS Configuration

Function

Export the BIOS configuration.

Format

Export-iBMCBIOSSetting -Session <$session> -DestFilePath<DestFilePath>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |
| <DestFilePath> | Specifies the path for exporting the configuration file. This parameter is mandatory. | If a local temporary directory of the iBMC is used, the directory must be the /tmp directory and a file name must be specified. If a remote directory is used, the value must be in File transfer protocol://User name:Password@Remote server IP address/Directory/File name format. The file transfer protocols include SFTP, HTTPS, NFS, CIFS, and SCP. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Export the BIOS configuration file to the tmp directory of the iBMC.

PS C:\> $Tasks = Export-iBMCBIOSSetting $session '/tmp/bios.xml'  
PS C:\> $Tasks  
  
Host         : 10.1.1.2 
Id           : 4  
Name         : Export Config File Task  
ActivityName : [10.1.1.2] Export Config File Task  
TaskState : Completed  
StartTime : 2018-11-14T17:52:01+08:00  
EndTime      : 2018-11-14T17:53:20+08:00  
TaskStatus : OK  
TaskPercent  : 100%

Output Description

Output description

| Output Item | Description |
| --- | --- |
| Host | Specifies the host name. |
| Id | Specifies the task ID. |
| Name | Specifies the task name. |
| ActivityName | Specifies the task activity name. |
| TaskState | Specifies the task execution status. |
| StartTime | Specifies the task start time. |
| EndTime | Specifies the task end time. |
| TaskStatus | Specifies the connection information return status. |
| TaskPercent | Specifies the task completion rate. |

Importing the BIOS Configuration

Function

Import the BIOS configuration.

Format

Import-iBMCBIOSSetting -Session <$session> -ConfigFilePath <ConfigFilePath>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |
| <ConfigFilePath> | Specifies the import path. This parameter is mandatory. | If a local directory is used, the value must be in Directory\File name or \\Server IP address/Directory/File name format. If a local temporary directory of the iBMC is used, the directory must be the /tmp directory and a file name must be specified. If a remote directory is used, the value must be in File transfer protocol://User name:Password@Remote server IP address/Directory/File name format. The file transfer protocols include SFTP, HTTPS, NFS, CIFS, and SCP. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Import the BIOS configuration.

PS C:\> $Tasks = Import-iBMCBIOSSetting $session 'C:\10.10.10.2.xml'  
PS C:\> $Tasks  
  
Host         : 10.1.1.2 
Id           : 2  
Name         : Import Config File Task  
ActivityName : [10.1.1.3] Import Config File Task  
TaskState : Completed  
StartTime : 2018-11-14T17:54:54+08:00  
EndTime      : 2018-11-14T17:56:06+08:00  
TaskStatus : OK  
TaskPercent  : 100%

Output Description

Output description

| Output Item | Description |
| --- | --- |
| Host | Specifies the host name. |
| Id | Specifies the task ID. |
| Name | Specifies the task name. |
| ActivityName | Specifies the task activity name. |
| TaskState | Specifies the task execution status. |
| StartTime | Specifies the task start time. |
| EndTime | Specifies the task end time. |
| TaskStatus | Specifies the connection information return status. |
| TaskPercent | Specifies the task completion rate. |

Restoring the Factory Settings

Function

Restore the factory settings.

This operation may affect the proper running of the server. Therefore, exercise caution when performing this operation.

Format

Restore-iBMCFactorySetting -Session <$session>

Parameters

| Parameter | Description |
| --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. |

Usage Guidelines

This function is supported only for iBMC 3.08 or later.

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Restore the factory settings.

PS C:\> Restore-iBMCFactorySetting $session

Restoring the Default BIOS Settings

Function

Restore the default BIOS settings.

This operation may affect the proper running of the server. Therefore, exercise caution when performing this operation.

Format

Reset-iBMCBIOSSetting -Session <$session>

Parameters

| Parameter | Description |
| --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. |

Usage Guidelines

Only V5 servers support this function.

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Restore the default BIOS settings.

PS C:\> Reset-iBMCBIOSSetting $session

Querying the NTP Resource Information

Function

Query the NTP resource information.

Format

Get-iBMCNTPSetting -Session <$session>

Parameters

| Parameter | Description |
| --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Query the NTP resource information.

PS C:\> Get-iBMCNTPSetting -Session $session  
  
Host                        : 10.1.1.2 
ServiceEnabled              : True  
PreferredNtpServer          : pre.huawei.com  
AlternateNtpServer          : alt.huawei.com  
NtpAddressOrigin            : Static  
MinPollingInterval          : 10  
MaxPollingInterval          : 12  
ServerAuthenticationEnabled : False

Output Description

Output description

| Output Item | Description |
| --- | --- |
| Host | Specifies the host name. |
| ServiceEnabled | Specifies whether to enable NTP. |
| PreferredNtpServer | Specifies the preferred NTP server. |
| AlternateNtpServer | Specifies the alternate NTP server. |
| NtpAddressOrigin | Specifies the NTP mode. Static IPv4 IPv6 |
| MinPollingInterval | Specifies the minimum polling interval. |
| MaxPollingInterval | Specifies the maximum polling interval. |
| ServerAuthenticationEnabled | Specifies the server authentication enabling status. |

Setting NTP Resource Attributes

Function

Set NTP resource attributes.

Format

Set-iBMCNTPSetting -Session <$session> -ServiceEnabled <ServiceEnabled> -PreferredNtpServer <PreferredNtpServer> -AlternateNtpServer<AlternateNtpServer> -NtpAddressOrigin <NtpAddressOrigin> -MinPollingInterval <MinPollingInterval> -MaxPollingInterval <MaxPollingInterval> -ServerAuthenticationEnabled <ServerAuthenticationEnabled>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |
| <ServiceEnabled> | Specifies whether to enable the NTP service. This parameter is optional. | True: enables the NTP function. False: disables the NTP function. |
| <PreferredNtpServer> | Specifies the preferred NTP server address. This parameter is optional. | IPv4 address IPv6 address Domain name (non-Chinese character string consisting of 1 to 67 characters) |
| <AlternateNtpServer> | Specifies the alternate NTP server address. This parameter is optional. | IPv4 address IPv6 address Domain name (non-Chinese character string consisting of 1 to 67 characters) |
| <NtpAddressOrigin> | Specifies the address mode. This parameter is optional. | Static: configures the address manually. IPv4: obtains the address from the IPv4 protocol stack automatically. IPv6: obtains the address from the IPv6 protocol stack automatically. |
| <MinPollingInterval> | Specifies the minimum polling interval. This parameter is optional. | Value range: The value ranges from 3 to 17 and cannot be greater than the maximum polling interval. |
| <MaxPollingInterval> | Specifies the maximum polling interval. This parameter is optional. | Value range: The value ranges from 3 to 17 and cannot be smaller than the minimum polling interval. |
| <ServerAuthenticationEnabled> | Specifies the server authentication enabling status. This parameter is optional. | True: enables server authentication. False: disables server authentication. |

Usage Guidelines

You can set the parameters of multiple services concurrently.

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Set NTP resource attributes.

PS C:\> Set-iBMCNTPSetting $session -ServiceEnabled $true          -PreferredNtpServer 'pre.huawei.com' -AlternateNtpServer 'alt.huawei.com' `          -NtpAddressOrigin Static -ServerAuthenticationEnabled $false `          -MinPollingInterval 10 -MaxPollingInterval 12

Importing the NTP Group Key

Function

Import the NTP group key.

Format

Import-iBMCNTPGroupKey -Session <$session> -KeyFileUri <KeyFileUri>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |
| <KeyFileUri> | Specifies the path of the key file. This parameter is mandatory. | If a local directory is used, the value must be in Directory\File name or \\Server IP address\Directory\File name format. If a local temporary directory of the iBMC is used, the directory must be the /tmp directory and a file name must be specified. If a remote directory is used, the value must be in File transfer protocol://User name:Password@Remote server IP address/Directory/File name format. The file transfer protocols include SFTP, HTTPS, NFS, CIFS, and SCP. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Import the NTP group key.

PS C:\> Import-iBMCNTPGroupKey -Session $session -KeyFileUri "E:\ntp.keys" 
 
Host         : 10.1.1.2 
Id           : 1 
Name         : ntp certificate import 
ActivityName : [10.1.1.2] ntp certificate import 
TaskState    : Completed 
StartTime    : 2018-12-21T05:51:46+00:00 
EndTime      : 2018-12-21T05:51:49+00:00 
TaskStatus   : OK 
TaskPercent  : 100%

Querying the SMTP Notification List

Function

Query the SMTP notification list.

Format

Get-iBMCSMTPRecipients -Session <$session>

Parameters

| Parameter | Description |
| --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Query the SMTP notification list.

PS C:\> $recipients = Get-iBMCSMTPRecipients -Session $session  
PS C:\> $recipients  
  
Host         : 10.1.1.2 
MemberId     : 0 
Enabled      : False 
EmailAddress : 
Description  : 
 
Host         : 10.1.1.2 
MemberId     : 1 
Enabled      : False 
EmailAddress : 
Description  : 
 
Host         : 10.1.1.2 
MemberId     : 2 
Enabled      : False 
EmailAddress : 
Description  : 
Description  : 
 
Host         : 10.1.1.2 
MemberId     : 3 
Enabled      : False 
EmailAddress : 
Description  :

Output Description

Output description

| Output Item | Description |
| --- | --- |
| Host | Specifies the host name. |
| MemberId | Specifies the ID. |
| Enabled | Specifies the enabling status. |
| EmailAddress | Specifies the receiver email address. |
| Description | Specifies the description of the sent content. |

Setting the SMTP Notification Object

Function

Set the SMTP notification object.

Format

Set-iBMCSMTPRecipient -Session <$session> -MemberId <MemberId> -Enabled <Enabled> -EmailAddress <EmailAddress> -Description <Description>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |
| <MemberId> | Specifies the ID, which uniquely identifies the notification object in the notification object list. This parameter is mandatory. | Only integers are supported. The value ranges from 0 to 3. |
| <Enabled> | Specifies the email enabling status. This parameter is optional. | True: enabled False: disabled |
| <EmailAddress> | Specifies the receiver email address. This parameter is optional. | - |
| <Description> | Specifies the description of the sent content. This parameter is optional. | - |

Usage Guidelines

You can set multiple attributes concurrently.

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Set the SMTP notification object.

PS C:\> Set-iBMCSMTPRecipient $session -MemberId 1 -Enabled $true -EmailAddress r2@huawei.com -Description 'desc'

Querying the SMTP Resource Information

Function

Query the SMTP resource information.

Format

Get-iBMCSMTPSetting -Session <$session>

Parameters

| Parameter | Description |
| --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Query the SMTP resource information.

PS C:\> Get-iBMCSMTPSetting -Session $session  
  
Host                  : 10.1.1.2 
ServiceEnabled        : True  
ServerAddress         : smtp.huawei.com  
TLSEnabled            : True  
AnonymousLoginEnabled : False  
SenderUserName        : smtp-sender  
SenderAddress         : smtp-sender@huawei.com  
EmailSubject          : Server Alert  
EmailSubjectContains  : {HostName, BoardSN, ProductAssetTag}  
AlarmSeverity         : Major

Output Description

Output description

| Output Item | Description |
| --- | --- |
| Host | Specifies the host name. |
| ServiceEnabled | Specifies the SMTP service enabling status. |
| ServerAddress | Specifies the SMTP server address. |
| TLSEnabled | Specifies whether to enable the TLS function. |
| AnonymousLoginEnabled | Specifies whether to use the anonymous mode. |
| SenderUserName | Specifies the user name of the sender. |
| SenderAddress | Specifies the sender email address. |
| EmailSubject | Specifies the email subject. |
| EmailSubjectContains | Specifies the additional information of the subject. HostName BoardSN ProductAssetTag |
| AlarmSeverity | Specifies the alarm sending level. |

Setting SMTP Resource Attributes

Function

Set SMTP resource attributes.

Format

Set-iBMCSMTPSetting -Session <$session> -ServiceEnabled <ServiceEnabled> -ServerAddress <ServerAddress> -TLSEnabled <TLSEnabled> -AnonymousLoginEnabled <AnonymousLoginEnabled> -SenderUserName <SenderUserName> -SenderAddress <SenderAddress> -SenderPassword <SenderPassword> -EmailSubject <EmailSubject> -EmailSubjectContains <EmailSubjectContains> -AlarmSeverity <AlarmSeverity>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |
| <ServiceEnabled> | Specifies the SMTP service enabling status. This parameter is optional. | True: enabled False: disabled |
| <ServerAddress> | Specifies the SMTP server address. This parameter is optional. | - |
| <TLSEnabled> | Specifies whether to enable the TLS function. This parameter is optional. | True: enabled False: disabled |
| <AnonymousLoginEnabled> | Specifies whether to use the anonymous mode. This parameter is optional. | True: anonymous False: non-anonymous |
| <SenderUserName> | Specifies the user name of the sender. This parameter is optional. | - |
| <SenderAddress> | Specifies the email address of the sender. This parameter is optional. | - |
| <SenderPassword> | Specifies the password of the sender. This parameter is optional. | - |
| <EmailSubject> | Specifies the email subject. This parameter is optional. | - |
| <EmailSubjectContains> | Specifies the additional information of the subject. This parameter is optional. | You can select one or more of the following additional items: HostName: indicates the host name. BoardSN: indicates the board serial number. ProductAssetTag: indicates the product asset tag. |
| <AlarmSeverity> | Specifies the alarm sending level. This parameter is optional. | Critical Major Minor Normal |

Usage Guidelines

You can set the parameters of multiple services concurrently.

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Set SMTP resource attributes.

PS C:\> $pwd = ConvertTo-SecureString -String "pwd12#$%^" -AsPlainText -Force  
PS C:\> $ServerIdentifer = ,@('HostName', 'BoardSN')  
PS C:\> Set-iBMCSMTPSetting $session -ServiceEnabled $false -ServerAddress smtp.huawei.com `          -TLSEnabled $false -AnonymousLoginEnabled $false `          -SenderUserName 'Huawei-iBMC' -SenderAddress "powershell@huawei.com"  -SenderPassword $pwd `          -EmailSubject 'iBMC Alarm Notification' -EmailSubjectContains $ServerIdentifer `          -AlarmSeverity Critical

Querying the SNMP Resource Information

Function

Query the SNMP resource information.

Format

Get-iBMCSNMPSetting -Session <$session>

Parameters

| Parameter | Description |
| --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Query the SNMP resource information.

PS C:\> Get-iBMCSNMPSetting -Session $session  
  
Host                : 10.1.1.2 
SnmpV1Enabled       : False  
SnmpV2CEnabled      : False  
SnmpV3Enabled       : True  
LongPasswordEnabled : True  
RWCommunityEnabled  : True  
SnmpV3AuthProtocol  : MD5  
SnmpV3PrivProtocol  : DES

Output Description

Output description

| Output Item | Description |
| --- | --- |
| Host | Specifies the host name. |
| SnmpV1Enabled | Specifies the SNMPv1 enabling status. |
| SnmpV2CEnabled | Specifies the SNMPv2c enabling status. |
| SnmpV3Enabled | Specifies the SNMPv3 enabling status. |
| LongPasswordEnabled | Specifies the long password enabling status. |
| RWCommunityEnabled | Specifies the read-write community name enabling status. |
| SnmpV3AuthProtocol | Specifies the SNMPv3 authentication algorithm. |
| SnmpV3PrivProtocol | Specifies the SNMPv3 encryption algorithm. |

Setting SNMP Resource Attributes

Function

Set SNMP resource attributes.

Format

Set-iBMCSNMPSetting -Session <$session> -SnmpV1Enabled <SnmpV1Enabled> -SnmpV2CEnabled <SnmpV2CEnabled> -LongPasswordEnabled <LongPasswordEnabled> -RWCommunityEnabled <RWCommunityEnabled> -ReadOnlyCommunity <ReadOnlyCommunity> -ReadWriteCommunity <ReadWriteCommunity> -SnmpV3AuthProtocol <SnmpV3AuthProtocol> -SnmpV3PrivProtocol <SnmpV3PrivProtocol>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |
| <SnmpV1Enabled> | Specifies the SNMPv1 enabling status. This parameter is optional. | True: enabled False: disabled |
| <SnmpV2CEnabled> | Specifies the SNMPv2c enabling status. This parameter is optional. | True: enabled False: disabled |
| <LongPasswordEnabled> | Specifies the long password enabling status. This parameter is optional. | True: enabled False: disabled |
| <RWCommunityEnabled> | Specifies the read-write community name enabling status. This parameter is optional. | True: enabled False: disabled |
| <ReadOnlyCommunity> | Specifies the read-only community name. This parameter is optional. | Value requirements: The value cannot contain spaces. The length is 1 to 32 bytes. After the long password is enabled, the length is 16 to 32 bytes. The value must contain at least two characters different from the characters in the password set last time. The read-only community name and read-write community name cannot be the same. When the password complexity check function is enabled, the minimum length is 8 bytes. The password must contain at least two types of the following characters: lowercase letters, uppercase letters, digits, and special characters. |
| <ReadWriteCommunity> | Specifies the read-write community name. This parameter is optional. | Value requirements: The value cannot contain spaces. The length is 1 to 32 bytes. After the long password is enabled, the length is 16 to 32 bytes. The value must contain at least two characters different from the characters in the password set last time. The read-only community name and read-write community name cannot be the same. When the password complexity check function is enabled, the minimum length is 8 bytes. The password must contain at least two types of the following characters: lowercase letters, uppercase letters, digits, and special characters. |
| <SnmpV3AuthProtocol> | Specifies the SNMPv3 authentication algorithm. This parameter is optional. | MD5 SHA |
| <SnmpV3PrivProtocol> | Specifies the SNMPv3 encryption algorithm. This parameter is optional. | DES AES |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Set SNMP resource attributes.

PS C:\> $ReadOnlyCommunity = ConvertTo-SecureString -String "SomeP@ssw0rd1" -AsPlainText -Force  
PS C:\> $ReadWriteCommunity = ConvertTo-SecureString -String "SomeP@ssw0rd2" -AsPlainText -Force  
PS C:\> Set-iBMCSNMPSetting $session -SnmpV1Enabled $false -SnmpV2CEnabled $false `        -LongPasswordEnabled $true -RWCommunityEnabled $true `        -ReadOnlyCommunity $ReadOnlyCommunity -ReadWriteCommunity $ReadWriteCommunity `        -SnmpV3AuthProtocol MD5 -SnmpV3PrivProtocol DES

Querying the SNMP Trap Server

Function

Query the SNMP Trap server.

Format

Get-iBMCSNMPTrapServer -Session <$session>

Parameters

| Parameter | Description |
| --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Query the SNMP Trap notification list.

PS C:\> Get-iBMCSNMPTrapServer -Session $session  
  
Host              : 10.1.1.2 
MemberId          : 0  
BobEnabled        : False  
Enabled           : False  
TrapServerAddress :  
TrapServerPort : 300  
  
Host              : 10.1.1.2 
MemberId          : 1  
BobEnabled        : False  
Enabled           : True  
TrapServerAddress : 10.10.10.3  
TrapServerPort : 310  
  
Host              : 10.1.1.2 
MemberId          : 2  
BobEnabled        : False  
Enabled           : False  
TrapServerAddress : 10.10.10.4  
TrapServerPort : 163  
  
Host              : 10.1.1.2 
MemberId          : 3  
BobEnabled        : True  
Enabled           : True  
TrapServerAddress : 10.10.10.2  
TrapServerPort : 202

Output Description

Output description

| Output Item | Description |
| --- | --- |
| Host | Specifies the host name. |
| MemberId | Specifies the array ID. |
| BobEnabled | Specifies the enabling status of in-band forwarding. |
| Enabled | Specifies the server enabling status. |
| TrapServerAddress | Specifies the server address. |
| TrapServerPort | Specifies the server port number. |

Setting the SNMP Trap Server

Function

Set the SNMP Trap server.

Format

Set-iBMCSNMPTrapServer -Session <$session> -MemberId <MemberId> -Enabled <Enabled> -TrapServerAddress <TrapServerAddress> -TrapServerPort <TrapServerPort>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |
| <MemberId> | Specifies the array ID, which uniquely identifies the notification object in the notification object list. This parameter is mandatory. | Only integers are supported. The value ranges from 0 to 3. |
| <Enabled> | Specifies the server enabling status. This parameter is optional. | True: enabled False: disabled |
| <TrapServerAddress> | Specifies the Trap server address. This parameter is optional. | The value is an IPv4 or IPv6 address or a domain name character string. |
| <TrapServerPort> | Specifies the SNMP Trap port number. This parameter is optional. | 1 to 65535 |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Set the SNMP Trap notification object.

PS C:\> Set-iBMCSNMPTrapServer $session -MemberId 1 -Enabled $true -TrapServerAddress 10.10.10.3 -TrapServerPort 1024

Querying the SNMP Trap Resource Information

Function

Query the SNMP Trap resource information.

Format

Get-iBMCSNMPTrapSetting -Session <$session>

Parameters

| Parameter | Description |
| --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Query the SNMP Trap resource information.

PS C:\> Get-iBMCSNMPTrapSetting -Session $session  
  
Host               : 10.1.1.2 
ServiceEnabled     : True  
TrapVersion        : V2C  
TrapV3User         : UserName  
TrapMode           : EventCode  
TrapServerIdentity : BoardSN  
AlarmSeverity      : Critical

Output Description

Output description

| Output Item | Description |
| --- | --- |
| Host | Specifies the host name. |
| ServiceEnabled | Specifies the Trap function enabling status. |
| TrapVersion | Specifies the SNMP Trap version information. |
| TrapV3User | Specifies the SNMP Trap V3 user name. |
| TrapMode | Specifies the SNMP Trap reporting mode. OID EventCode PreciseAlarm |
| TrapServerIdentity | Specifies the host ID. |
| AlarmSeverity | Specifies the alarm sending level. |

Setting SNMP Trap Resource Attributes

Function

Set SNMP Trap resource attributes.

Format

Set-iBMCSNMPTrapSetting -Session <$session> -ServiceEnabled <ServiceEnabled> -TrapVersion <TrapVersion> -TrapV3User <TrapV3User> -TrapMode <TrapMode> -TrapServerIdentity <TrapServerIdentity> -CommunityName <CommunityName> -AlarmSeverity <AlarmSeverity>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |
| <ServiceEnabled> | Specifies the Trap function enabling status. This parameter is optional. | True: enabled False: disabled |
| <TrapVersion> | Specifies the SNMP Trap version. This parameter is optional. | V1 V2C V3 |
| <TrapV3User> | Specifies the SNMP Trap V3 user name. This parameter is optional. | The value is an existing local user name. This parameter is valid only when TrapVersion is set to V3. |
| <TrapMode> | Specifies the SNMP Trap reporting mode. This parameter is optional. | OID: indicates the OID mode. EventCode: indicates the event code mode. PreciseAlarm: indicates the precise alarm mode. |
| <TrapServerIdentity> | Specifies the user name of the sender. This parameter is optional. This parameter is mandatory when TrapMode is set to OID or PreciseAlarm. | BoardSN ProductAssetTag HostName |
| <CommunityName> | Specifies the SNMP Trap community name. This parameter is optional. | Value requirements: The Trap community name cannot be set when the Trap version is V3. Password complexity check: When this function is enabled, the value must be a combination of at least two types of the following characters: lowercase letters, uppercase letters, digits, and special characters. The length must be 8 to 18 bytes. The value must contain at least two characters different from the characters in the password set last time. When this function is disabled, the length must be 1 to 18 bytes. The value cannot contain spaces. NOTE If TrapV3User is set, the community name is invalid. |
| <AlarmSeverity> | Specifies the alarm sending level. This parameter is optional. | Critical Major Minor Normal |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Set SNMP Trap resource attributes.

PS C:\> $CommunityName = ConvertTo-SecureString -String "SomeP@ssw0rd" -AsPlainText -Force  
PS C:\> Set-iBMCSNMPTrapSetting -Session $session -ServiceEnabled $true -TrapVersion V2C `          -TrapV3User chajian -TrapMode EventCode -TrapServerIdentity BoardSN `          -CommunityName $CommunityName -AlarmSeverity Critical

Querying the Syslog Notification List

Function

Query the Syslog notification list.

Format

Get-iBMCSyslogServer -Session <$session>

Parameters

| Parameter | Description |
| --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Query the Syslog notification list.

PS C:\> $Servers = Get-iBMCSyslogServer $session  
PS C:\> $Servers  
  
Host     : 10.1.1.2 
MemberId : 0  
Enabled  : False  
Address  :  
Port     : 0  
LogType  : {OperationLog, SecurityLog, EventLog}  
  
Host     : 10.1.1.2 
MemberId : 1  
Enabled  : False  
Address  :  
Port     : 0  
LogType  : {OperationLog, SecurityLog, EventLog}  
  
Host     : 10.1.1.2 
MemberId : 2  
Enabled  : False  
Address  :  
Port     : 0  
LogType  : {OperationLog, SecurityLog, EventLog}  
  
Host     : 10.1.1.2 
MemberId : 3  
Enabled  : False  
Address  :  
Port     : 0  
LogType  : {OperationLog, SecurityLog, EventLog}

Output Description

Output description

| Output Item | Description |
| --- | --- |
| Host | Specifies the host name. |
| MemberId | Specifies the array ID. |
| Enabled | Specifies the server enabling status. |
| Address | Specifies the server address. |
| Port | Specifies the Syslog server port number. |
| LogType | Specifies the type of reported logs. |

Setting the Syslog Notification Object

Function

Set the Syslog notification object.

Format

Set-ibmcSyslogServer -Session <$session> -MemberId <MemberId> -Enabled <Enabled> -Address <Address> -Port <Port> -LogType <LogType>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |
| <MemberId> | Specifies the array ID, which uniquely identifies the notification object in the notification object list. This parameter is mandatory. | Only integers are supported. The value ranges from 0 to 3. |
| <Enabled> | Specifies the server enabling status. This parameter is optional. | True: enabled False: disabled |
| <Address> | Specifies the server address. This parameter is optional. | The value is an IPv4 or IPv6 address or a domain name character string. |
| <Port> | Specifies the Syslog server port number. This parameter is optional. | 1 to 65535 |
| <LogType> | Specifies the type of reported logs. This parameter is optional. | You can select one or more of the following log types: OperationLog: indicates that operations logs are reported. SecurityLog: indicates that security logs are reported. EventLog: indicates that event logs are reported. |

Usage Guidelines

You can set the parameters of multiple services concurrently.

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Set the Syslog notification object.

PS C:\> $LogType = ,@("OperationLog", "SecurityLog", "EventLog")  
PS C:\> Set-ibmcSyslogServer $session -MemberId 1 -Enabled $true -Address 10.10.10.2 -Port 515 -LogType $LogType

Querying the Syslog Resource Information

Function

Query the Syslog resource information.

Format

Get-iBMCSyslogSetting -Session <$session>

Parameters

| Parameter | Description |
| --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Query the Syslog resource information.

PS C:\> $syslog = Get-iBMCSyslogSetting $session  
PS C:\> $syslog  
  
Host                 : 10.1.1.2 
ServiceEnabled       : True 
ServerIdentitySource : BoardSN 
AlarmSeverity        : Normal 
TransmissionProtocol : UDP

Output Description

Output description

| Output Item | Description |
| --- | --- |
| Host | Specifies the host name. |
| ServiceEnabled | Specifies the Syslog service enabling status. |
| ServerIdentitySource | Specifies the Syslog host ID. |
| AlarmSeverity | Specifies the alarm severity. Normal Minor Major Critical |
| TransmissionProtocol | Specifies the transmission protocol type. UDP TCP TLS |

Setting the Syslog Resource Information

Function

Set the Syslog resource information.

Format

Set-iBMCSyslogSetting -Session <$session> -ServiceEnabled <ServiceEnabled> -ServerIdentitySource <ServerIdentitySource> -AlarmSeverity <AlarmSeverity> -TransmissionProtocol

<TransmissionProtocol>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |
| <ServiceEnabled> | Specifies the Syslog service enabling status. This parameter is optional. | True: enabled False: disabled |
| <ServerIdentitySource> | Indicates the Syslog host ID. This parameter is optional. | BoardSN: indicates the board serial number. ProductAssetTag: indicates the product asset tag. HostName: indicates the host name. |
| <AlarmSeverity> | Specifies the alarm severity. This parameter is optional. | Critical Major Minor Normal |
| <TransmissionProtocol> | Specifies the transmission protocol type. This parameter is optional. | UDP TCP TLS |

Usage Guidelines

You can set the parameters of multiple services concurrently.

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Set the Syslog resource information.

PS C:\> Set-iBMCSyslogSetting $session -ServiceEnabled $true -ServerIdentitySource HostName `          -AlarmSeverity Major -TransmissionProtocol UDP

Querying the In-Band Firmware Version

Function

Query the in-band firmware version.

Format

Get-iBMCInbandFirmware -Session <$session>

Parameters

| Parameter | Description |
| --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Query the in-band firmware version.

PS C:\> Get-iBMCInbandFirmware $session 
   
 Host                               : 10.1.1.2 
 SR430C-M 1G (SAS3108)@[RAID Card1] : 4.270.00-4382  
 LOM (X722)@[LOM]                   : 3.33 0x80000f09 255.65535.255  
 SPService                          : @{APPVersion=1.09; OSVersion=1.09; DataVersion=1.09}

Querying the Out-of-Band Firmware Version

Function

Query the out-of-band firmware version.

Format

Get-iBMCOutbandFirmware -Session <$session>

Parameters

| Parameter | Description |
| --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Query the out-of-band firmware version.

PS C:\> Get-iBMCOutbandFirmware $session 
  
Host                               : 10.1.1.2 
ActiveBMC                          : 3.18 
BackupBMC                          : 3.18 
Bios                               : 0.81 
MainBoardCPLD                      : 2.02 
chassisDiskBP1CPLD                 : 1.10

Upgrading the In-Band Firmware

Function

Upgrade the in-band firmware.

Format

Update-iBMCInbandFirmware -Session <$session> -Type <Type> -FileUri <FileUri> -SignalFileUri <SignalFileUri>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |
| <Type> | Specifies the firmware type. This parameter is mandatory. | Firmware: RAID controller card and NIC SP: Smart Provisioning |
| <FileUri> | Specifies the path of the upgrade file on the server. This parameter is mandatory. NOTE The firmware upgrade file is in .zip format. The Smart Provisioning upgrade file is in .ISO format. | The directory where the upgrade file is stored cannot contain the following special characters: ¦¦,;&&$¦>>><. Only remote directories are supported. The value must be in File transfer protocol://User name:Password@Reomote server IP address/Directory/File name format. If the value of Type is Firmware, the HTTPS, SFTP, NFS, CIFS, SCP, and FILE file transfer protocols are supported. If the value of Type is SP, only the NFS and CIFS file transfer protocols are supported. |
| <SignalFileUri> | Specifies the digital signature (in .asc format) of the upgrade package. If the value of Type is Firmware, this parameter is mandatory. | The directory where the upgrade file is stored cannot contain the following special characters: ¦¦,;&&$¦>>><. The HTTPS, SFTP, NFS, CIFS, SCP, and FILE file transfer protocols are supported. |

Usage Guidelines

None

Example

# Use a file in a remote directory to upgrade Smart Provisioning, enable Smart Provisioning, and restart the server.

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Upgrade the in-band firmware.

PS C:\> Update-iBMCInbandFirmware -Session $session -Type Firmware ` 
           -FileUri "nfs://10.10.10.2/data/nfs/NIC(X722)-Electrical-05022FTM-FW(3.33).zip" ` 
           -SignalFileUri "nfs://10.10.10.2/data/nfs/NIC(X722)-Electrical-05022FTM-FW(3.33).zip.asc" ` 
           -UpgradeMode Recover

- Enable Smart Provisioning.

PS C:\> Set-iBMCSPService -Session $session -StartEnabled $true -SysRestartDelaySeconds 60

- Restart the server.

PS C:\> Set-iBMCServerPower -Session $session -ResetType ForceRestart

- Query the configuration result resource of the Smart Provisioning service.

PS C:\> $Result = Get-iBMCSPTaskResult -Session $session 
PS C:\> $Result 
 
Host      : 10.1.1.2 
Id        : 1 
Name      : SP Result 
Status    : Finished 
OSInstall : 
Clone     : 
Recover   : 
Upgrade   : @{Progress=Successful; Detail=System.Object[]}

- Check whether the firmware versions are upgraded to the target versions.

PS C:\> Get-iBMCInbandFirmware $session 
 
Host                               : 10.1.1.2 
SR430C-M 1G (SAS3108)@[RAID Card1] : 4.270.00-4382 
LOM (X722)@[LOM]                   : 3.33 0x80000f09 255.65535.255 
SPService                          : @{APPVersion=1.16; OSVersion=1.16; DataVersion=1.16}

# Use a file in a remote directory to upgrade Smart Provisioning.

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Upgrade Smart Provisioning.

PS C:\> Update-iBMCInbandFirmware -Session $session -Type SP ` 
          -FileUri "nfs://10.10.10.2/data/nfs/Firmware.ISO ` 
          -UpgradeMode Recover

- Check whether the firmware versions are upgraded to the target versions.

PS C:\> Get-iBMCInbandFirmware $session 
 
Host                               : 10.1.1.2 
SR430C-M 1G (SAS3108)@[RAID Card1] : 4.270.00-4382 
LOM (X722)@[LOM]                   : 3.33 0x80000f09 255.65535.255 
SPService                          : @{APPVersion=1.16; OSVersion=1.16; DataVersion=1.16}

Upgrading the Out-of-Band Firmware

Function

Upgrade the out-of-band firmware.

Format

Update-iBMCOutbandFirmware -Session <$session> -FileUri <FileUri>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |
| <FileUri> | Specifies the path of the upgrade file on the server. This parameter is mandatory. | The file URL cannot exceed 256 characters. If a local directory is used, the value must be in Directory\File name or \\Server IP address\Directory\File name format. If a remote directory is used, the value must be in File transfer protocol://Remote server IP address/Directory/File name format. The file transfer protocols include HTTPS, SCP, SFTP, CIFS, TFTP, and NFS. |

Usage Guidelines

The upgrade takes effect after the BIOS and CPLD are powered off.

Example

# Upgrade the out-of-band firmware using a local file.

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Upgrade the out-of-band firmware.

PS C:\> Update-iBMCOutbandFirmware -Session $session -FileUri E:\2288H_V5_5288_V5-iBMC-V318.hpm  
  
Host         : 10.1.1.2 
Id           : 1  
Name         : Upgarde Task  
ActivityName : [10.1.1.2] Upgarde Task  
TaskState    : Completed  
StartTime    : 2018-11-23T08:57:45+08:00  
EndTime      : 2018-11-23T09:01:24+08:00  
TaskStatus   : OK  
TaskPercent  : 100%

- Check whether the firmware versions are upgraded to the target versions.

PS C:\> Get-iBMCOutbandFirmware $session 
 
Host                               : 10.1.1.2 
ActiveBMC                          : 3.18 
BackupBMC                          : 3.18 
Bios                               : 0.81 
MainBoardCPLD                      : 2.02 
chassisDiskBP1CPLD                 : 1.10

# Upgrade the out-of-band firmware using a file in a remote directory.

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Upgrade the out-of-band firmware.

PS C:\> Update-iBMCOutbandFirmware -Session $session `          -FileUri nfs://10.10.10.3/data/nfs/2288H_V5_5288_V5-iBMC-V318.hpm  
  
Host         : 10.1.1.2 
Id           : 1  
Name         : Upgarde Task  
ActivityName : [10.1.1.2] Upgarde Task  
TaskState    : Completed  
StartTime    : 2018-11-23T08:57:45+08:00  
EndTime      : 2018-11-23T09:01:24+08:00  
TaskStatus   : OK  
TaskPercent  : 100%

- Check whether the firmware versions are upgraded to the target versions.

PS C:\> Get-iBMCOutbandFirmware $session 
 
Host                               : 10.1.1.2 
ActiveBMC                          : 3.18 
BackupBMC                          : 3.18 
Bios                               : 0.81 
MainBoardCPLD                      : 2.02 
chassisDiskBP1CPLD                 : 1.10

Querying the CPU Health Status

Function

Query the overall CPU health status and component health status.

Format

Get-iBMCProcessorsHealth -Session <$session>

Parameters

| Parameter | Description |
| --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Query the overall CPU health status and component health status.

PS C:\> $health = Get-iBMCProcessorsHealth -Session $session 
 PS C:\> $health 
   
 Host    : 10.1.1.2  
 Summary : @{HealthRollup=OK}  
 ID#1    : @{Health=OK; State=Enabled; DeviceLocator=CPU1}  
 ID#2    : @{Health=OK; State=Enabled; DeviceLocator=CPU2}

Output Description

Output description

| Output Item | Description |
| --- | --- |
| Host | Specifies the host name. |
| Summary | Specifies the overall CPU health status. |
| ID#X | Specifies the health status of the CPU whose ID is X. |

Querying the RAID Controller Card Health Status

Function

Query the overall RAID controller card health status and component health status.

Format

Get-iBMCRAIDControllersHealth -Session <$session>

Parameters

| Parameter | Description |
| --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Query the overall RAID controller card health status and component health status.

PS C:\> $health = Get-iBMCRAIDControllersHealth -Session $session 
 PS C:\> $health 
   
 Host            : 10.1.1.2  
 Summary         : @{HealthRollup=OK}  
 ID#RAIDStorage0 : @{Health=OK; State=Enabled; Name=RAID Card1 Controller}  
 ID#RAIDStorage1 : @{Health=OK; State=Enabled; Name=PCIe Card 5 (RAID) Controller}

Output Description

Output description

| Output Item | Description |
| --- | --- |
| Host | Specifies the host name. |
| Summary | Specifies the overall RAID controller card health status. |
| ID#X | Specifies the health status of the RAID controller card whose ID is X. |

Querying the Memory Health Status

Function

Query the overall memory health status and component health status.

Format

Get-iBMCMemoryHealth -Session <$session>

Parameters

| Parameter | Description |
| --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Query the overall memory health status and component health status.

PS C:\> $health = Get-iBMCMemoryHealth -Session $session 
 PS C:\> $health 
   
 Host                : 10.1.1.2  
 Summary             : @{HealthRollup=OK}  
 ID#mainboardDIMM000 : @{Health=OK; State=Enabled; DeviceLocator=DIMM000}  
 ID#mainboardDIMM010 : @{Health=OK; State=Enabled; DeviceLocator=DIMM010}  
 ID#mainboardDIMM030 : @{Health=OK; State=Enabled; DeviceLocator=DIMM030}  
 ID#mainboardDIMM040 : @{Health=OK; State=Enabled; DeviceLocator=DIMM040}  
 ID#mainboardDIMM100 : @{Health=OK; State=Enabled; DeviceLocator=DIMM100}  
 ID#mainboardDIMM110 : @{Health=OK; State=Enabled; DeviceLocator=DIMM110}  
 ID#mainboardDIMM130 : @{Health=OK; State=Enabled; DeviceLocator=DIMM130}  
 ID#mainboardDIMM140 : @{Health=OK; State=Enabled; DeviceLocator=DIMM140}

Output Description

Output description

| Output Item | Description |
| --- | --- |
| Host | Specifies the host name. |
| Summary | Specifies the overall memory health status. |
| ID#X | Specifies the health status of the DIMM whose ID is X. |

Querying the Power Supply Health Status

Function

Query the overall power supply health status and component health status.

Format

Get-iBMCPowerSuppliesHealth -Session <$session>

Parameters

| Parameter | Description |
| --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Query the overall power supply health status and component health status.

PS C:\> $health = Get-iBMCPowerSuppliesHealth -Session $session 
 PS C:\> $health 
   
 Host       : 10.1.1.2  
 Summary    : @{HealthRollup=Critical}  
 MemberId#0 : @{Health=OK; State=Enabled; Name=PS1}  
 MemberId#1 : @{Health=Critical; State=Enabled; Name=PS2}

Output Description

Output description

| Output Item | Description |
| --- | --- |
| Host | Specifies the host name. |
| Summary | Specifies the overall power supply health status. |
| MemberId#X | Specifies the health status of the PSU whose ID is X. |

Querying the Drive Health Status

Function

Query the overall drive health status and component health status.

Format

Get-iBMCDrivesHealth -Session <$session>

Parameters

| Parameter | Description |
| --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Query the overall drive health status and component health status.

PS C:\> $health = Get-iBMCDrivesHealth -Session $session 
 PS C:\> $health | fl 
   
 Host              : 10.1.1.2  
 Summary           : @{HealthRollup=OK}  
 ID#HDDPlaneDisk0  : @{Health=OK; State=; Name=Disk0}  
 ID#HDDPlaneDisk1  : @{Health=OK; State=; Name=Disk1}  
 ID#HDDPlaneDisk2  : @{Health=OK; State=; Name=Disk2}  
 ID#HDDPlaneDisk3  : @{Health=OK; State=; Name=Disk3}  
 ID#HDDPlaneDisk4  : @{Health=OK; State=; Name=Disk4}  
 ID#HDDPlaneDisk5  : @{Health=OK; State=; Name=Disk5}  
 ID#HDDPlaneDisk6  : @{Health=OK; State=; Name=Disk6}  
 ID#HDDPlaneDisk7  : @{Health=OK; State=; Name=Disk7}  
 ID#HDDPlaneDisk40 : @{Health=OK; State=; Name=Disk40}  
 ID#HDDPlaneDisk41 : @{Health=OK; State=; Name=Disk41}

Output Description

Output description

| Output Item | Description |
| --- | --- |
| Host | Specifies the host name. |
| Summary | Specifies the overall hard drive health status. |
| ID#X | Specifies the health status of the hard drive whose ID is X. |

Querying the NIC Health Status

Function

Query the overall NIC health status and component health status.

Format

Get-iBMCNetworkAdaptersHealth -Session <$session>

Parameters

| Parameter | Description |
| --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Query the overall NIC health status and component health status.

PS C:\> $health = Get-iBMCNetworkAdaptersHealth -Session $session 
 PS C:\> $health 
   
 Host            : 10.1.1.2  
 Summary         : @{HealthRollup=OK}  
 ID#mainboardLOM : @{Health=OK; State=Enabled; Name=mainboardLOM}

Output Description

Output description

| Output Item | Description |
| --- | --- |
| Host | Specifies the host name. |
| Summary | Specifies the overall NIC health status. |
| ID#X | Specifies the health status of the NIC whose ID is X. |

Querying the Fan Health Status

Function

Query the overall fan health status and component health status.

Format

Get-iBMCFansHealth -Session <$session>

Parameters

| Parameter | Description |
| --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Query the overall fan health status and component health status.

PS C:\> $health = Get-iBMCFansHealth -Session $session 
 PS C:\> $health 
   
 Host       : 10.1.1.2  
 Summary    : @{HealthRollup=OK}  
 MemberId#0 : @{Health=OK; State=Enabled; Name=Fan Module1 Front}  
 MemberId#1 : @{Health=OK; State=Enabled; Name=Fan Module2 Front}  
 MemberId#2 : @{Health=OK; State=Enabled; Name=Fan Module3 Front}  
 MemberId#3 : @{Health=OK; State=Enabled; Name=Fan Module4 Front}

Output Description

Output description

| Output Item | Description |
| --- | --- |
| Host | Specifies the host name. |
| Summary | Specifies the overall fan health status. |
| MemberId#X | Specifies the health status of the fan whose ID is X. |

Querying the CPU Information

Function

Query the CPU information.

Format

Get-iBMCProcessors -Session <$session>

Parameters

| Parameter | Description |
| --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Query the CPU information.

PS C:\> $ProcessorsArray = Get-iBMCProcessors -Session $session 
 PS C:\> $ProcessorsArray 
   
 Host                    : 10.1.1.2  
 Id                      : 1  
 ProcessorType           : CPU  
 ProcessorArchitecture   : x86  
 InstructionSet          : x86-64  
 Manufacturer            : Intel(R) Corporation  
 Model                   : Intel(R) Xeon(R) Silver 4110 CPU @ 2.10GHz  
 IdentificationRegisters : 54-06-05-00-FF-FB-EB-BF  
 MaxSpeedMHz             : 4000  
 TotalCores              : 8  
 TotalThreads            : 16  
 Socket                  : 0  
 L1CacheKiB              : 512  
 L2CacheKiB              : 8192  
 L3CacheKiB              : 11264  
 DeviceLocator           : CPU1  
 Position                : mainboard  
 PartNumber              : 41020679  
 Temperature             : 30  
 EnabledSetting          : True  
 FrequencyMHz            : 2100  
 OtherParameters         : 64-bit Capable| Multi-Core| Hardware Thread| Execute Protection| Enhanced Virtualization| Power/Performance Control  
 Status                  : @{State=Enabled; Health=Warning}  
   
 Host                    : 10.1.1.2  
 Id                      : 2  
 ProcessorType           : CPU  
 ProcessorArchitecture   : x86  
 InstructionSet          : x86-64  
 Manufacturer            : Intel(R) Corporation  
 Model                   : Intel(R) Xeon(R) Silver 4110 CPU @ 2.10GHz  
 IdentificationRegisters : 54-06-05-00-FF-FB-EB-BF  
 MaxSpeedMHz             : 4000  
 TotalCores              : 8  
 TotalThreads            : 16  
 Socket                  : 1  
 L1CacheKiB              : 512  
 L2CacheKiB              : 8192  
 L3CacheKiB              : 11264  
 DeviceLocator           : CPU2  
 Position                : mainboard  
 PartNumber              : 41020679  
 Temperature             : 31  
 EnabledSetting          : True  
 FrequencyMHz            : 2100  
 OtherParameters         : 64-bit Capable| Multi-Core| Hardware Thread| Execute Protection| Enhanced Virtualization| Power/Performance Control  
 Status                  : @{State=Enabled; Health=OK}

Output Description

Output description

| Output Item | Description |
| --- | --- |
| Host | Specifies the host name. |
| Id | Specifies the CPU resource ID. |
| ProcessorArchitecture | Specifies the CPU resource architecture. The options are as follows: x86 IA-64 ARM MIPS OEM |
| InstructionSet | Specifies the CPU resource instruction set. The options are as follows: x86 x86-64 IA-64 ARM-A32 ARM-A64 MIPS32 MIPS64 OEM |
| Manufacturer | Specifies the CPU resource manufacturer. |
| Model | Specifies the CPU resource model. |
| IdentificationRegisters | Specifies the CPU resource identification register. |
| MaxSpeedMHz | Specifies the maximum frequency of the CPU resource. |
| TotalCores | Specifies the total number of cores of the CPU resource. |
| TotalThreads | Specifies the total number of threads of the CPU resource. |
| Socket | Specifies the CPU resource slot number. |
| L1CacheKiB | Specifies the L1 cache of the CPU resource. |
| L2CacheKiB | Specifies the L2 cache of the CPU resource. |
| L3CacheKiB | Specifies the L3 cache of the CPU resource. |
| DeviceLocator | Specifies the CPU resource silkscreen. |
| Position | Specifies the CPU resource container. |
| PartNumber | Specifies the CPU resource part number. |
| Temperature | Specifies the CPU resource temperature. |
| EnabledSetting | Specifies the CPU resource enabling status. |
| FrequencyMHz | Specifies the CPU resource frequency. |
| OtherParameters | Specifies other parameters. |
| Status | Specifies the CPU resource status. |

Querying the RAID Controller Card Information

Function

Query the RAID controller card information.

Format

Get-iBMCRAIDControllers -Session <$session>

Parameters

| Parameter | Description |
| --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Query the RAID controller card information.

PS C:\> $RAID = Get-iBMCRAIDControllers -Session $Session 
 PS C:\> $RAID 
   
Host                     : 10.1.1.2 
Id                       : RAIDStorage0 
Name                     : RAID Card1 Controller 
Description              : RAID Controller 
Status                   : @{State=Enabled; Health=OK} 
SpeedGbps                : 12 
FirmwareVersion          : 5.010.00-0839 
SupportedDeviceProtocols : {SAS} 
Manufacturer             : 
Model                    : SAS3508 
SupportedRAIDLevels      : {RAID0, RAID1, RAID5, RAID6...} 
Mode                     : Non-RAID 
CachePinnedState         : False 
SASAddress               : 5505dac310072000 
ConfigurationVersion     : 4.1610.00-0149 
MemorySizeMiB            : 2048 
MaintainPDFailHistory    : True 
CopyBackState            : True 
SmarterCopyBackState     : True 
JBODState                : False 
OOBSupport               : True 
CapacitanceName          : 
CapacitanceStatus        : @{State=Absent; Health=} 
DriverInfo               : @{DriverName=; DriverVersion=} 
DDRECCCount              : 0 
MinStripeSizeBytes       : 65536 
MaxStripeSizeBytes       : 1048576 
Drives                   : {HDDPlaneDisk0, HDDPlaneDisk1}

Output Description

Output description

| Output Item | Description |
| --- | --- |
| Host | Specifies the host name. |
| Id | Specifies the storage resource ID. |
| Name | Specifies the storage controller name. |
| Description | Specifies the storage controller description information. |
| Status | Specifies the storage controller status. The options are as follows: State: indicates the storage controller enabling status. Health: indicates the storage controller health status. |
| SpeedGbps | Specifies the storage controller interface rate. |
| FirmwareVersion | Specifies the storage controller firmware version. |
| SupportedDeviceProtocols | Specifies the protocol type supported by the storage controller. The options are as follows: SPI PCIe AHCI UHCI SAS SATA USB NVMe FC iSCSI FCoE NVMeOverFabrics SMB NFSv3 NFSv4 HTTP HTTPS FTP SFTP |
| Manufacturer | Specifies the storage controller manufacturer. |
| Model | Specifies the storage controller model. |
| SupportedRAIDLevels | Specifies the RAID level supported by the storage controller. The options are as follows: RAID0 RAID1 RAID5 RAID6 RAID10 RAID50 RAID60 |
| Mode | Specifies the storage driver mode. The options are as follows: Non-RAID RAID |
| CachePinnedState | Specifies whether cache pinned is enabled for the storage driver. |
| SASAddress | Specifies the storage controller address. |
| ConfigurationVersion | Specifies the storage controller configuration version. |
| MemorySizeMiB | Specifies the memory capacity of the storage controller. |
| MaintainPDFailHistory | Specifies the enabling status of the fault recording function of the storage driver. |
| CopyBackState | Specifies the enabling status of the copyback function of the storage controller. |
| SmarterCopyBackState | Specifies the enabling status of the SMART error copyback function of the storage controller. |
| JBODState | Specifies the enabling status of the driver pass-through function. |
| OOBSupport | Specifies whether the controller supports out-of-band management. |
| CapacitanceName | Specifies the storage controller BBU name. |
| CapacitanceStatus | Specifies the storage controller BBU status. The options are as follows: Health: indicates the storage controller BBU health status. State: indicates the storage controller BBU enabling status. |
| DriverInfo | Specifies the controller driver information. |
| DDRECCCount | Specifies the correctable memory error count of the storage controller. |
| MinStripeSizeBytes | Specifies the minimum strip size supported by the controller. |
| MaxStripeSizeBytes | Specifies the maximum strip size supported by the controller. |
| Drives | Specifies the list of drives managed by the controller. |

Querying the Memory Information

Function

Query the memory information.

Format

Get-iBMCMemory -Session <$session>

Parameters

| Parameter | Description |
| --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Query the memory information.

PS C:\> $MemoriesArray = Get-iBMCMemory -Session $session 
 PS C:\> $MemoriesArray 
   
 Host                : 10.1.1.2  
 Id                  : mainboardDIMM000  
 CapacityMiB         : 16384  
 Manufacturer        : Samsung  
 OperatingSpeedMhz   : 2133  
 SerialNumber        : 0x177E9BFD  
 PartNumber          :  
 MemoryDeviceType    : DDR4  
 DataWidthBits       : 72  
 RankCount           : 2  
 DeviceLocator       : DIMM000  
 BaseModuleType      : RDIMM  
 Socket              : 0  
 Controller          : 0  
 Channel             : 0  
 Slot                : 0  
 MinVoltageMillivolt : 1200  
 Technology          : Synchronous| Registered (Buffered)  
 Position            : mainboard  
 Status              : @{Health=OK; State=Enabled}  
   
 Host                : 10.1.1.2  
 Id                  : mainboardDIMM001  
 CapacityMiB         : 16384  
 Manufacturer        : Samsung  
 OperatingSpeedMhz   : 2133  
 SerialNumber        : 0x177E9BFE  
 PartNumber          :  
 MemoryDeviceType    : DDR4  
 DataWidthBits       : 72  
 RankCount           : 2  
 DeviceLocator       : DIMM001  
 BaseModuleType      : RDIMM  
 Socket              : 0  
 Controller          : 0  
 Channel             : 0  
 Slot                : 1  
 MinVoltageMillivolt : 1200  
 Technology          : Synchronous| Registered (Buffered)  
 Position            : mainboard  
 Status              : @{Health=OK; State=Enabled}

Output Description

Output description

| Output Item | Description |
| --- | --- |
| Host | Specifies the host name. |
| Id | Specifies the memory resource ID. |
| CapacityMiB | Specifies the memory capacity. The unit is MB. |
| Manufacturer | Specifies the memory resource manufacturer. |
| OperatingSpeedMhz | Specifies the memory resource rate. |
| SerialNumber | Specifies the memory resource serial number. |
| PartNumber | Specifies the memory resource part number. |
| MemoryDeviceType | Specifies the memory resource type. |
| DataWidthBits | Specifies the data bandwidth of the memory resource. |
| RankCount | Specifies the number of ranks of the memory resource. |
| DeviceLocator | Specifies the memory resource silkscreen. |
| BaseModuleType | Specifies the basic module type of the memory resource. The options are as follows: RDIMM UDIMM SO_DIMM LRDIMM Mini_RDIMM Mini_UDIMM SO_RDIMM_72b SO_UDIMM_72b SO_DIMM_16b SO_DIMM_32b |
| Socket | Specifies the slot number of the CPU to which the memory resource belongs. |
| Controller | Specifies the controller ID of the memory resource. |
| Channel | Specifies the channel ID of the memory resource. |
| Slot | Specifies the slot number of the memory resource. |
| MinVoltageMillivolt | Specifies the minimum voltage of the memory resource. |
| Technology | Specifies the memory technology of the memory resource. |
| Position | Specifies the memory container of the memory resource. |
| Status | Specifies the memory resource status. |

Querying the Power Supply Information

Function

Query the power supply information.

Format

Get-iBMCPowerSupplies -Session <$session>

Parameters

| Parameter | Description |
| --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Query the power supply information.

PS C:\> $PowerSuppliesArray = Get-iBMCPowerSupplies -Session $session 
 PS C:\> $PowerSuppliesArray 
   
 Host               : 10.1.1.2  
 FirmwareVersion    : DC:108 PFC:107  
 LineInputVoltage   : 0  
 Manufacturer       : LITEON  
 MemberId           : 0  
 Model              : PS-2152-2H  
 Name               : PS1  
 PartNumber         : 02131336  
 PowerCapacityWatts : 1500  
 PowerSupplyType    :  
 Redundancy         : {@{@odata.id=/redfish/v1/Chassis/1/Power#/Redundancy/0}}  
 SerialNumber       : 2102131336CSJ3005736  
 Status             : @{State=Enabled; Health=OK}  
 ActiveStandby      : Active  
 DeviceLocator      : PS1  
 InputAmperage      : 0  
 OutputAmperage     : 0  
 OutputVoltage      : 0  
 Position           : chassis  
 PowerInputWatts    : 0  
 PowerOutputWatts   : 0  
 Protocol           : PSU  
   
 Host               : 10.1.1.2  
 FirmwareVersion    : DC:108 PFC:107  
 LineInputVoltage   : 225  
 Manufacturer       : LITEON  
 MemberId           : 1  
 Model              : PS-2152-2H  
 Name               : PS2  
 PartNumber         : 02131336  
 PowerCapacityWatts : 1500  
 PowerSupplyType    : AC  
 Redundancy         : {@{@odata.id=/redfish/v1/Chassis/1/Power#/Redundancy/0}}  
 SerialNumber       : 2102131336CSJ3001326  
 Status             : @{State=Enabled; Health=OK}  
 ActiveStandby      : Active  
 DeviceLocator      : PS2  
 InputAmperage      : 0  
 OutputAmperage     : 0.234375  
 OutputVoltage      : 0.046875  
 Position           : chassis  
 PowerInputWatts    : 204  
 PowerOutputWatts   : 188  
 Protocol           : PSU

Output Description

Output description

| Output Item | Description |
| --- | --- |
| Host | Specifies the host name. |
| FirmwareVersion | Specifies the firmware version of the PSU. |
| LineInputVoltage | Specifies the input voltage of the PSU. |
| Manufacturer | Specifies the PSU manufacturer. |
| MemberId | Specifies the PSU ID, which uniquely identifies the PSU in the PSU list. |
| Model | Specifies the PSU model. |
| Name | Specifies the PSU name. |
| PartNumber | Specifies the PSU part number. |
| PowerCapacityWatts | Specifies the output power of the PSU. |
| PowerSupplyType | Specifies the power supply type of the PSU. The options are as follows: Unknown AC DC ACorDC |
| Redundancy | Specifies the list of power supply redundancy groups to which the PSU belongs. The options are as follows: @odata.id: indicates the access path of the power supply redundancy group. |
| SerialNumber | Specifies the PSU serial number. |
| Status | Specifies the PSU status. The options are as follows: State: indicates whether the PSU is enabled. Health: indicates the PSU health status. |
| ActiveStandby | Specifies the active/standby mode of the PSU. The options are as follows: Active Standby |
| DeviceLocator | Specifies the PSU silkscreen. |
| InputAmperage | Specifies the input current of the PSU. |
| OutputAmperage | Specifies the output current of the PSU. |
| OutputVoltage | Specifies the output voltage of the PSU. |
| Position | Specifies the PSU container. |
| PowerInputWatts | Specifies the input power of the PSU. |
| PowerOutputWatts | Specifies the output power of the PSU. |
| Protocol | Specifies the protocol used by the PSU. The options are as follows: PSMI PSU PMBUS PMBUS OEM |

Querying the Drive Information

Function

Query the drive information.

Format

Get-iBMCDrives -Session <$session> -StorageId <StorageId>

Parameters

| Parameter | Description |
| --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. |
| <StorageId> | Specifies the drive ID. This parameter is optional. |

Usage Guidelines

None

Example

# Query all drive information.

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Query all drive information.

PS C:\> $Drives = Get-iBMCDrives -Session $Session 
 PS C:\> $Drives 
   
 Host                          : 10.1.1.2  
 Id                            : HDDPlaneDisk0  
 Name                          : Disk0  
 Model                         : MG04ACA400N  
 Revision                      : FJ3J  
 Status                        : @{State=Enabled; Health=OK}  
 CapacityBytes                 : 3999999721472  
 FailurePredicted              : False  
 Protocol                      : SATA  
 MediaType                     : HDD  
 Manufacturer                  : TOSHIBA  
 SerialNumber                  : 38DGK77LF77D  
 CapableSpeedGbs               : 6  
 NegotiatedSpeedGbs            : 12  
 PredictedMediaLifeLeftPercent :  
 IndicatorLED                  : Off  
 HotspareType                  : None  
 StatusIndicator               : OK  
 Location                      : {@{Info=Disk0; InfoFormat=DeviceName}}  
 DriveID                       : 0  
 FirmwareStatus                : Online  
 HoursOfPoweredUp              : 6056  
 PatrolState                   : DoneOrNotPatrolled  
 Position                      : HDDPlane  
 RebuildProgress               :  
 RebuildState                  : DoneOrNotRebuilt  
 SASAddress                    : {500e004aaaaaaa00, 0000000000000000}  
 SASSmartInformation           :  
 SATASmartInformation          : @{AttributeRevision=; AttributeRevisionNumber=; AttributeItemList=System.Object[]}  
 SpareforLogicalDrives         : {}  
 TemperatureCelsius            : 33  
 Type                          : Disk  
   
 Host                          : 10.1.1.2  
 Id                            : HDDPlaneDisk1  
 Name                          : Disk1  
 Model                         : MG04ACA400N  
 Revision                      : FJ3J  
 Status                        : @{State=Enabled; Health=OK}  
 CapacityBytes                 : 3999999721472  
 FailurePredicted              : False  
 Protocol                      : SATA  
 MediaType                     : HDD  
 Manufacturer                  : TOSHIBA  
 SerialNumber                  : 38DFK62PF77D  
 CapableSpeedGbs               : 6  
 NegotiatedSpeedGbs            : 12  
 PredictedMediaLifeLeftPercent :  
 IndicatorLED                  : Off  
 HotspareType                  : None  
 StatusIndicator               : OK  
 Location                      : {@{Info=Disk1; InfoFormat=DeviceName}}  
 DriveID                       : 1  
 FirmwareStatus                : UnconfiguredGood  
 HoursOfPoweredUp              : 6058  
 PatrolState                   : DoneOrNotPatrolled  
 Position                      : HDDPlane  
 RebuildProgress               :  
 RebuildState                  : DoneOrNotRebuilt  
 SASAddress                    : {500e004aaaaaaa01, 0000000000000000}  
 SASSmartInformation           :  
 SATASmartInformation          : @{AttributeRevision=; AttributeRevisionNumber=; AttributeItemList=System.Object[]}  
 SpareforLogicalDrives         : {}  
 TemperatureCelsius            : 33  
 Type                          : Disk

# Query the specified drive information.

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Query the specified drive information.

PS C:\> $Drives = Get-iBMCDrives -Session $Session -StorageId RAIDStorage0 
PS C:\> $Drives 
 
Host                          : 10.1.1.2 
Id                            : HDDPlaneDisk0 
Name                          : Disk0 
Model                         : MG04ACA400N 
Revision                      : FJ3J 
Status                        : @{State=Enabled; Health=OK} 
CapacityBytes                 : 3999999721472 
FailurePredicted              : False 
Protocol                      : SATA 
MediaType                     : HDD 
Manufacturer                  : TOSHIBA 
SerialNumber                  : 38DGK77LF77D 
CapableSpeedGbs               : 6 
NegotiatedSpeedGbs            : 12 
PredictedMediaLifeLeftPercent : 
IndicatorLED                  : Off 
HotspareType                  : None 
StatusIndicator               : OK 
Location                      : {@{Info=Disk0; InfoFormat=DeviceName}} 
DriveID                       : 0 
FirmwareStatus                : Online 
HoursOfPoweredUp              : 6056 
PatrolState                   : DoneOrNotPatrolled 
Position                      : HDDPlane 
RebuildProgress               : 
RebuildState                  : DoneOrNotRebuilt 
SASAddress                    : {500e004aaaaaaa00, 0000000000000000} 
SASSmartInformation           : 
SATASmartInformation          : @{AttributeRevision=; AttributeRevisionNumber=; AttributeItemList=System.Object[]} 
SpareforLogicalDrives         : {} 
TemperatureCelsius            : 33 
Type                          : Disk

Output Description

Output description

| Output Item | Description |
| --- | --- |
| Host | Specifies the host name. |
| Id | Specifies the drive resource ID. |
| Name | Specifies the drive resource name. |
| Model | Specifies the drive model. |
| Revision | Specifies the drive version information. |
| Status | Specifies the drive status. The options are as follows: State: specifies the drive enabling status. Health: specifies the drive health status. |
| CapacityBytes | Specifies the drive capacity. The unit is byte. |
| FailurePredicted | Specifies whether failure prediction is enabled on the drive. The options are as follows: true false |
| Protocol | Specifies the protocol that the drive complies with. The options are as follows: SPI PCIe AHCI UHCI SAS SATA USB NVMe FC iSCSI FCoE NVMeOverFabrics SMB NFSv3 NFSv4 HTTP HTTPS FTP SFTP |
| MediaType | Specifies the drive media type. The options are as follows: HDD SSD SMR |
| Manufacturer | Specifies the drive manufacturer. |
| SerialNumber | Specifies the drive serial number. |
| CapableSpeedGbs | Specifies the maximum rate of the drive interface. |
| NegotiatedSpeedGbs | Specifies the negotiation rate of the drive interface. |
| PredictedMediaLifeLeftPercent | Specifies the remaining lifetime percentage of the drive. |
| IndicatorLED | Specifies the status of the location indicator of the drive. The options are as follows: Off Blinking |
| HotspareType | Specifies the hot spare status of the drive. The options are as follows: None Global Dedicated |
| StatusIndicator | Specifies the indicator status of the drive. The options are as follows: OK Fail Rebuild PredictiveFailureAnalysis Hotspare InAFailedArray InACriticalArray |
| Location | Specifies the drive silkscreen. |
| DriveID | Specifies the drive ID specified when the logical drive is created. |
| FirmwareStatus | Specifies the drive status. The options are as follows: UnconfiguredGood UnconfigureBad HotSpareDrive Offline Failed Online GettingCopied JBOD UnconfiguredShieded HotSpareShielded ConfiguredShielded Foreign Active Standby Sleep DSTInProgress SMARTOfflineDataCollection SCTCommand Rebuilding |
| HoursOfPoweredUp | Specifies the duration when the drive is powered on. |
| PatrolState | Specifies the drive inspection status. The options are as follows: DoneOrNotPatrolled Patrolling |
| Position | Specifies the drive container. |
| RebuildProgress | Specifies the data rebuild progress of the PCIe SSD. |
| RebuildState | Specifies the data rebuild status of the PCIe SSD. The options are as follows: DoneOrNotRebuilt Rebuilding |
| SASAddress | Specifies the SAS address of the drive. |
| SASSmartInformation | Specifies the SMART information of the SAS interface. |
| SATASmartInformation | Specifies the SMART information of the SATA interface. |
| SpareforLogicalDrives | Specifies the list of logical drives to which the drive belongs when the drive is a dedicated hot spare drive. |
| TemperatureCelsius | Specifies the current temperature of the drive. |
| Type | Specifies the drive type. The options are as follows: Disk SDCard PCIe SSD Card |

Querying the NIC Information

Function

Query all NIC information.

Format

Get-iBMCNetworkAdapters -Session <$session>

Parameters

| Parameter | Description |
| --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Query all NIC information.

PS C:\> $NetworkAdaptersArray = Get-iBMCNetworkAdapters -Session $session 
 PS C:\> $NetworkAdaptersArray 
   
 Host              : 10.1.1.2  
 Id                : mainboardLOM  
 Manufacturer      : Intel  
 Model             : X722  
 Status            : @{State=Enabled; Health=OK}  
 Name              : LOM  
 DriverName        :  
 DriverVersion     :  
 CardManufacturer  : Huawei  
 CardModel         : 2*10GE+2*GE  
 DeviceLocator     : LOM  
 Position          : mainboard  
 NetworkTechnology : {Ethernet}  
 RootBDF           : 0000:19:03.0  
 Configuration     :  
 NetworkPorts      : {@{Name=1; Id=1; PhysicalPortNumber=1; LinkStatus=Down; AssociatedNetworkAddresses=System.Object[]; PortType=OpticalPort; BDF=0000:1a:0  
                     0.0; FirmwarePackageVersion=; DriverVersion=; DriverName=}, @{Name=2; Id=2; PhysicalPortNumber=2; LinkStatus=Down; AssociatedNetworkAdd  
                     resses=System.Object[]; PortType=OpticalPort; BDF=0000:1a:00.1; FirmwarePackageVersion=; DriverVersion=; DriverName=}, @{Name=3; Id=3;  
                     PhysicalPortNumber=3; LinkStatus=Up; AssociatedNetworkAddresses=System.Object[]; PortType=ElectricalPort; BDF=0000:1a:00.2; FirmwarePac  
                     kageVersion=; DriverVersion=; DriverName=}, @{Name=4; Id=4; PhysicalPortNumber=4; LinkStatus=Down; AssociatedNetworkAddresses=System.Ob  
                     ject[]; PortType=ElectricalPort; BDF=0000:1a:00.3; FirmwarePackageVersion=; DriverVersion=; DriverName=}}  
   
 Host              : 10.1.1.2  
 Id                : mainboardMEZZ1  
 Manufacturer      : Intel  
 Model             : 2*82599  
 Status            : @{Health=OK; State=Enabled}  
 Name              : MZ312  
 DriverName        :  
 DriverVersion     :  
 CardManufacturer  : Huawei  
 CardModel         : 4*10G Mezzanine Card  
 DeviceLocator     : mainboard  
 Position          : MEZZ1  
 NetworkTechnology : {Ethernet}  
 NetworkPorts      : {@{Name=1; Id=1; PhysicalPortNumber=1; LinkStatus=; AssociatedNetworkAddresses=System.Object[]; PortType=}, @{Name=2; Id=2; PhysicalPor  
                     tNumber=2; LinkStatus=; AssociatedNetworkAddresses=System.Object[]; PortType=}, @{Name=3; Id=3; PhysicalPortNumber=3; LinkStatus=; Asso  
                     ciatedNetworkAddresses=System.Object[]; PortType=}, @{Name=4; Id=4; PhysicalPortNumber=4; LinkStatus=; AssociatedNetworkAddresses=Syste  
                     m.Object[]; PortType=}}

Output Description

Output description

| Output Item | Description |
| --- | --- |
| Host | Specifies the host name. |
| Id | Specifies the NIC resource ID. |
| Manufacturer | Specifies the chip manufacturer of the NIC. |
| Model | Specifies the NIC model. |
| Status | Specifies the NIC status. The options are as follows: Health: indicates the NIC health status. State: indicates the NIC enabling status. |
| Name | Specifies the external name of the NIC. |
| DriverName | Specifies the driver name of the NIC. |
| DriverVersion | Specifies the driver version of the NIC. |
| CardManufacturer | Specifies the NIC manufacturer. |
| CardModel | Specifies the NIC model. |
| DeviceLocator | Specifies the NIC silkscreen. |
| Position | Specifies the NIC container. |
| NetworkTechnology | Specifies the network protocol of the NIC. The options are as follows: Ethernet FC iSCSI FCoE OPA IB |
| RootBDF | Specifies the root port BDF of the NIC. |
| Configuration | Specifies the NIC configuration information. |
| NetworkPorts | Specifies the collection of network port objects related to the NIC. |

Querying the Fan Information

Function

Query all fan information.

Format

Get-iBMCFans -Session <$session>

Parameters

| Parameter | Description |
| --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Query all fan information.

PS C:\> $FansArray = Get-iBMCFans -Session $session 
 PS C:\> $FansArray 
   
 Host                      : 10.1.1.2  
 MemberId                  : 0  
 Name                      : Fan Module1 Front  
 Reading                   : 4920  
 LowerThresholdNonCritical :  
 LowerThresholdCritical    :  
 LowerThresholdFatal       :  
 UpperThresholdNonCritical :  
 UpperThresholdCritical    :  
 UpperThresholdFatal       :  
 MinReadingRange           :  
 MaxReadingRange           :  
 Status                    : @{State=Enabled; Health=OK}  
 ReadingUnits              : RPM  
 PartNumber                : 02311VSF  
 Position                  : chassis  
 SpeedRatio                : 32  
   
 Host                      : 10.1.1.2  
 MemberId                  : 1  
 Name                      : Fan Module2 Front  
 Reading                   : 4800  
 LowerThresholdNonCritical :  
 LowerThresholdCritical    :  
 LowerThresholdFatal       :  
 UpperThresholdNonCritical :  
 UpperThresholdCritical    :  
 UpperThresholdFatal       :  
 MinReadingRange           :  
 MaxReadingRange           :  
 Status                    : @{State=Enabled; Health=OK}  
 ReadingUnits              : RPM  
 PartNumber                : 02311VSF  
 Position                  : chassis  
 SpeedRatio                : 32

Output Description

Output description

| Output Item | Description |
| --- | --- |
| Host | Specifies the host name. |
| MemberId | Specifies the fan sensor ID. |
| Name | Specifies the fan sensor name. |
| Reading | Specifies the current reading of the fan sensor. |
| LowerThresholdNonCritical | Specifies the lower threshold for a non-critical alarm of the fan sensor. |
| LowerThresholdCritical | Specifies the lower threshold for a critical alarm of the fan sensor. |
| LowerThresholdFatal | Specifies the lower threshold for a fatal alarm of the fan sensor. |
| UpperThresholdNonCritical | Specifies the upper threshold for a non-critical alarm of the fan sensor. |
| UpperThresholdCritical | Specifies the upper threshold for a critical alarm of the fan sensor. |
| UpperThresholdFatal | Specifies the upper threshold for a fatal alarm of the fan sensor. |
| MinReadingRange | Specifies the minimum speed reading of the fan sensor. |
| MaxReadingRange | Specifies the maximum speed reading of the fan sensor. |
| Status | Specifies the fan sensor status. The options are as follows: State: indicates whether the fan sensor is enabled. Health: indicates the fan sensor health status. |
| ReadingUnits | Specifies the unit of the fan speed reading of the fan sensor. The options are as follows: RPM Percent |
| PartNumber | Specifies the fan sensor part number. |
| Position | Specifies the fan sensor container. |
| SpeedRatio | Specifies the current reading of the fan sensor. The unit is percent. |

Querying the Configuration Result Resource of the Smart Provisioning Service

Function

Query the configuration result resource of the Smart Provisioning service.

Format

Get-iBMCSPTaskResult -Session <$session>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Query the configuration result resource of the Smart Provisioning service.

PS C:\> $Result = Get-iBMCSPTaskResult -Session $session 
 PS C:\> $Result 
   
 Host      : 10.1.1.2 
 Id        : 1  
 Name      : SP Result  
 Status    : Finished  
 OSInstall :  
 Clone     :  
 Recover   :  
 RaidCfg   : @{Progress=Successful; Detail=System.Object[]}

- Query details about the RAID configuration result.

PS C:\> $result.RaidCfg | fl 
   
 Progress : Successful  
 Detail   : {@{Status=Successful; Description=import raid config Successfully.; CardModel=LSI3008; StartTime=2019-01-28           14:33:21; Operation=import; EndTime=2019-01-28 14:33:34}, @{Status=Successful; Description=Get RAID           configuration Successfully; CardModel=LSI3008; StartTime=2019-01-28 14:34:30; Operation=export;           EndTime=2019-01-28 14:35:27}}

Output Description

Output description

| Output Item | Description |
| --- | --- |
| Id | Specifies the configuration result resource ID. |
| Name | Specifies the configuration result resource name. |
| Status | Specifies the configuration status. The options are as follows: Idle: idle status Init: being initialized Deploying: being deployed Timeout: deployment timed out Finished: deployment completed |
| OSInstall | Specifies the basic deployment result. |
| Clone | Specifies the clone configuration result. |
| Recover | Specifies the restoration configuration result. |
| RaidCfg | Specifies the RAID configuration result. |

Modifying the Smart Provisioning Service Resource Attributes

Function

Modify the Smart Provisioning service resource attributes.

Format

Set-iBMCSPService -Session <$session> -StartEnabled <StartEnabled> -SysRestartDelaySeconds <SysRestartDelaySeconds>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |
| <StartEnabled> | Specifies whether to enable the server to start from Smart Provisioning. | true: enabled false: disabled |
| <SysRestartDelaySeconds> | Specifies the system restart time. | The value is an integer greater than 0, and the unit is second. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Modify the Smart Provisioning service resource attributes.

PS C:\> Set-iBMCSPService -Session $session -StartEnabled $true -SysRestartDelaySeconds 60

Exporting the RAID Configuration of the Smart Provisioning Service

Function

Export the RAID configuration of the Smart Provisioning service.

Format

Export-iBMCSPRAIDSetting -Session <$session>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Export the RAID configuration of the Smart Provisioning service.

PS C:\> Export-iBMCSPRAIDSetting -Session $Session

Creating the RAID Configuration of the Smart Provisioning Service

Function

Create the RAID configuration of the Smart Provisioning service.

Format

Add-iBMCSPRAIDVolume -Session <$session> -Location <Location> -DeviceName <DeviceName> -CardModel <CardModel> -VolumeName <VolumeName> -CapacityMB <CapacityMB> -BootEnabled <BootEnabled> -RAIDLevel <RAIDLevel> -Drives <Drives> -DeviceName<DeviceName>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |
| <Location> | Specifies the RAID controller card location information. This parameter is mandatory. | - |
| <DeviceName> | Specifies the RAID controller card silkscreen name. This parameter is mandatory. | - |
| <CardModel> | Specifies the RAID controller card model. This parameter is optional. | LSI3008 |
| <VolumeName> | Specifies the logical drive name. This parameter is optional. | - |
| <CapacityMB> | Specifies the preset logical drive size. This parameter is optional. | - |
| <BootEnabled> | Specifies whether to enable the boot drive. This parameter is mandatory. | true false |
| <RAIDLevel> | Specifies the RAID level. This parameter is mandatory. | RAID0 RAID1 RAID10 RAID1E |
| <Drives> | Specifies the ID list of physical member drives. This parameter is mandatory. | Use commas (,) to separate multiple IDs, for example, ,@(0,1,2). |

Usage Guidelines

Only scenarios involving a single LSI SAS3008 RAID controller card are supported.

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Create the RAID configuration of the Smart Provisioning service.

PS C:\> $Drives = ,@(0, 1) 
 PS C:\> $VolumeName = "Volume1" 
 PS C:\> Add-iBMCSPRAIDVolume $session -Location mainboard -DeviceName RAIDCard1 ` 
          -VolumeName $VolumeName -CapacityMB 1048576 -BootEnabled $true ` 
          -RAIDLevel RAID1 -Drives $Drives

- Export the RAID configuration of the Smart Provisioning service.

PS C:\> Export-iBMCSPRAIDSetting  -Session $Session

- Enable the function of starting the server from Smart Provisioning.

PS C:\> Set-iBMCSPService -Session $session -StartEnabled $true -SysRestartDelaySeconds 60

- Restart the server.

PS C:\> Set-iBMCServerPower -Session $session -ResetType ForceRestart

- Query the configuration result during the server restart and Smart Provisioning running processes.

PS C:\> Get-iBMCSPTaskResult -Session $session

- After the Smart Provisioning running is complete, query the RAID configuration resource of the Smart Provisioning service.

PS C:\> Get-iBMCSPRAIDSetting -Session $Session

Deleting the RAID Configuration of the Smart Provisioning Service

Function

Delete the RAID configuration of the Smart Provisioning service.

Format

Clear-iBMCSPRAIDSetting -Session <$session> -Location <Location> -DeviceName <DeviceName> -CardModel <CardModel>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |
| <Location> | Specifies the RAID controller card location information. This parameter is mandatory. | - |
| <DeviceName> | Specifies the RAID controller card silkscreen name. This parameter is mandatory. | - |
| <CardModel> | Specifies the RAID controller card model. This parameter is optional. | LSI3008 |

Usage Guidelines

Only scenarios involving a single LSI SAS3008 RAID controller card are supported.

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Delete the RAID configuration of the Smart Provisioning service.

PS C:\> Clear-iBMCSPRAIDSetting $session -Location mainboard -DeviceName RAIDStorage1

- Export the RAID configuration of the Smart Provisioning service.

PS C:\> Export-iBMCSPRAIDSetting  -Session $Session

- Enable the function of starting the server from Smart Provisioning.

PS C:\> Set-iBMCSPService -Session $session -StartEnabled $true -SysRestartDelaySeconds 60

- Restart the server.

PS C:\> Set-iBMCServerPower -Session $session -ResetType ForceRestart

- Query the configuration result during the server restart and Smart Provisioning running processes.

PS C:\> Get-iBMCSPTaskResult -Session $session

- After the Smart Provisioning running is complete, query the RAID configuration resource of the Smart Provisioning service.

PS C:\> Get-iBMCSPRAIDSetting -Session $Session

Modifying the RAID Configuration of the Smart Provisioning Service

Function

Modify the RAID configuration of the Smart Provisioning service.

Format

Set-iBMCSPRAIDSetting -Session <$session> -Location <Location> -DeviceName <DeviceName> -HotSpareDrives <HotSpareDrives>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |
| <Location> | Specifies the RAID controller card location information. This parameter is mandatory. | - |
| <DeviceName> | Specifies the RAID controller card silkscreen name. This parameter is mandatory. | - |
| <HotSpareDrives> | Specifies the global hot spare drive list. This parameter is mandatory. | Use commas (,) to separate multiple IDs, for example, ,@(0,1,2). |
| <CardModel> | Specifies the RAID controller card model. This parameter is optional. | LSI3008 |

Usage Guidelines

Only scenarios involving a single LSI SAS3008 RAID controller card are supported.

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Modify the RAID configuration of the Smart Provisioning service.

PS C:\> $HotSpareDrives = ,@(0,1) 
 PS C:\> Set-iBMCSPRAIDSetting $session -Location mainboard -DeviceName RAIDStorage1 -HotSpareDrives $HotSpareDrives

- Export the RAID configuration of the Smart Provisioning service.

PS C:\> Export-iBMCSPRAIDSetting  -Session $Session

- Enable the function of starting the server from Smart Provisioning.

PS C:\> Set-iBMCSPService -Session $session -StartEnabled $true -SysRestartDelaySeconds 60

- Restart the server.

PS C:\> Set-iBMCServerPower -Session $session -ResetType ForceRestart

- Query the configuration result during the server restart and Smart Provisioning running processes.

PS C:\> Get-iBMCSPTaskResult -Session $session

- After the Smart Provisioning running is complete, query the RAID configuration resource of the Smart Provisioning service.

PS C:\> Get-iBMCSPRAIDSetting -Session $Session

Querying the RAID Configuration Resource of the Smart Provisioning Service

Function

Query the RAID configuration resource of the Smart Provisioning service.

Format

Get-iBMCSPRAIDSetting -Session <$session>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |

Usage Guidelines

Before querying the configuration resource, export the current RAID configuration of the Smart Provisioning service according to 3.70 Exporting the RAID Configuration of the Smart Provisioning Service.

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Export the RAID configuration of the Smart Provisioning service.

PS C:\> Export-iBMCSPRAIDSetting  -Session $Session

- Enable the function of starting the server from Smart Provisioning.

PS C:\> Set-iBMCSPService -Session $session -StartEnabled $true -SysRestartDelaySeconds 60

- Restart the server.

PS C:\> Set-iBMCServerPower -Session $session -ResetType ForceRestart

- Query the configuration result during the server restart and Smart Provisioning running processes.

PS C:\> Get-iBMCSPTaskResult -Session $session

- After the Smart Provisioning running is complete, query the RAID configuration resource of the Smart Provisioning service.

PS C:\> Get-iBMCSPRAIDSetting -Session $Session 
   
 Host           : 10.1.1.2  
 Id             : mainboardRaidCard1  
 Name           : SP RAID Current Configuration  
 CardModel      : LSI3008  
 DeviceName     : RAIDCard1  
 GlobalHotSpare : {5, 6}  
 Location       : mainboard  
 DriveGroupList : {@{VolumeList=System.Object[]; VolumeRaidLevel=RAID1; Drives=System.Object[]}}

Output Description

Output description

| Output Item | Description |
| --- | --- |
| Host | Specifies the host name. |
| Id | Specifies the ID of the current configuration resource of the RAID controller card. |
| Name | Specifies the name of the current configuration resource of the RAID controller card. |
| CardModel | Specifies the RAID controller card model. The options are as follows: LSI3008 |
| DeviceName | Specifies the silkscreen name of the RAID controller card. |
| GlobalHotSpare | Specifies the global hot spare drive list. |
| Location | Specifies the RAID controller card location information. |
| DriveGroupList | Specifies the drive group information. |

Querying the OS Installation Configuration Resources of the Smart Provisioning Service

Function

Query information about the OS installation configuration resources of the Smart Provisioning service. This resource is under license control and can be used only with a license.

Format

Get-iBMCOSDeployConfig -Session <$session>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Query the OS installation configuration resources of the Smart Provisioning service.

PS C:\> $DeployConfig = Get-ibmcOSDeployConfig $session 
PS C:\> $DeployConfig 
 
Host          : 10.10.1.2 
Id            : 1 
Name          : SP OS Install Parameter 
InstallMode   : Recommended 
OSType        : Win2016 
BootType      : UEFIBoot 
CDKey         : *****-*****-*****-*****-***** 
RootPwd       : ******* 
HostName      : huawei 
Autopart      : False 
AutoPosition  : True 
Language      : en-US 
TimeZone      : Eastern Standard Time 
Keyboard      : 0x00000409 
CheckFirmware : False 
Partition     : {@{Name=C; FileSystem=swap; Size=32}} 
Software      : {@{FileName=iBMA }} 
NetCfg        : {@{Device=; IPv4Addresses=System.Object[]; IPv6Addresses=System.Object[]; NameServers=System.Object[]}} 
Packages      : {@{PackageName=System.Object[]; PatternName=System.Object[]}}

Output Description

Output description

| Output Item | Description |
| --- | --- |
| Host | Specifies the host name. |
| Id | Specifies the ID of the OS installation configuration resource. |
| Name | Specifies the name of the OS installation configuration resource. |
| InstallMode | Specifies the installation mode. |
| OSType | Specifies the OS type. |
| BootType | Specifies the BIOS boot mode. |
| CDKey | Specifies the installation key. |
| RootPwd | Specifies the initial administrator password. |
| HostName | Specifies the host name. |
| Autopart | Specifies the automatic partition. |
| AutoPosition | Specifies whether automatic selection of the installation disk is supported. |
| Language | Specifies the language. |
| TimeZone | Specifies the time zone. |
| Keyboard | Specifies the keyboard. |
| CheckFirmware | Specifies the firmware check function. |
| Partition | Specifies the partition information. |
| Software | Specifies the software information. |
| NetCfg | Specifies the network configuration information. |
| Packages | Specifies the list of modules to be installed during the Linux system deployment. |

Creating the OS Installation Configuration for the Smart Provisioning Service

Function

Create the OS installation configuration for the Smart Provisioning service. This resource is under license control and can be used only with a license.

Format

Set-iBMCOSDeployConfig -Session <$session> -ConfigFileURI <ConfigFileURI>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |
| <ConfigFileURI> | Specifies the configuration file path of the OS to be deployed by using Smart Provisioning. | The configuration file path must be a local path. The configuration file must be in the JSON format. For details about the parameters in the configuration file, see Table 3-42. |

Parameter description

| Parameter | Description | Value |
| --- | --- | --- |
| InstallMode | Specifies the installation mode. This parameter is mandatory. | The option is as follows: Recommended |
| OSType | Specifies the type of the OS to be installed. This parameter is mandatory. | The options are as follows: RHEL6U9 RHEL7U3 RHEL7U4 RHEL7U5 CentOS6U9 CentOS7U3 CentOS7U4 CentOS7U5 ESXi6.0 ESXi6.5 ESXi6.7 SLES11SP4 SLES12SP2 SLES12SP3 Ubuntu16.04 Ubuntu16.04.1 Ubuntu16.04.2 Win2016 Win2016 Standard Desktop Win2016 Standard Core Win2016 Datacenter Desktop Win2016 Datacenter Core Win2012_R2 Win2012_R2 Standard Desktop Win2012_R2 Standard Core Win2012_R2 Datacenter Desktop Win2012_R2 Datacenter Core EulerOSV2SP3 |
| BootType | Specifies the BIOS boot mode. This parameter is optional. | The options are as follows: UEFIBoot LegacyBoot SecureBoot |
| CDKey | Specifies the installation key of the Windows OS. This parameter is optional. | For Windows, this parameter is optional and can be set to a 25-character value with every five characters connected by a hyphen (-). The value can contain uppercase letters (A to Z), lowercase letters (a to z), and digits (0 to 9). For Linux, this parameter is left empty. For VMware, this parameter is left empty. |
| RootPwd | Specifies the initial administrator password. This parameter is mandatory. | For Windows, the value is a sting of at least six characters. For SUSE, the value is a sting of at least six characters. For CentOS, Redhat and EulerOS, the value a sting of at least six characters excluding #, $, and space. For VMware, the value a string of at least seven characters. For the ESXi 6.7, the password must consist of letters, digits, and special characters. |
| HostName | Specifies the host name. This parameter is optional. | The value contains a maximum of 15 characters, including uppercase letters (A to Z), lowercase letters (a to z), digits (0 to 9), and hyphens (-). For Linux, this parameter is optional and takes effect only after the network is configured. For Windows, this parameter is optional. For VMware, this parameter is optional and takes effect only after the network is configured. |
| Language | Specifies the system language. This parameter is mandatory. For details about examples, see Table 3-43. | The parameter is a character string. For details, see the installation guide of each OS. For Linux, this parameter is mandatory. For Windows, this parameter is mandatory. For VMware, this parameter is left empty. |
| TimeZone | Specifies the system time zone. This parameter is mandatory. For details about examples, see Table 3-43. | The parameter is a character string. For details, see the installation guide of each OS. For Linux, this parameter is mandatory. For Windows, this parameter is mandatory. For VMware, this parameter is left empty. |
| Keyboard | Specifies the system keyboard layout. This parameter is mandatory. For details about examples, see Table 3-43. | The parameter is a character string. For details, see the installation guide of each OS. For Linux, this parameter is mandatory. For Windows, this parameter is mandatory. For VMware, this parameter is left empty. |
| CheckFirmware | Specifies whether to verify firmware. This parameter is mandatory. | The options are as follows: true false |
| AutoPosition | Specifies whether automatic selection of the installation disk is supported. This parameter is mandatory. | The option is as follows: true (The installation disk can only be automatically selected now.) |
| Autopart | Specifies whether the automatic partitioning is supported. This parameter is mandatory. | The options are as follows: For Linux and VMware, the value is true. For Windows, the value is false. |
| Partition | Specifies the partition information. This parameter is optional. Format: { "Name": "string", "FileSystem": " NTFS ", "Size": "string" } | Object. The value is a partition list. For Windows, the value of Name is any uppercase letter from C to Z. The value of FileSystem is NTFS. The value of Size is greater than 32. If it is set to max, the entire disk is used as the data disk. For Linux, the value of Name does not contain special characters <>¦:& or spaces, such as /, /home, and swap. The value of FileSystem is ext4, ext3, ext2 or xfs. The value of Size is greater than 0. The size of the root partition must be greater than 10, and the size of the swap partition must be greater than 1. If the value is set to max, the remaining space will be allocated. VMware does not support this function. |
| Software | Specifies the software list. This parameter is mandatory. Format: { "FileName": "iBMA" } | Object array. The value is the list of software to be installed. The option is as follows: iBMA |
| Device | Specifies the device information. This parameter is mandatory. Silkprint is optional, and Name and MAC are mandatory. If Silkprint is entered, the values of Name and MAC will not be used. Format: { "Name": "eth0", "MAC":"00:00:00:00:00:00", "Silkprint": { "Location":"mainboard", "DeviceName": "PCIeCard1", "Port": "1" } } | Object. The value is the information of the device for which the network needs to be configured. Name: indicates the device name. MAC: indicates the device MAC address. Silkprint: indicates the NIC silkscreen information. Location: indicates the location parameter. Device: indicates the device name. Port: indicates the port number. |
| IPv4Addresses | Specifies the IPv4 address information of the network port. This parameter is mandatory. Format: { "Address":"Address", "SubnetMask":"SubnetMask", "AddressOrigin":"AddressOrigin", "Gateway":"Gateway" } | Object array. The value is the information of the IPv4 to be configured. Address: indicates the IPv4 address. SubnetMash: indicates the subnet mask. AddressOrigin: indicates the mode for obtaining the IPv4 address, of which the value can be Static or DHCP. Gateway: indicates the IPv4 gateway address. |
| IPv6Addresses | Specifies the IPv6 address information of the network port. This parameter is mandatory. Format: { "Address":"Address", "PrefixLength":"PrefixLength", "AddressOrigin":"AddressOrigin" , "Gateway":"Gateway" } | Object array. The value is the information of the IPv6 to be configured. Address: indicates the IPv6 address. PrefixLength: indicates the prefix length of the IPv6 address. AddressOrigin: indicates the mode for obtaining the IPv6 address, of which the value can be Static or DHCP. Gateway: indicates the IPv6 gateway address. |
| NameServers | Specifies the DNS server address. This parameter is optional. Format: { "DNS":"127.0.0.1" } | Specifies the IP address of the DNS server. The value can be an IPv4 or IPv6 address. |
| PackageName | Specifies the package name. This parameter is optional. Format: { "PackageName": ["gcc","aspell"] } | Character string array. During the Linux deployment process, select the packages to be installed. The value can contain uppercase letters (A to Z), lowercase letters (a to z), digits (0 to 9), hyphens (-), underscores (_), and spaces. One or more values can be specified. |
| PatternName | Specifies the pattern group name. This parameter is optional. Format: { "PatternName": ["x11","base"] } | Character string array. During the Linux deployment process, select the pattern groups to be installed. The value can contain uppercase letters (A to Z), lowercase letters (a to z), digits (0 to 9), hyphens (-), underscores (_), and spaces. One or more values can be specified. |

Reference examples

| OSType | Language | TimeZone | Keyboard |
| --- | --- | --- | --- |
| RHEL/CentOS/EulerOS/Ubuntu | en_US.UTF-8 | America/New_York | us |
| SLES | en_US | America/New_York | english-us |
| Windows | en-US | Eastern Standard Time | 0x00000409 |

Usage Guidelines

None

Example

# Deploy the OS.

- Create the configuration of the system deployment.

  - Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

  - Create the configuration of the system deployment.

PS C:\> $ConfigFileURI = 'C:\ibmc-os-deploy-config-winserver2016.json' 
PS C:\> Set-iBMCOSDeployConfig -Session $session -ConfigFileURI $ConfigFileURI 
 
Host          : 10.10.1.2 
Id            : 1 
Name          : SP OS Install Parameter 
InstallMode   : Recommended 
OSType        : Win2016 
BootType      : UEFIBoot 
CDKey         : *****-*****-*****-*****-***** 
RootPwd       : ******* 
HostName      : huawei 
Autopart      : False 
AutoPosition  : True 
Language      : en-US 
TimeZone      : Eastern Standard Time 
Keyboard      : 0x00000409 
CheckFirmware : False 
Partition     : {@{Name=C; FileSystem=swap; Size=32}} 
Software      : {@{FileName=iBMA }} 
NetCfg        : {@{Device=; IPv4Addresses=System.Object[]; IPv6Addresses=System.Object[]; NameServers=System.Object[]}} 
Packages      : {@{PackageName=System.Object[]; PatternName=System.Object[]}}

- Connect to the virtual media.

PS C:\> $OSImageFileURI = 'nfs://10.10.10.3/winserver2016.iso' 
PS C:\> Connect-iBMCVirtualMedia $session -ImageFilePath $OSImageFileURI

- Enable the function of starting the server from Smart Provisioning.

PS C:\> Set-iBMCSPService -Session $session -StartEnabled $true -SysRestartDelaySeconds 60

- Restart the OS.

PS C:\> Set-iBMCServerPower -Session $session -ResetType ForceRestart

Restoring the Default Configuration of a Specified RAID Controller Card

Function

Restore the default configuration of a specified RAID controller card.

Format

Restore-iBMCRAIDController -Session <$session> -StorageId <StorageId>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |
| <StorageId> | Specifies the RAID controller card ID. This parameter is mandatory. | Set this parameter based on the ID queried in 3.62 Querying the RAID Controller Card Information. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- estore the default configuration of a specified RAID controller card.

PS C:\> Restore-iBMCRAIDController -Session $Session -StorageId RAIDStorage0

Modifying the Information of a Specified RAID Controller Card Resource

Function

Modify the information of a specified RAID controller card resource.

Format

Set-iBMCRAIDController -Session <$session> -StorageId <StorageId> -CopyBackEnabled <CopyBackEnabled> -SmarterCopyBackEnabled <SmarterCopyBackEnabled> -JBODEnabled <JBODEnabled>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |
| <StorageId> | Specifies the RAID controller card ID. This parameter is mandatory. | Set this parameter based on the ID queried in 3.62 Querying the RAID Controller Card Information. |
| <CopyBackEnabled> | Specifies the copyback function enabling status. | true false |
| <SmarterCopyBackEnabled> | Specifies the enabling status of the SMART error copyback function. | true false NOTE If this parameter is set to true, CopyBackEnabled must be set to true. |
| <JBODEnabled> | Specifies the enabling status of the driver pass-through function. | true false |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Modify the information of a specified RAID controller card resource.

PS C:\> Set-iBMCRAIDController -Session $session -StorageId RAIDStorage0 -CopyBackEnabled $true -SmarterCopyBackEnabled $true -JBODEnabled $true

Modifying the Attributes of a Specified Drive

Function

Modify the attributes of a specified drive.

Format

Set-iBMCDrive -Session <$session> -DriveId <DriveId> -State <State> -LEDState <LEDState> -HotSpareType <HotSpareType> -VolumeId <VolumeId>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |
| <DriveId> | Specifies the drive resource ID. This parameter is mandatory. | Set this parameter based on the ID queried in 3.65 Querying the Drive Information. |
| <State> | Specifies the drive status. This parameter is optional. | The drive status can be switched between the following states: Online and Offline UnconfiguredGood and JBOD UnconfigureBad and UnconfiguredGood NOTE Before setting the drive status to JBOD, run the Set-iBMCRAIDController command to enable the JBOD function of the controller. For details, see 3.78 Modifying the Information of a Specified RAID Controller Card Resource. |
| <LEDState> | Specifies the location indicator status. This parameter is optional. | Off Blinking |
| <HotSpareType> | Specifies the hot spare status of the drive. This parameter is optional. | None Global Dedicated |
| <VolumeId> | Specifies the ID of the associated logical drive when the drive is a dedicated hot spare disk. NOTE You do not need to set this parameter when HotSpareType is None or Global. | Set this parameter based on the ID queried in 3.80 Querying the Logical Drive Resource Information. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Modify the attributes of a specified drive.

Change the drive status to JBOD.

PS C:\> Set-iBMCDrive -Session $session -DriveId HDDPlaneDisk0 -State JBOD

Change the hot spare status of the logical drive whose ID is LogicalDrive0 to Dedicated.

PS C:\> Set-iBMCDrive -Session $session -DriveId HDDPlaneDisk0 -HotSpareType Dedicated -VolumeId LogicalDrive0

Querying the Logical Drive Resource Information

Function

Query the logical drive information about of a specified RAID controller card.

Format

Get-iBMCVolume -Session <$session> -StorageId <StorageId>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |
| <StorageId> | Specifies the storage resource ID. | Set this parameter based on the ID queried in 3.62 Querying the RAID Controller Card Information. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Query the information of a specified logical drive resource.

PS C:\> $Volumes = Get-iBMCVolume -Session $Session -StorageId RAIDStorage0 
 PS C:\> $Volumes  
   
 Host                      : 10.1.1.2  
 Id                        : LogicalDrive0  
 Name                      : LogicalDrive0  
 CapacityBytes             : 1099511627776  
 VolumeType                : Mirrored  
 OptimumIOSizeBytes        : 1048576  
 Status                    : @{State=Enabled; Health=OK}  
 VolumeName                : Volume-ps  
 RaidControllerID          : 0  
 VolumeRaidLevel           : RAID1  
 DefaultReadPolicy         : NoReadAhead  
 DefaultWritePolicy        : WriteBackWithBBU  
 DefaultCachePolicy        : DirectIO  
 ConsistencyCheck          : False  
 SpanNumber                : 1  
 NumDrivePerSpan           : 2  
 Spans                     : {@{SpanName=Span0; Drives=System.Object[]}}  
 CurrentReadPolicy         : NoReadAhead  
 CurrentWritePolicy        : WriteBackWithBBU  
 CurrentCachePolicy        : DirectIO  
 AccessPolicy              : ReadWrite  
 BootEnable                : True  
 BGIEnable                 : True  
 SSDCachecadeVolume        : False  
 SSDCachingEnable          : False  
 AssociatedCacheCadeVolume : {}  
 DriveCachePolicy          : Unchanged  
 OSDriveName               :  
 InitializationMode        : UnInit

Output Description

Output description

| Output Item | Description |
| --- | --- |
| Host | Specifies the host name. |
| Id | Specifies the ID of the logical drive resource. |
| Name | Specifies the name of the logical drive resource. |
| CapacityBytes | Specifies the capacity of the logical drive. |
| VolumeType | Specifies the redundancy type of the logical drive. The options are as follows: RawDevice NonRedundant Mirrored StripedWithParity SpannedMirrors SpannedStripesWithParit y |
| OptimumIOSizeBytes | Specifies the strip size of the logical drive. |
| Status | Specifies the logical drive status. The options are as follows: Health: indicates the health status of the logical drive. State: indicates the enabling status of the logical drive. |
| VolumeName | Specifies the name of the logical drive. |
| RaidControllerID | Specifies the ID of the controller to which the logical drive belongs. |
| VolumeRaidLevel | Specifies the RAID level of the logical drive. The options are as follows: RAID0 RAID1 RAID2 RAID3 RAID4 RAID5 RAID6 RAID10 RAID1E RAID20 RAID30 RAID40 RAID50 RAID60 |
| DefaultReadPolicy | Specifies the default read policy of the logical drive. The options are as follows: NoReadAhead ReadAhead |
| DefaultWritePolicy | Specifies the default write policy of the logical drive. The options are as follows: WriteThrough WriteBackWithBBU WriteBack |
| DefaultCachePolicy | Specifies the default cache policy of the logical drive. The options are as follows: CachedIO DirectIO |
| ConsistencyCheck | Specifies the enabling status of the consistency check function. |
| SpanNumber | Specifies the number of spans of the logical drive. |
| NumDrivePerSpan | Specifies the number of member drives of a span. |
| Spans | Specifies the subattributes. The options are as follows: SpanName: indicates the name of the span. Drives: indicates the list of member drives contained in the span. |
| CurrentReadPolicy | Specifies the current read policy of the logical drive. |
| CurrentWritePolicy | Specifies the current write policy of the logical drive. |
| CurrentCachePolicy | Specifies the current cache policy of the logical drive. |
| AccessPolicy | Specifies the access policy of the logical drive. The options are as follows: ReadWrite ReadOnly Blocked Hidden |
| BootEnable | Specifies whether the logical drive is the boot drive. |
| BGIEnable | Specifies the enabling status of background initialization. |
| SSDCachecadeVolume | Specifies whether the logical drive is a CacheCade logical drive. |
| SSDCachingEnable | Specifies whether a CacheCade logical drive can be used. |
| AssociatedCacheCadeVolume | Specifies the access path of the associated CacheCade logical drive. |
| DriveCachePolicy | Specifies the cache policy of the physical drive. The options are as follows: Enabled Disabled Unchanged |
| OSDriveName | Specifies the OS drive letter corresponding to the logical drive. |
| InitializationMode | Specifies the initialization mode of the logical drive. The options are as follows: UnInit QuickInit FullInit |

Modifying the Logical Drive Resource Attributes

Function

Modify the logical drive resource attributes of a specified RAID controller card.

Format

Set-iBMCVolume -Session <$session> -StorageId <StorageId> -VolumeId <VolumeId> -VolumeName <VolumeName> -DefaultReadPolicy <DefaultReadPolicy> -DefaultWritePolicy <DefaultWritePolicy> -DefaultCachePolicy <DefaultCachePolicy> -AccessPolicy <AccessPolicy> -DriveCachePolicy <DriveCachePolicy> -BootEnabled <BootEnabled> -BGIEnabled <BGIEnabled> -SSDCachingEnabled <SSDCachingEnabled>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |
| <StorageId> | Specifies the storage resource ID. | Set this parameter based on the ID queried in 3.62 Querying the RAID Controller Card Information. |
| <VolumeId> | Specifies the logical drive resource ID. This parameter is mandatory. | Set this parameter based on the ID queried in 3.80 Querying the Logical Drive Resource Information. |
| <VolumeName> | Specifies the logical drive name. This parameter is optional. | - |
| <DefaultReadPolicy> | Specifies the default read policy of the logical drive. This parameter is optional. | NoReadAhead: The Read Ahead function is disabled. ReadAhead: The Read Ahead function is enabled. The controller pre-reads sequential data or the data predicted to be used and saves it in the cache. |
| <DefaultWritePolicy> | Specifies the default write policy of the logical drive. This parameter is optional. | WriteThrough: After the drive subsystem receives all data, the controller sends the host a signal indicating that data transmission is complete. WriteBackWithBBU: When no BBU is configured for the controller or the BBU is faulty, the controller automatically switches to the WriteThrough mode. WriteBack: After receiving all the transmitted data, the controller cache returns a data transmission completion signal to the host. |
| <DefaultCachePolicy> | Specifies the default cache policy of the logical drive. This parameter is optional. | CachedIO: All the read and write requests are processed by the RAID controller card cache. Use this value only when CacheCade 1.1 is configured. This value is not recommended in other scenarios. DirectIO: This value has different definitions in read and write scenarios. In read scenarios, data is directly read from physical drives. When DefaultReadPolicy is set to ReadAhead, data read requests are processed by the RAID controller card cache. In write scenarios, data write requests are processed by the RAID controller card cache. When DefaultWritePolicy is set to WriteThrough, data write requests are not processed by the RAID controller card cache and data is directly written into physical drives. |
| <AccessPolicy> | Specifies the access policy of the logical drive. This parameter is optional. | ReadWrite: The logical drive is readable and writable. ReadOnly: The logical drive can only be read. Blocked: Access to the logical drive is denied. |
| <DriveCachePolicy> | Specifies the cache policy of the member drive. This parameter is optional. | Unchanged: The default cache policy is retained. Enabled: During the read/write process, data is written to the cache before being written to physical drives, improving the write performance. However, if the system is powered off unexpectedly, data will be lost if no protection mechanism is available. Disabled: During the read/write process, data is not written to the cache before being written to physical drives. When the system is powered off unexpectedly, data will not be lost. |
| <BootEnabled> | Specifies whether the drive is the boot drive. This parameter is optional. | This parameter can be set to true. |
| <BGIEnabled> | Specifies whether to enable background initialization. This parameter is optional. | true false |
| <SSDCachingEnabled> | Specifies whether to use the CacheCade logical drive as the cache. This parameter is optional. | true false |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Modify the logical drive resource attributes of a specified RAID controller card.

PS C:\> Set-iBMCVolume -Session $session -StorageId RAIDStorage0 -VolumeId LogicalDrive0 -VolumeName Volume1 ` 
          -DefaultCachePolicy CachedIO -DefaultWritePolicy WriteBack -DefaultReadPolicy ReadAhead ` 
          -AccessPolicy ReadOnly -DriveCachePolicy Enabled ` 
          -BootEnabled $true -BGIEnabled $true

Initializing a Logical Drive

Function

Initialize the logical drive of a specified RAID controller card.

Format

Initialize-iBMCVolume -Session <$session> -StorageId <StorageId> -VolumeId <VolumeId> -InitAction <InitAction>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |
| <StorageId> | Specifies the storage resource ID. | Set this parameter based on the ID queried in 3.62 Querying the RAID Controller Card Information. |
| <VolumeId> | Specifies the logical drive ID. This parameter is mandatory. | Set this parameter based on the ID queried in 3.80 Querying the Logical Drive Resource Information. |
| <InitAction> | Specifies the initialization mode. This parameter is mandatory. | The following initialization modes are supported: QuickInit: indicates the quick initialization. No task will be created. FullInit: indicates the complete initialization. A task will be created. CancelInit: indicates that the initialization is canceled. No task will be created. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Initialize the logical drive of a specified RAID controller card.

PS C:\> Initialize-iBMCVolume -Session $session -StorageId RAIDStorage0 ` 
          -VolumeId LogicalDrive0 -InitAction QuickInit

Deleting a Logical Drive

Function

Delete the logical drive of a specified RAID controller card.

Format

Remove-iBMCVolume -Session <$session> -StorageId <StorageId> -VolumeId <VolumeId>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |
| <StorageId> | Specifies the storage resource ID. | Set this parameter based on the ID queried in 3.62 Querying the RAID Controller Card Information. |
| <VolumeId> | Specifies the logical drive resource ID. This parameter is mandatory. | Set this parameter based on the ID queried in 3.80 Querying the Logical Drive Resource Information. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Delete the logical drive of a specified RAID controller card.

PS C:\> Remove-iBMCVolume -Session $session -StorageId RAIDStorage0 -VolumeId LogicDrive0         
 
Host         : 10.1.1.2     
Id           : 3     
Name         : volume deletion task     
ActivityName : [10.1.1.2] volume deletion task     
TaskState    : Completed     
StartTime    : 2019-01-06T22:48:09+00:00     
EndTime      : 2019-01-06T22:48:13+00:00     
TaskStatus   : OK     
TaskPercent  :

Creating a Logical Drive

Function

Create a logical drive under a specified RAID controller card.

Format

Add-iBMCVolume -Session <$session> -StorageId <StorageId> -CacheCade <CacheCade> -RAIDLevel <RAIDLevel> -Drives <Drives> -VolumeName <VolumeName> -StripSize <StripSize> -CapacityMB <CapacityMB> -DefaultReadPolicy <DefaultReadPolicy> -DefaultWritePolicy <DefaultWritePolicy> -DefaultCachePolicy <DefaultCachePolicy> -AccessPolicy <AccessPolicy> -DriveCachePolicy<DriveCachePolicy> -InitMode <InitMode> -SpanNumber <SpanNumber>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |
| <StorageId> | Specifies the storage resource ID. This parameter is mandatory. | Set this parameter based on the ID queried in 3.62 Querying the RAID Controller Card Information. |
| <CacheCade> | Specifies whether the logical drive is created as a CacheCade logical drive. | When a CacheCade logical drive is to be created, set this parameter to true. When a non-CacheCade logical drive is to be created, this parameter is optional and can be set to false. |
| <RAIDLevel> | Specifies the RAID level of the logical drive. This parameter is mandatory. | RAID0 RAID1 RAID5 RAID6 RAID10 RAID50 RAID60 NOTE When a CacheCade logical drive is to be created, this parameter can only be set to RAID0 or RAID1. You do not need to set this parameter when adding a logical drive to an existing drive group. |
| <Drives> | Specifies the list of member drives. This parameter is mandatory. | Format: [disk1,disk2,...,diskN] NOTE The member drives must use the same type of interfaces and storage media. Enter the ID of a physical drive in the available drive group if you want to add a logical drive to an existing drive group. Set this parameter based on the DriveID queried in 3.65 Querying the Drive Information, when FirmwareStatus is set to UnconfiguredGood. |
| <VolumeName> | Specifies the logical drive name. This parameter is optional. | - |
| <StripSize> | Specifies the strip size of the logical drive. This parameter is optional. | The unit is byte. The options are as follows: 65536 131072 262144 524288 1048576 NOTE You do not need to set this parameter when creating a CacheCade logical drive. |
| <CapacityMB> | Specifies the logical drive capacity. This parameter is optional. | The value must be an integer in bytes. The value must be a multiple of 1048576 (1 MB). NOTE You do not need to set this parameter when creating a CacheCade logical drive. |
| <DefaultReadPolicy> | Specifies the default read policy of the logical drive. This parameter is optional. | NoReadAhead: The Read Ahead function is disabled. ReadAhead: The Read Ahead function is enabled. The controller pre-reads sequential data or the data predicted to be used and saves it in the cache. NOTE You do not need to set this parameter when creating a CacheCade logical drive. |
| <DefaultWritePolicy> | Specifies the default write policy of the logical drive. This parameter is optional. | WriteThrough: After the drive subsystem receives all data, the controller sends the host a signal indicating that data transmission is complete. WriteBackWithBBU: When no BBU is configured for the controller or the BBU is faulty, the controller automatically switches to the WriteThrough mode. WriteBack: After receiving all the transmitted data, the controller cache returns a data transmission completion signal to the host. NOTE You cannot set this parameter to WriteBack when creating a CacheCade logical drive. |
| <DefaultCachePolicy> | Specifies the default cache policy of the logical drive. This parameter is optional. | CachedIO: All the read and write requests are processed by the RAID controller card cache. Use this value only when CacheCade 1.1 is configured. This value is not recommended in other scenarios. DirectIO: This value has different definitions in read and write scenarios. In read scenarios, data is directly read from physical drives. When DefaultReadPolicy is set to ReadAhead, data read requests are processed by the RAID controller card cache. In write scenarios, data write requests are processed by the RAID controller card cache. When DefaultWritePolicy is set to WriteThrough, data write requests are not processed by the RAID controller card cache and data is directly written into physical drives. |
| <AccessPolicy> | Specifies the access policy of the logical drive. This parameter is optional. | The following policies are supported: ReadWrite: The logical drive is readable and writable. ReadOnly: The logical drive can only be read. Blocked: Access to the logical drive is denied. NOTE You do not need to set this parameter when creating a CacheCade logical drive. |
| <DriveCachePolicy> | Specifies the cache policy of the member drive. This parameter is optional. | The following policies are supported: Unchanged: The default cache policy is retained. Enabled: During the read/write process, data is written to the cache before being written to physical drives, improving the write performance. However, if the system is powered off unexpectedly, data will be lost if no protection mechanism is available. Disabled: During the read/write process, data is not written to the cache before being written to physical drives. When the system is powered off unexpectedly, data will not be lost. NOTE You do not need to set this parameter when creating a CacheCade logical drive. |
| <InitMode> | Specifies the initialization mode of the logical drive. This parameter is optional. | UnInit: The initialization is not performed. QuickInit: Write zeros for the first and last 10 MB of the logical drive. After that, the logical drive status changes to Optimal. FullInit: Initialize the logical drive. Before the initialization is complete, the logical drive status is initialization. NOTE You do not need to set this parameter when creating a CacheCade logical drive. |
| <SpanNumber> | Specifies the number of spans of the logical drive. This parameter is optional. | Set this parameter to 1 if the RAID level is RAID 0, RAID 1, RAID 5, or RAID 6. Set this parameter to a value ranging from 2 to 8 if the RAID level is RAID 10, RAID 50, or RAID 60. NOTE You do not need to set this parameter when creating a CacheCade logical drive. You do not need to set this parameter when adding a logical drive to an existing drive group. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Create a logical drive under a specified RAID controller card.

PS C:\> $Drives = ,@(0, 1) 
PS C:\> $Volumes = Add-iBMCVolume -Session $session -StorageId RAIDStorage0 -CacheCade $false -RAIDLevel RAID1 -Drives $Drives 
   
 Host         : 10.1.1.2 
 Id           : 4  
 Name         : volume creation task  
 ActivityName : [10.1.1.2] volume creation task  
 TaskState    : Completed  
 StartTime    : 2019-01-07T09:37:15+00:00  
 EndTime      : 2019-01-07T09:37:29+00:00  
 TaskStatus   : OK  
 TaskPercent  :

Collecting iBMC Logs in One-click Mode

Function

Collect the maintenance information about all modules of a board.

Format

Export-iBMCMaintenanceInfo -Session <$session> -ExportTo <ExportTo>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |
| <ExportTo> | Specifies the path for exporting the configuration file. This parameter is mandatory. | If a local temporary directory of the iBMC is used, the directory must be the /tmp directory and a file name must be specified. If a remote directory is used, the value must be in File transfer protocol://User name:Password@Remote server IP address/Directory/File name format. The file transfer protocols include HTTPS, SCP, SFTP, CIFS, and NFS. |

Usage Guidelines

The format of the exported file must be .tar.gz.

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Export iBMC logs.

Export iBMC logs to the tmp directory of the iBMC.

PS C:\> $ExportPath = "/tmp/collect.tar.gz" 
 PS C:\> $Tasks = Export-iBMCMaintenanceInfo -Session $session -ExportTo $ExportPath 
 PS C:\> $Tasks 
   
 Host         : 10.1.1.2 
 Id           : 1  
 Name         : Export Dump File Task  
 ActivityName : [10.1.1.2] Export Dump File Task  
 TaskState    : Completed  
 StartTime    : 2019-01-19T04:22:13+00:00  
 EndTime      : 2019-01-19T04:30:19+00:00  
 TaskStatus   : OK  
 TaskPercent  : 100%

Export iBMC logs a remote directory.

PS C:\> $ExportPath = "nfs://10.10.10.3/data/nfs/collect.tar.gz" 
 PS C:\> $Tasks = Export-iBMCMaintenanceInfo -Session $session -ExportTo $ExportPath 
 PS C:\> $Tasks 
   
 Host         : 10.1.1.2 
 Id           : 1  
 Name         : Export Dump File Task  
 ActivityName : [10.1.1.2] Export Dump File Task  
 TaskState    : Completed  
 StartTime    : 2019-01-19T04:22:13+00:00  
 EndTime      : 2019-01-19T04:30:19+00:00  
 TaskStatus   : OK  
 TaskPercent  : 100%

Output Description

Output description

| Output Item | Description |
| --- | --- |
| Host | Specifies the host name. |
| Id | Specifies the task ID. |
| Name | Specifies the task name. |
| ActivityName | Specifies the task connection status. |
| TaskState | Specifies the task execution status. The options are as follows: New Starting Running Suspended Interrupted Pending Stopping Completed Killed Exception Service |
| StartTime | Specifies the task start time. |
| EndTime | Specifies the task end time. |
| TaskStatus | Specifies the task execution status. |
| TaskPercent | Specifies the task execution progress. |

Downloading a BMC File

Function

Download a BMC file.

Format

Invoke-iBMCFileDownload -Session <$session> -BMCFileUri <BMCFileUri> -LocalFileUri <LocalFileUri>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |
| <BMCFileUri> | Specifies the path of the file in the BMC. | The value is /tmp/web and its subdirectories. |
| <LocalFileUri> | Specifies the path for storing the downloaded file. | The value is a local directory. The value is in the format of Directory\File name or \\Server IP address\Directory\File name. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Download a BMC file.

PS C:\> $BMCFilePath = "/tmp/web/2288H_V5_5288_V5-iBMC-V318.hpm" 
 PS C:\> $LocalFilePath = "E:\2288H_V5_5288_V5-iBMC-V318.hpm" 
 PS C:\> Invoke-iBMCFileDownload -Session $session -BMCFileUri $BMCFilePath -LocalFileUri $LocalFilePath

Uploading a File

Function

Use a Redfish interface to upload a file, and store the file in the /tmp/web directory.

Format

Invoke-iBMCFileUpload -Session <$session> -FileUri <FileUri>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |
| <FileUri> | Specifies the path of the file. | Select the corresponding file in the local directory. The file types supported by a x86 V3 board are as follows: .hpm, .cer, .pem, .cert, .crt, .pfx, .p12, .xml, .keys, and .pub The file types supported by a x86 V5 board are as follows: .hpm, .zip, .asc, .cer, .pem, .cert, .crt, .pfx, .p12, .xml, .keys, and .pub The maximum size of an. hpm file supported by a x86 V3 board is 46 MB. The maximum size of an .hpm, .zip, or .asc file supported by a V5 board is 60 MB. The maximum size of a .cer, .pem, .cert, .crt, .xml, or .p12 file is 1 MB. The maximum size of a .pfx or .keys file is 2 MB. The maximum size of a .pub file is 2 KB. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Upload a file to the /tmp/web directory.

PS C:\> $Path = Invoke-iBMCFileUpload -Session $session -FileUri E:\2288H_V5_5288_V5-iBMC-V318.hpm 
 PS C:\> $Path 
   
 Path  
 ----  
 /tmp/web/2288H_V5_5288_V5-iBMC-V318.hpm

Querying the License Service Information

Function

Query the license service information.

Format

Get-iBMCLicense -Session <$session>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Query the license service information.

PS C:\> $License = Get-iBMCLicense -Session $session 
PS C:\> $License 
 
Host            : 10.1.1.2 
Id              : LicenseService 
Name            : License Service 
Capability      : {Local, Remote} 
DeviceESN       : A5A7D970F13580158FBBCA5009F17234D1A6C5CB 
InstalledStatus : Installed 
RevokeTicket    : 
LicenseClass    : Advanced 
LicenseStatus   : Normal 
LicenseInfo     : @{FileFormatVersion=3.10; GeneralInfo=; CustomerInfo=; NodeInfo=; SaleInfo=} 
AlarmInfo       : @{RemainGraceDay=0; RemainCommissioningDay=0; ProductESNValidState=Valid; FileState=Normal; ProductESNMatchState=Matched; ProductVersionMatchState=Matched}

Output Description

Output description

| Output Item | Description |
| --- | --- |
| Host | Specifies the host name. |
| Id | Specifies the ID of a license service resource. |
| Name | Specifies the name of a license service resource. |
| Capability | Specifies the license management capabilities. The options are as follows: Local: supports the local license management. Remote: supports the remote license management. |
| DeviceESN | Specifies the ESN of a device. |
| InstalledStatus | Specifies the installation status of a license file. The options are as follows: NotInstalled Installed Installing |
| RevokeTicket | Specifies the license revocation code. |
| LicenseClass | Specifies the license file level when the license file is managed locally. When it is managed remotely, this field is invalid. The options are as follows: NotInstalled Advanced |
| LicenseStatus | Specifies the license status. The options are as follows: N/A Default GracePeriod Normal Emergency Commissioning Unknown |
| LicenseInfo | Specifies the license file information. |
| AlarmInfo | Specifies the license alarm information. |

Installing a License

Function

Install a license.

Format

Install-iBMCLicense -Session <$session> -LicenseFileURI <LicenseFileURI> -LicenseSource <LicenseSource>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |
| <LicenseFileURI> | Specifies the path of the license file. | If a local directory is used, the value must be in Directory\File name or \\Server IP address\Directory\File name format. If a local temporary directory of the iBMC is used, the directory must be the /tmp directory and a file name must be specified. If a remote directory is used, the value must be in File transfer protocol://User name:Password@Remote server IP address/Directory/File name format. The file transfer protocols include HTTPS, SCP, SFTP, CIFS, and NFS, and the protocol name must be in lower case. |
| <LicenseSource> | Specifies the source of the license file. | The options are as follows: iBMC FusionDirector eSight If the parameter is not configured, the default value is iBMC. |

Usage Guidelines

None

Example

# Use a local file to install a license.

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Install a license.

PS C:\> Install-iBMCLicense -Session $session ` 
          -LicenseFileURI "E:\huawei\PowerShell\LIC2288H_V5_2_20180905LTM65C.xml" ` 
          -LicenseSource iBMC

# Use the iBMC local temporary directory to install the license.

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Install a license.

PS C:\> Install-iBMCLicense -Session $session ` 
          -LicenseFileURI "/tmp/LIC2288H_V5_2_20180905LTM65C.xml"

# Use the remote directory to install a license.

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Install a license.

PS C:\> Install-iBMCLicense -Session $session ` 
          -LicenseFileURI "nfs://10.10.10.2/data/nfs/LIC2288H_V5_2_20180905LTM65C.xml"

Exporting a License

Function

Export a license.

Format

Export-iBMCLicense -Session <$session> -ExportTo <ExportTo>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |
| <ExportTo> | Specifies the path of the exported file. | If a local temporary directory of the iBMC is used, the directory must be the /tmp directory and a file name must be specified. If a remote directory is used, the value must be in File transfer protocol://User name:Password@Remote server IP address/Directory/File name format. The file transfer protocols include HTTPS, SCP, SFTP, CIFS, and NFS. |

Usage Guidelines

None

Example

# Use the iBMC local temporary directory to export a license.

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Export a license.

PS C:\> Export-iBMCLicense -Session $session -ExportTo "/tmp/License.xml"

# Use the remote directory to export a license.

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Export a license.

PS C:\> Export-iBMCLicense -Session $session -ExportTo "NFS://112.93.129.100/data/nfs/License.xml"

Deleting a License

Function

Delete a license.

Format

Remove-iBMCLicense -Session <$session>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Delete a license.

PS C:\> Remove-iBMCLicense -Session $session

Querying iBMC Network Port Information

Function

This command is used to query information about the iBMC network port.

Format

Get-iBMCIP -Session <$session>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Query information about the iBMC network port.

PS C:\Users\Administrator> Get-iBMCIP -Session $session 
 
 
Host                  : 172.26.100.8 
Id                    : 04338932ede2 
Name                  : Manager Ethernet Interface 
PermanentMACAddress   : 04:33:89:32:ed:e2 
HostName              : server8 
FQDN                  : server8.plugin.com 
VLAN                  : @{VLANEnable=False; VLANId=0} 
IPv4Addresses         : {@{Address=172.26.100.8; SubnetMask=255.255.0.0; Gateway=172.26.0.1; AddressOrigin=Static}} 
IPv6Addresses         : {@{Address=172:26::8; PrefixLength=64; AddressOrigin=Static}, @{Address=fe80::633:89ff:fe32:ede2; PrefixLength=64; AddressOrigin=LinkLocal}} 
IPv6StaticAddresses   : {@{Address=172:26::8; PrefixLength=64}} 
IPv6DefaultGateway    : 172:26::1 
NameServers           : {172.26.201.7, } 
IPVersion             : IPv4AndIPv6 
NetworkPortMode       : Fixed 
ManagementNetworkPort : @{Type=Dedicated; PortNumber=1}

Output Description

Output information

| Parameter | Description |
| --- | --- |
| Host | Host name. |
| Id | Task ID. |
| Name | Task name. |
| PermanentMACAddress | MAC address of the iBMC network port. |
| HostName | Host name of the iBMC. |
| FQDN | Fully qualified domain name (FQDN) of the iBMC. |
| VLAN | VLAN information of the iBMC network port. VLANEnable: indicates whether VLAN is enabled. VLANId: VLAN ID. |
| IPv4Addresses | IPv4 address information of the iBMC network port. Address: IPv4 address. SubnetMask: subnet mask of the IPv4 address. Gateway: IPv4 gateway address. AddressOrigin: acquisition mode of the IPv4 address. |
| IPv6Addresses | IPv6 address information of the iBMC network port. Address: IPv6 address. PrefixLength: prefix length of the IPv6 address. AddressOrigin: acquisition mode of the IPv6 address. |
| IPv6StaticAddresses | Static IPv6 address information of the iBMC network port. Address: IPv6 address. PrefixLength: prefix length of the IPv6 address. |
| IPv6DefaultGateway | IPv6 gateway address of the iBMC network port. |
| NameServers | Addresses of the preferred and alternate DNS servers when the iBMC network port address is dynamically assigned by using the DHCP. The server IP address can be an IPv4 or IPv6 address. |
| IPVersion | IP versions to be used. |
| NetworkPortMode | Network port mode. Value: Fixed Automatic |
| ManagementNetworkPort | Management network Port. Type: type of the management network port. PortNumber: silkscreen of the management network port. NOTE If a dedicated network port is used, PortNumber indicates the serial number of the port, not the silkscreen. |

Setting iBMC Network port Information

Function

This command is used to configure the iBMC network port information.

Format

Set-iBMCIP -Session <$session> -IPVersion <IPVersion> -IPv4Address <IPv4Address> -IPv4SubnetMask <IPv4SubnetMask> -IPv4Gateway <IPv4Gateway> -IPv4AddressOrigin <IPv4AddressOrigin> -IPv6Address <IPv6Address> -IPv6PrefixLength <IPv6PrefixLength> -IPv6Gateway <IPv6Gateway> -IPv6AddressOrigin  <IPv6AddressOrigin>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |
| <IPVersion> | Specifies the IP versions to be used. This parameter is optional. | IPv4: enables IPv4 only. IPv6: enables IPv6 only. IPv4AndIPv6: enables IPv4 and IPv6. |
| <IPv4Address> | Specifies the IPv4 address of the iBMC network port. This parameter is optional. | - |
| <IPv4SubnetMask> | Specifies the subnet mask of the IPv4 address. This parameter is optional. | - |
| <IPv4Gateway> | Specifies the IPv4 gateway address of the iBMC network port. This parameter is optional. | - |
| <IPv4AddressOrigin> | Specifies the mode in which the IPv4 address of the iBMC network port is obtained. This parameter is optional. | Static DHCP |
| <IPv6Address> | Specifies the IPv6 address of the iBMC network port. This parameter is optional. | - |
| <IPv6PrefixLength> | Specifies the prefix length of the IPv6 address. This parameter is optional. | - |
| <IPv6Gateway> | Specifies the IPv6 gateway address of the iBMC network port. This parameter is optional. | - |
| <IPv6AddressOrigin> | Specifies the mode in which the IPv6 address of the iBMC network port is obtained. This parameter is optional. | Static DHCPv6 |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Configure the iBMC network port information.

PS C:\> Set-iBMCIP -Session $Session -IPVersion IPv4AndIPv6 -IPv4Address 10.1.1.2 -IPv4SubnetMask 255.255.0.0 -IPv4Gateway 10.1.0.1 -IPv4AddressOrigin Static -IPv6Address fc00:10:2 -IPv6PrefixLength 64 -IPv6Gateway fc00:10:1 -IPv6AddressOrigin Static

Querying LDAP Information

Function

Query LDAP information.

Format

Get-iBMCLDAP -Session <$session> -LDAPID <LDAPID>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |
| <LDAPID> | Specifies the ID of the LDAP resource. This parameter is optional. NOTE If no LDAP ID is specified, information about all domain controllers will be queried. | An integer ranging from 1 to 6. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Query LDAP information.

PS C:\> $LDAPID = 1 
PS C:\> Get-iBMCLDAP -Session $Session -LDAPID $LDAPID 
 
Host                           : 10.1.1.2 
Id                             : 1 
Name                           : Ldap Controller 
LdapServerAddress              : 
LdapPort                       : 636 
UserDomain                     : ,DC= 
BindDN                         : 
BindPassword                   : 
CertificateVerificationEnabled : False 
CertificateVerificationLevel   : 
CertificateInformation         : 
CertificateChainInformation    : 
LdapGroups                     : {@{MemberId=0; GroupName=; GroupDomain=CN=,OU=,DC=; GroupRole=No Access; 
GroupLoginRule=System.Object[]; 
                                 GroupLoginInterface=System.Object[]}, @{MemberId=1; GroupName=; 
GroupDomain=CN=,OU=,DC=; GroupRole=No Access; 
                                 GroupLoginRule=System.Object[]; GroupLoginInterface=System.Object[]}, 
@{MemberId=2; GroupName=; GroupDomain=CN=,OU=,DC=; 
                                 GroupRole=No Access; GroupLoginRule=System.Object[]; 
GroupLoginInterface=System.Object[]}, @{MemberId=3; GroupName=; 
                                 GroupDomain=CN=,OU=,DC=; GroupRole=No Access; GroupLoginRule=System.Object[]; 
GroupLoginInterface=System.Object[]}...}

Output Description

For details, see Parameters.

Modifying LDAP Information

Function

Modify LDAP information.

Format

Set-iBMCLDAP -Session <$session> -LDAPID <LDAPID> -LDAPAddress <LDAPAddress> -LDAPPort <LDAPPort> -UserDomain <UserDomain> -BindDN <BindDN> -BindPassword <BindPassword> -CertificateVerificationEnabled <CertificateVerificationEnabled> -CertificateVerificationLevel <CertificateVerificationLevel> -GroupID <GroupID> -GroupName <GroupName> -GroupDomain <GroupDomain> -GroupRole <GroupRole> -GroupLoginRule <GroupLoginRule> -GroupLoginInterface <GroupLoginInterface>

Parameters

| Parameter | Parameters | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |
| <LDAPID> | Specifies the ID of the LDAP resource. This parameter is mandatory. | An integer ranging from 1 to 6. |
| <LDAPAddress> | Specifies the address of the domain controller. This parameter is optional. | The value can be an IPv4 or IPv6 address, or a domain name. |
| <LDAPPort> | Specifies the port number of the domain controller. This parameter is optional. | An integer ranging from 1 to 65535. |
| <UserDomain> | Specifies the user domain of the domain controller. This parameter is optional. | Character string, for example: CN=test,,DC=huawei,DC=com |
| <BindDN> | Specifies the Distinguished name (DN) of an LDAP proxy user. This parameter is optional. | A string of 0 to 255 characters. |
| <BindPassword> | Specifies the authentication password of the LDAP proxy user. This parameter is optional. | A string of 0 to 20 characters. |
| <CertificateVerificationEnabled> | Specifies whether the certificate verification is enabled. This parameter is optional. | True False |
| <CertificateVerificationLevel> | Specifies the certificate verification level. This parameter is valid only when the certificate verification is enabled. | Demand Allow |
| <GroupID> | Specifies the ID of an LDAP user group. This parameter is optional. | An integer ranging from 0 to 4. |
| <GroupName> | Specifies the name of an LDAP user group. This parameter is optional. | Character string |
| <GroupDomain> | Specifies the domain of an LDAP user group. This parameter is optional. | Character string, for example: CN=qwert,OU=admin,DC=huawei,DC=com |
| <GroupRole> | Specifies the role of an LDAP user group. This parameter is optional. | Administrator Operator Commonuser Noaccess CustomRole1 CustomRole2 CustomRole3 CustomRole4 |
| <GroupLoginRule> | Specifies the login rules that apply to an LDAP user group. This parameter is optional. | One or more of Rule1, Rule2, and Rule3. |
| <GroupLoginInterface> | Specifies the interfaces through which the LDAP group members can log in to iBMC. | One or more of Web, SSH, and Redfish. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Modify LDAP information.

PS C:\> $BindPwd = ConvertTo-SecureString -String bind-password -AsPlainText -Force 
PS C:\> $GroupLoginRule = ,@("Rule1", "Rule2", "Rule3") 
PS C:\> $GroupLoginInterface = ,@("Web","SSH","Redfish") 
PS C:\> $result = Set-iBMCLDAP -Session $Session -LDAPID 1 -LDAPAddress "ldap.huawei.com" -LDAPPort 635 -UserDomain 'CN=test,,DC=huawei,DC=com' -BindDN test -BindPassword $BindPwd -CertificateVerificationEnabled $true -CertificateVerificationLevel Allow -GroupID 0 -GroupName qwert -GroupDomain 'CN=qwert,OU=admin,DC=huawei,DC=com' -GroupRole Administrator -GroupLoginRule $GroupLoginRule -GroupLoginInterface $GroupLoginInterface

Importing an LDAP certificate to the iBMC

Function

This command is used to import an LDAP certificate to the iBMC.

Format

Import-iBMCLDAPCert -Session <$session> -LDAPID <LDAPID> -LDAPCertPath <LDAPCertPath>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |
| <LDAPID> | Specifies the ID of the LDAP resource. This parameter is mandatory. | An integer ranging from 1 to 6. |
| <LDAPCertPath> | Specifies the source directory of the certificate to be imported. This parameter is mandatory. | If the certificate is to be imported from a client (local PC) to the iBMC, enter the path in Folder\File name or \\Client IP address\Folder\File name format. If the certificate is to be imported from a temporary directory of the iBMC, enter /tmp/File name. If the certificate is to be imported from a remote server to the iBMC, enter the path in the Transfer Protocol://User name:Password@IP address of the remote server/Folder/File name. The supported file transfer protocols include HTTPS, SCP, SFTP, CIFS, and NFS. |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Import an LDAP certificate to the iBMC.

Import an LDAP certificate from a local PC.

PS C:\> $Result = Import-iBMCLDAPCert -Session $Session -LDAPID 1 -LDAPCertPath 'c:\ca.cer' 
PS C:\> $Result 
 
Host              : 10.1.1.2 
MessageId         : iBMC.1.0.LDAPCertImportSuccess 
RelatedProperties : 
Message           : The LDAP certificate is imported successfully. 
MessageArgs       : 
Severity          : OK 
Resolution        : None

Import an LDAP certificate from a remote server to the iBMC.

PS C:\> $Result = Import-iBMCLDAPCert -Session $Session -LDAPID 1 -LDAPCertPath 'nfs://10.1.1.100/data/ldap.cer"' 
PS C:\> $Result 
 
Host         : 10.1.1.2 
Id           : 1 
Name         : ldap root cert import 
ActivityName : [10.1.1.2] ldap root cert import 
TaskState    : Completed 
StartTime    : 2019-11-28T16:44:46+08:00 
EndTime      : 2019-11-28T16:44:47+08:00 
TaskStatus   : OK 
TaskPercent  : 100%

Output Description

Output information (import from a local PC)

| Parameter | Description |
| --- | --- |
| Host | Host name. |
| MessageId | Message ID. |
| RelatedProperties | Properties related to the message. |
| Message | Details. |
| MessageArgs | Message parameters. |
| Severity | Severity level. |
| Resolution | Handling suggestions. |

Output information (import from a remote server)

| Parameter | Description |
| --- | --- |
| Host | Host name. |
| Id | Task ID. |
| Name | Task name. |
| ActivityName | Name of the task activity. |
| TaskState | Task progress. |
| StartTime | Time when the task was started. |
| EndTime | Time when the task was complete. |
| TaskStatus | Task execution status. |
| TaskPercent | Task completion ratio. |

Setting the LDAP Status

Function

Enable or disable the LDAP function.

Format

Set-iBMCLDAPServiceEnabled -Session <$session> -LdapServiceEnabled <LdapServiceEnabled>

Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| <$session> | Specifies the session. This parameter is mandatory. | - |
| <LdapServiceEnabled> | Specifies whether the LDAP service is enabled. This parameter is mandatory. | True False |

Usage Guidelines

None

Example

- Log in to the iBMC. For details, see 3.3 Connecting to the iBMC.

- Enable the LDAP function.

PS C:\> $User = Set-iBMCUser -Session $session -LdapServiceEnabled $true 
PS C:\> $User 
 
Host               : 10.1.1.2 
Id                 : 12 
Name               : User Account 
LdapServiceEnabled : true

Output Description

Output information

| Parameter | Description |
| --- | --- |
| Host | Host name. |
| ID | Task ID. |
| Name | Task name. |
| LdapServiceEnabled | LDAP service status, which can be enabled or disabled. |

FAQs

4.1  How to Invoke Cmdlets Without Using a Plaintext Password

4.2  How to Use the CA Root Certificate to Authenticate a Huawei Server

4.3  How to Invoke a Cmdlets Interface

4.4  How to Record Logs as a Non-Administrator User

  - How to Invoke Cmdlets Without Using a Plaintext Password

            - Generate a key and password file.

- Generate a 32-bit key and save it to the aes.key file.

PS C:\Users\Administrator> $KeyFile = "C:\aes.key"  
PS C:\Users\Administrator> $Key = New-Object Byte[] 32  
PS C:\Users\Administrator> [Security.Cryptography.RNGCryptoServiceProvider]::Create().GetBytes($Key)

- Use the key to generate a password file and save the file.

PS C:\Users\Administrator> $Key | Out-File $KeyFile 
PS C:\Users\Administrator> Read-Host "Enter Password" -AsSecureString | ConvertFrom-SecureString -key $Key |Out-File "C:\pwd.txt" 
Enter Password: **********  
PS C:\Users\Administrator>

- You can obtain the aes.key and pwd.txt files from the specified directory in the environment. The encrypted key and password file can be used in any Cmdlets environment.

aes.key and pwd.txt files

Use the password file and key file to create credential information.

PS C:\Users\Administrator> $userName = "root"  
PS C:\Users\Administrator> $passwdFile = "C:\pwd.txt"  
PS C:\Users\Administrator> $keyFile = "C:\aes.key"  
PS C:\Users\Administrator> $key = Get-Content $keyFile  
PS C:\Users\Administrator> $cred = New-Object -TypeName System.Management.Automation.PSCredential -ArgumentList $userName, (Get-Content $passwdFile | ConvertTo-SecureString -key $key)

Use the credential information to set up a session.

PS C:\Users\Administrator> Connect-iBMC -Address 10.10.10.1 -Credential $cred -TrustCert  
  
Id                  : Blade4  
Name                : Manager  
ManagerType         : BMC  
FirmwareVersion     : 3.00  
UUID                : 14373629-DC99-A151-E811-F789DE92D0A2  
Model               : iBMC  
Health              : OK  
State               : Enabled  
DateTime            : 2019-02-19T03:43:33+00:00  
DateTimeLocalOffset : GMT  
Address             : 10.10.10.1  
BaseUri             : https://10.10.10.1  
Location            : /redfish/v1/SessionService/Sessions/96c42b21c44822ea  
Alive               : True  
AuthToken           : 4e2c838c6f6514a6a1741e09a98b1330  
TrustCert           : True

----End

How to Use the CA Root Certificate to Authenticate a Huawei Server

            - Set the iBMC FQDN.

- Log in to the iBMC WebUI, and choose Configuration > Network.

- In the DNS area, set the domain name, for example, cert.com, as shown in Figure 4-2.

Setting the domain name

- Click Save.

- Set the iBMC host name, for example, Server2, as shown in Figure 4-3.

Setting the iBMC host name

- Click Save. The Confirm dialog box is displayed.

- Click Restart Now for the setting to take effect.

Configure the iBMC DNS.

- Log in to the DNS server and configure the forward and reverse resolution for the iBMC.

- Set the DNS server in Step 2.1 as the DNS server in the Cmdlets environment.

- In the Cmdlets environment, open a browser, enter https://server2.cert.com in the address box, and press Enter to log in to the iBMC.

If the server iBMC WebUI is displayed, the binding is successful.

Generate an iBMC server certificate.

- Generate a CSR file.

  - Log in to the iBMC WebUI, and choose Configuration > SSL Certificate.

The SSL Certificate page is displayed, as shown in Figure 4-4.

SSL Certificate page

  - Click Customize.

The customization page is displayed, as shown in Figure 4-5.

Customization page

  - Click Generate, and save the generated server.csr file to a local path.

- Generate a server certificate using the CA root certificate and CSR file.

For example, use the OpenSSL tool to run the following command to generate a server certificate:

openssl x509 -req -days 3650 -sha256 -extensions server_cert -extfile openssl.conf -CA ca.cer -CAkey ca.key -CAserial ca.srl -CAcreateserial -in server.csr -out server.cer

Import the CA root certificate to the Cmdlets running environment.

If the CA root certificate has been imported, you do not need to import it again.

- Double-click the root certificate ca.cer.

The Certificate dialog box s displayed, as shown Figure 4-6.

Certificate

- Click Install Certificate.

The Certificate Import Wizard dialog box is displayed.

- Select Local Machine, as shown in Figure 4-7.

Setting Local Machine

- Click Next.

The Certificate Store dialog box is displayed.

- Select Place all certificates in the following store, as shown by (1) in Figure 4-8.

Certificate Store

- Click Browse, as shown by (2) in Figure 4-8.

The Select Certificate Store dialog box is displayed.

- Select Trusted Root Certification Authorities and click OK, as shown by (3) and (4) in Figure 4-8.

- Click Next.

The dialog box shown in Figure 4-9 is displayed.

Certificate imported

- Click Finish.

The dialog box shown in Figure 4-10 is displayed.

Import successful

- Click OK.

- In the dialog box displayed as shown in Figure 4-6, click OK.

Import the iBMC server certificate.

- Log in to the iBMC WebUI, and choose Configuration > SSL Certificate > Customize.

The customization page is displayed, as shown in Figure 4-11.

Customization page

- Click Browse and select the server.cer file, as shown in Figure 4-12.

Importing the server certificate

- Click Import. The Confirm dialog box is displayed.

- Click Restart Now for the setting to take effect.

Verify the result.

In the Cmdlets environment, log in to the iBMC WebUI using the domain name. No alarm information about the certificate is displayed, as shown in Figure 4-13.

No alarm

Use Cmdlets to create an iBMC connection without adding -TrustCert. The session is created successfully.

PS C:\Users\Administrator> $session = Connect-iBMC -Address server2.cert.com -Credential $cred 
PS C:\Users\Administrator> $session 
 
 
Id                  : Blade4 
Name                : Manager 
ManagerType         : BMC 
FirmwareVersion     : 3.00 
UUID                : 14373629-DC99-A151-E811-F789DE92D0A2 
Model               : iBMC 
Health              : OK 
State               : Enabled 
DateTime            : 2019-02-23T01:09:19+08:00 
DateTimeLocalOffset : Asia/Shanghai 
Address             : server2.cert.com 
BaseUri             : https://server2.cert.com 
Location            : /redfish/v1/SessionService/Sessions/cdbc3a0503c9c670 
Alive               : True 
AuthToken           : 859a096a61462675213390d65306578a

----End

How to Invoke a Cmdlets Interface

The following uses asset tag query and configuration as an example:

Import-Module -Name Huawei-iBMC-Cmdlets -Force 
  
try { 
  # connect to ibmc 
  $iBMCAddressList = @("10.1.1.1", "10.1.1.2") 
  $Sessions = Connect-iBMC -Address $iBMCAddressList -Username root -Password "Huawei12#$" -TrustCert 
  
  # filter all invalid sessions 
  $InvaildSessions = @($($Sessions | Where-Object {$_ -is [Exception]})) 
  # display session connect failed reason 
  if ($InvaildSessions.Count -gt 0) { 
    $InvaildSessions | ForEach-Object { 
      Write-Error -Exception $_ 
    } 
    return 
  } 
  
  # Codes start 
  
  # set asset tag 
  $AssertTagList = @("server1", "server2") 
  $SetResults = Set-iBMCAssetTag -Session $Sessions -AssetTag $AssertTagList 
  # filter all invalid result 
  $InvaildSetResults = @($($SetResults | Where-Object {$_ -is [Exception]})) 
  # display result failed reason 
  if ($InvaildSetResults.Count -gt 0) { 
    $InvaildSetResults | ForEach-Object { 
      Write-Error -Exception $_ 
    } 
    return 
  } 
  
  # get asset tag 
  $GetResults = Get-iBMCAssetTag -Session $Sessions 
  # filter all invalid result 
  $InvaildGetResults = @($($GetResults | Where-Object {$_ -is [Exception]})) 
  # display result failed reason 
  if ($InvaildGetResults.Count -gt 0) { 
    $InvaildGetResults | ForEach-Object { 
      Write-Error -Exception $_ 
    } 
    return 
  } 
  # output result 
  $count = $iBMCAddressList.Count 
  
  for($i = 0; $i -lt $count; $i++) { 
    Write-Output "iBMCAddress: $($iBMCAddressList[$i])" 
    Write-Output $($GetResults[$i] | fl) 
  } 
   
  # Codes end 
} 
finally { 
  # disconnect all valid sessions 
  $VaildSessions = $Sessions | Where-Object {$_ -isnot [Exception]} 
  if ($null -ne $VaildSessions) { 
      Disconnect-iBMC -Session $VaildSessions | Out-Null 
  } 
}

How to Record Logs as a Non-Administrator User

            - Log in to the running environment of the plug-in as an administrator.

Go to the plug-in installation directory, for example, C:\Program Files\WindowsPowerShell\Modules\Huawei-iBMC-Cmdlets\common.

Find the Log4Net.xml log configuration file, as shown in Figure 4-14.

Log4Net.xml

Open the Log4Net.xml file in editing mode and change the log saving path to the path of a non-administrator user.

For example, in C:\Users\operator\huawei.ibmc.cmdlets.log, operator indicates a non-administrator user.

----End

Obtaining Help

  - Preparing to Contact Huawei Technical Support

If a fault persists during routine maintenance or troubleshooting, contact Huawei technical support.

To rectify a fault, make the following preparations before you contact Huawei technical support.

Collecting Fault Information

You need to collect the following information:

Your company name and detailed address

Name and telephone number of the contact person

Time when the fault occurred

Fault symptom

Device type and software version

Measures taken after the fault occurred and results

Fault severity and deadline for rectifying the fault

Preparing for Debugging

When you seek technical support, Huawei technical support may ask you to perform some operations to further collect fault information or even rectify the fault. You need to make preparations before seeking technical support. For example, prepare spare server parts and controller cards, screwdrivers, screws, serial cables, network cables, and other necessary objects.

Obtaining Help from Huawei Support Website

Huawei provides timely and efficient technical support over local offices, secondary technical support systems, telephones, remote technologies, and onsite instructions.

Huawei technical support system consists of:

Technical Support Department at Huawei Headquarters

Technical support centers in local offices

Huawei support website

Customer service center

Huawei support website: http://support.huawei.com/enterprise

To view the latest product documentation at http://support.huawei.com, perform the following steps:

- Log in to http://support.huawei.com/enterprise.

- Click Login. The Login page is displayed.

- Enter your user name, password, and verification code, and click Login. The Technical Support page is displayed.

- In the navigation tree, click TECHNICAL SUPPORT > Technical Support > Product and Solution Support and select a product manual based on the product name.

Alternatively, you can quickly locate a product manual by entering a keyword in the Search text box in the upper right corner of the web page.